| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Generalization of allocation granularity definitions |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Yuval Lomnitz Dov Andelman Intel Corp. | Voice: +972-3-9205773 E-mail: Yuval.lomnitz@intel.com dov.andelman@intel.com |
| Re: | Letter Ballot 26b |  |
| Abstract | This contribution is part of several contributions completing definitions of AMC for MIMO zone and is concerned with the allocation granularity. We extend the definitions from PUSC to AMC. |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Generalization of allocation granularity definitions

Yuval Lomnitz, Dov Andelman - Intel Corp.

# Purpose

Allocation granularity definitions were made for the case of PUSC+MIMO with dedicated pilots, since the fact the slot duration (2 symbols) is shorter than the pilot period (4 symbols) can result in allocations without sufficient pilots. Conditioned on the acceptance of C802.16maint-08/007r3 the same situation may occur for AMC2x3 and AMC3x2 in MIMO zone with dedicated pilots. This contribution generalizes the allocation granularity definitions that were defined for PUSC+MIMO+dedicated pilots to AMC (or any other permutation)

Note: the contribution C802.16maint-08/007r2 covering different topics related to the AMC+MIMO zone was split into the following contributions in order to enable separate discussion/decision on each item, and this contribution is part of this pack:

- C802.16maint-08/007r3 (Definitions for AMC permutation slot structure in MIMO zone): covers definitions of AMC slot structures in the map IE-s and STC mapping (DL and UL), except matrix A support for 2x3 slot structure

- C802.16maint-08/087 (Definitions for matrix A in 2x3 AMC structure): adds the definition of matrix A support (both DL and UL)

- C802.16maint-08/088 – Modified pilot splitting pattern for AMC MIMO: both UL and DL.

- C80216maint-08/089 – Generalization of allocation granularity definitions: this contribution generalizes the allocation granularity definitions that were defined for PUSC+MIMO+dedicated pilots to AMC.

- C80216maint-08/090 - Correction to MIMO Pilot and Data Power

Text Changes

11.8.3.7.5 OFDMA SS demodulator for MIMO support

[Change in the table as follows]

Bit #20: Allocation granularity in a DL PUSC/AMC STC zone with dedicated pilots

Bit #21: Concurrent allocation support in a DL PUSC/AMC STC zone with dedicated pilots

[Change the text below the table as follows]

Bits #20,#21 define allocation granularity support for PUSC/AMC STC zones with dedicated pilots. They do not apply to single antenna or single stream operation. The granularity is a function of the slot duration and the pilot period. For PUSC-STC with 2 antennas, the slot duration equals 2 symbols and the pilot period equals 4 symbols. For AMC NxM in STC zone the slot duration equals M symbols and the pilot period equals 6 symbols.

If bit #20 is set to 1, the allocations for an MS in a DL PUSC/AMC STC zone with dedicated pilots must meet the following constraints, applicable to both non-HARQ and HARQ sub-bursts:

1) The allocation must be a rectangle.

2) The smallest OFDMA symbol number of the allocation must be a multiple of the pilot period two slot-durations (4 symbols) relative to the smallest OFDMA symbol number of the zone.

3) The time duration of the allocation must be a multiple of the pilot period two slot-durations (4 symbols).

If bit #20 is set to 0, it indicates that the MS supports a granularity of one slot-duration (2 symbols for DL PUSC) for an allocation in any DL STC zone with dedicated pilots so long as the allocation in each subchannel in AMC and/or in each major group in PUSC is equal to or larger than the pilot period the minimum allocation duration of two slot-durations (4 symbols for DL PUSC).

If bit #21 is set to 1, no two allocations for the MS may occupy the same slot duration (2 symbols) in a DL PUSC/AMC STC zone with dedicated pilots. If bit #21 is set to 0, the MS can support multiple allocations in a given slot duration in any DL STC zone with dedicated pilots.