| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Allocation granularity definitions for MIMO AMC |  |
| Date Submitted | 2008-06-19 |  |
| Source(s) | Yuval Lomnitz Dov Andelman Ayelet Doron Assaf Gurevitz Intel Corp. | Voice: +972-3-9205773 E-mail: Yuval.lomnitz@intel.com dov.andelman@intel.com ayelet.doron@intel.com assaf.gurevitz@intel.com |
| Re: | Letter Ballot 26d |  |
| Abstract | This contribution is intended for completing definitions of AMC for MIMO zone and is concerned with the allocation granularity, with accordance with TWG resolutions. |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Allocation granularity definitions for MIMO-AMC

Yuval Lomnitz, Dov Andelman, Ayelet Doron, Assaf Gurevitz - Intel Corp.

# Purpose

Allocation granularity definitions were made for the case of PUSC+MIMO with dedicated pilots, since the fact that slot duration (2 symbols) is shorter than the pilot period (4 symbols) can result in allocations without sufficient pilots. For AMC 2x3 in STC zone with dedicated pilots the same situation may occur for matrix B allocations. This contribution generalizes the allocation granularity definitions that were defined for PUSC + MIMO + dedicated pilots to AMC (or any other permutation) and adds capability bits for AMC.

For AMC:

- bit #22 relates to allocation granularity of matrix B bursts, which can be 6+6*n (when bit#22 is set to 1) or 6+3*n (when bit#22 is set to 0).

- bit #23 relates to concurrency of bursts with the different ranks (rank1 and rank2), which can be not concurrent (when bit#23 is set to 1) or concurrent bursts (when bit#23 is set to 0).

Text Changes

11.8.3.7.5 OFDMA SS demodulator for MIMO support

[Change in the table as follows]

Bit #22–23: Reserved

Bit #22: Allocation granularity in a DL AMC STC zone with dedicated pilots for Matrix B

Bit #23: Concurrent allocation support in a DL AMC STC zone with dedicated pilots for bursts with different ranks

[Change the text below the table as follows]

Bits #20,#21 and #22,#23 define allocation granularity support for PUSC and AMC STC zones with dedicated pilots, respectively. They do not apply to single antenna or single stream operation. In AMC STC zone bit #22 applies only to bursts with Matrix B and bit #23 applies only to bursts with different ranks (different number of streams).

The granularity is a function of the slot duration and the pilot period. For PUSC-STC with 2 antennas, the slot duration equals 2 symbols and the pilot period equals 4 symbols. For AMC 2x3 in STC zone the slot duration equals 3 symbols and the pilot period equals 6 symbols.

If bit #20 or bit #22 are is set to 1, the allocations for an MS in a DL PUSC or AMC STC zone with dedicated pilots respectively, must meet the following constraints, applicable to both non-HARQ and HARQ sub-bursts:

1) The allocation must be a rectangle.

2) The smallest OFDMA symbol number of the allocation must be a multiple of the pilot period two slot-durations (4 symbols) relative to the smallest OFDMA symbol number of the zone.

3) The time duration of the allocation must be a multiple of the pilot period two slot-durations (4 symbols).

If bit #20 or bit #22 are is set to 0, it indicates that the MS supports a granularity of one slot-duration (2 symbols for DL PUSC or 3 symbols for DL AMC, respectively) for an allocation in any DL STC zones with dedicated pilots, so long as the allocation in each subchannel in AMC and/or in each major group in PUSC is equal to or larger than the pilot period the minimum allocation duration of two slot-durations (4 symbols for DL PUSC).

If bit #21 or bit #23 are is set to 1, no two allocations for the MS may occupy the same slot duration (2 symbols) in a DL PUSC or AMC STC zones with dedicated pilots, respectively. For AMC this restriction applies only to bursts with different ranks, i.e. bursts with the same rank may be concurrent. If bit #21 or bit #23 are is set to 0, the MS can support multiple allocations in a given slot duration in any DL PUSC or AMC STC zones with dedicated pilots, respectively.