| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reallocation scheme in Persistent Allocation |  |
| Date Submitted | 2008-03-07 |  |
| Source(s) | Sungkyung Kim Sungcheol Chang Kwangjae Lim Chulsik Yoon | Voice: +82-42-860-6448 E-mail: cyrano@etri.re.kr scchang@etri.re.kr kjlim@etri.re.kr csyoon@etri.re.kr |
| Re: | LB26a |  |
| Abstract | The resource allocation scheme of 802.16e is extended to enable persistent allocations (also referred to as periodic allocations). |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reallocation scheme in Persistent Allocation

# 1. Introduction

Persistent allocation scheme is a relevant solution for increase in VoIP capacity. It can greatly reduce the periodic MAP overhead since allocation IE only appears once with the fixed resource position and fixed transmission format during the some available time. Many contributions to persistent allocation were issued in last meeting and have been in the way of harmonization by Rapporteur Group [PA]. However, in case of reallocation by AMC or/and change of buffer status, the MAP IE format relative to the reallocation in persistent allocation still has some redundancy. This contribution proposes the modified MAP IE format which can eliminate these redundancies.

# 2. Proposed Solution

In order to relinquish the allocated radio resources during voice inactive interval, many allocations and de-allocations will take place in VoIP service using persistent allocation scheme. Also, the changes of MCS for link adaptation owing to channel variations need the reassignment of persistent allocation. As one solution to AMC in persistent allocation scheme, we may consider that new persistent allocation will be made according to the updated MCS after the existing persistent resource is released. Figure 1 and Figure 2 show the examples of resource re-assignment in persistent allocation. As shown in Figure 1, in case that the MS-2 requires more slots because of poor channel condition, the MAP should include not only information fields of de-allocation/repacking but also information fields of allocation which appear in a persistent sub-burst IE.

Figure 1. De-allocation/allocation for the extension of MS-2’s persistent resource

Figure 2 shows the case that some slots allocated to MS-2 is reduced in persistent allocation. Like the preceding example in Figure 1, the fields for both de-allocation/repacking and allocation are needed. These two separate repacking and re-allocation will generate some overheads which can be, however, minimized with careful consideration that both processes and closely coupled.

Figure 2. De-allocation/allocation for the shrink of MS-2’s persistent resource

This contribution proposes a modified persistent sub-burst IE which can perform both reallocation and repacking using one bundle of information fields. Figure 3 shows the re-arrangement of persistent allocation using the proposed IE format.

The proposed IE formats are shown on the session of proposed text in detail. “Delta Duration” field in modified persistent sub-burst IE provides information about reallocation and repacking. In case that RCID of MS is equal to RCID value in the IE relative to the bundle of information fields, the corresponding MS should update its own allocation information using “Delta Duration” field. Otherwise, this field should be also interpreted as repacking by the MSs.

Moreover, 1 MSB of “Delta Duration” field indicates the direction of resource rearrangement. When it is set to 1, then the reallocated slots could be increased such as the case of the resource extension of MS-2 in Figure 3. Here we note that slot offset for repacking should be the sum of the difference of duration(other LSBs of “Delta Duration” field) and its own slot offset, ensuring that repacked and reallocated region dose not cause some collisions with the involved MSs. Otherwise, in case that it is set to 0, then reallocated slots could be decreased and the direction of repacking is equal to the direction of repacking in de-allocation case.

Accordingly, the proposed persistent sub-burst IE can effectually reduce some MAP overhead while the existing persistent scheme is still working in MSs.

Figure 3. Reallocation for MS-2’s persistent resource

# 3. Proposed Text

[Add new section to 8.4.5.3.28]

8.4.5.3.28 HARQ DL MAP Persistent Allocation IE

Table XXX: Persistent HARQ DL MAP allocation IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent HARQ DL MAP IE () { | -- | -- |
| Extended-2 DIUC | 4 | Persistent HARQ DL MAP IE = TBD |
| Length | 8 | Length in bytes |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| OFDMA Symbol offset | 8 | Offset from the start of DL sub-frame |
| Subchannel offset | 7 |  |
| Number of OFDMA symbols | 7 |  |
| Number of subchannels | 7 |  |
| Rectangular sub-burst indication | 1 | Indicates sub-burst allocations are time-first rectangular. The duration field in each sub-burst IE specifies the number of subchannels for each rectangular allocation. This is only valid for AMC allocations and all allocations with dedicated pilots. When this field is clear, sub-bursts shall be allocated in frequency-first manner and the duration field reverts to the default operation |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition TLV in DCD |
| } | -- | -- |
| Mode | 4 | Indicates the mode in this HARQ region 0b0000: Persistent Chase HARQ 0b0001: Persistent Incremental redundancy HARQ for CTC 0b0010: Persistent Incremental redundancy HARQ for Convolutional Code 0b0011: Persistent MIMO Chase HARQ 0b0100: Persistent MIMO IR HARQ 0b0101: Persistent MIMO IR HARQ for Convolutional Code 0b0110: Persistent MIMO STC HARQ 0b0111 to 0b1111 : reserved |
| Subburst IE Length | 8 | Length, in nibbles, to indicate the size of the subburst IE in this HARQ mode. The MS may skip DL HARQ Subburst IE if it does not support the HARQ mode. However, the MS shall decode NACK Channel field from each DL HARQ Subburst IE to determine the UL ACK channel it shall use for its DL HARQ burst. |
| if (Mode == 0b0000) { | -- | -- |
| Persistent Chase HARQ sub-burst IE | variable | -- |
| } else if (Mode == 0b0001) { |  |  |
| Persistent Incremental redundancy HARQ for CTC sub-burst IE | variable | -- |
| } else if (Mode == 0b0010) { | -- | -- |
| Persistent Incremental redundancy HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0011) { | -- | -- |
| Persistent MIMO Chase HARQ | variable | -- |
| } else if (Mode == 0b0100) { |  |  |
| Persistent MIMO IR HARQ | variable | -- |
| } else if (Mode == 0b0101) { | -- | -- |
| Persistent MIMO IR HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0110) { |  |  |
| Persistent MIMO STC HARQ | variable |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| } |  |  |

### Table YYY - Persistent DL HARQ Chase Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_DL_HARQ_Chase_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| For (j=1;j<Number of sub bursts; j++) { |  |  |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| RCID_IE() | variable |  |
| If (Allocation Flag == 0) { |  |  |
| Repacking Flag | 1 | 0 = no repacking 1 = repacking |
| RCID_IE() | variable | -- |
| if (Repacking Flag ==1) { |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID_IE() | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| Repacking Flag | 1 | 0 = no repacking 1 = repacking Note: The case of non-persistent should not be repacked. |
| If (Repacking Flag == 0) { |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to the first allocation instance only; [Mo-Han please elaborate as to what you would like say here…] |
| Sub-burst DIUC indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Sub-burst DIUC indicator ==1) { |  |  |
| DIUC | 4 | - |
| Repetition coding indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } |  |  |
| ACID | 4 | - |
| AI_SN | 1 | Initial AI_SN for each ACID |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (ACK disable== 0) { |  |  |
| ACK channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| } |  |  |
| Dedicated DL control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of dedicated DL control indicator ==1){ |  | - |
| Duration (d) | 4 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d–1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE () | variable |  |
| } |  |  |
| } |  |  |
| If (Repacking Flag == 1) { |  |  |
| Delta duration | variable | Difference of duration between previous allocation and newly allocation (reallocation) in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame This field provides information about both reallocation and repacking. In case that RCID of a MS involved in the persistent allocation is equal to a value of RCID_IE, the MS should update allocation information from this value. Otherwise, the MS should interpret this field as repacking. [The case of reallocation ] - 1 MSB: 0 == extension, 1 == shrink - Other LSBs: the difference between new allocation and previous allocation. [The case of repacking] - 1 MSB: 0 == negative duration, 1 == positive duration - Other LSBs: the amount of repacking. In case of negative duration, slot offset after repacking will be higher value. Otherwise, in case of positive duration, this is the same operation of normal repacking to eliminate resource holes. |
| Slot Offset | variable | 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Sub-burst DIUC indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Sub-burst DIUC indicator ==1) { |  |  |
| DIUC | 4 | - |
| Repetition coding indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |

Table YYY – Persistent DL HARQ IR CTC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_DL_HARQ_IR_CTC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| For (j=1;j<Number of sub bursts; j++) { |  |  |
| RCID_IE() | variable |  |
| Allocation Flag | 1 | 0 = de-allocate 1 = allocate |
| If (Allocation Flag == 0) { |  |  |
| Repacking Flag | 1 | 0 = no repacking 1 = repacking |
| RCID_IE() | variable | -- |
| if (Repacking Flag ==1) { |  |  |
| NEP | 4 |  |
| NSCH | 4 |  |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| Repacking Flag | 1 | 0 = no repacking 1 = repacking Note: The case of non-persistent should not be repacked. |
| If (Repacking Flag == 0) { |  |  |
| NEP | 4 |  |
| NSCH | 4 |  |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to the first allocation instance only; [Mo-Han please elaborate as to what you would like say here…] |
| Nep | 4 | - |
| Nsch | 4 |  |
| SPID | 2 |  |
| ACID | 4 | - |
| AI_SN | 1 | Initial AI_SN for each ACID |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (ACK disable== 0) { |  |  |
| ACK channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| } |  |  |
| Dedicated DL control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of dedicated DL control indicator ==1){ |  | - |
| Duration (d) | 4 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d–1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE () | variable |  |
| } |  |  |
| } |  |  |
| If (Repacking Flag == 1) { |  |  |
| NEP | 4 |  |
| Delta NSCH | variable | Difference of duration between previous allocation and newly allocation (reallocation) in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame This field provides information about both reallocation and repacking. In case that RCID of a MS involved in the persistent allocation is equal to a value of RCID_IE, the MS should update allocation information from this value. Otherwise, the MS should interpret this field as repacking. [The case of reallocation ] - 1 MSB: 0 == extension, 1 == shrink - Other LSBs: the difference between new allocation and previous allocation. [The case of repacking] - 1 MSB: 0 == negative duration, 1 == positive duration - Other LSBs: the amount of repacking. In case of negative duration, slot offset after repacking will be higher value. Otherwise, in case of positive duration, this is the same operation of normal repacking to eliminate resource holes. |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |

Table YYY – Persistent DL HARQ IR CC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_DL_HARQ_IR_CC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| For (j=1;j<Number of sub bursts; j++) { |  |  |
| RCID_IE() | variable | -- |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| If (Allocation Flag == 0) { |  |  |
| Repacking Flag | 1 | 0 = no repacking 1 = repacking |
| RCID_IE() | variable | -- |
| if (Repacking Flag ==1) { |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID_IE() | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| Repacking Flag | 1 | 0 = no repacking 1 = repacking Note: The case of non-persistent should not be repacked. |
| If (Repacking Flag == 0) { |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable |  |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| MAP NACK Channel index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to the first allocation instance only; [Mo-Han please elaborate as to what you would like say here…] |
| Sub-burst DIUC indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Sub-burst DIUC indicator==1) { |  |  |
| DIUC | 4 | - |
| Repetition coding indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } |  |  |
| ACID | 4 | -- |
| AI_SN | 1 | Initial AI_SN for each ACID |
| SPID | 2 | -- |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (ACK disable== 0) { |  |  |
| ACK channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| } |  |  |
| Dedicated DL control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of dedicated DL control indicator ==1){ |  | - |
| Duration (d) | 4 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d–1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE () | variable |  |
| } |  |  |
| } |  |  |
| If (Repacking Flag == 1) { |  |  |
| Delta duration | variable | Difference of duration between previous allocation and newly allocation (reallocation) in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame This field provides information about both reallocation and repacking. In case that RCID of a MS involved in the persistent allocation is equal to a value of RCID_IE, the MS should update allocation information from this value. Otherwise, the MS should interpret this field as repacking. [The case of reallocation ] - 1 MSB: 0 == extension, 1 == shrink - Other LSBs: the difference between new allocation and previous allocation. [The case of repacking] - 1 MSB: 0 == negative duration, 1 == positive duration - Other LSBs: the amount of repacking. In case of negative duration, slot offset after repacking will be higher value. Otherwise, in case of positive duration, this is the same operation of normal repacking to eliminate resource holes. |
| Slot Offset | variable | 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Sub-burst DIUC indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Sub-burst DIUC indicator ==1) { |  |  |
| DIUC | 4 | - |
| Repetition coding indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |