| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reallocation Scheme in Persistent Allocation |  |
| Date Submitted | 2008-03-19 |  |
| Source(s) | Sungkyung Kim, Sungcheol Chang, Kwangjae Lim, Chulsik Yoon, Seokheon Cho, Sunggeun Jin, Hyun Lee | Voice: +82-42-860-6448 E-mail: cyrano@etri.re.kr scchang@etri.re.kr kjlim@etri.re.kr csyoon@etri.re.kr *<http://stardards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot 26b |  |
| Abstract | The resource allocation scheme of 802.16e is extended to enable persistent allocations (also referred to as periodic allocations). |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reallocation Scheme in Persistent Allocation

Sungkyung Kim, Sungcheol chang, Kwangjae Lim, Chulsik Yoon,

Seokheon cho, Sunggeun Jin, Hyun Lee,

ETRI

# 1. Introduction

Persistent allocation scheme is a relevant solution to increase VoIP capacity. It can significantly reduce the periodic MAP overhead since allocation IE only appears once with the fixed resource position and fixed transmission format during an available time. Many contributions to persistent allocation were issued in last meeting and have been treated in the way of harmonization by Rapporteur Group [PA]. However, in case of reallocation by AMC or/and change of buffer status, the MAP IE format relative to the reallocation in persistent allocation still has redundancies. This contribution proposes the modified MAP IE format which can eliminate these redundancies.

Through the harmonization process, the MAP NACK channel has been defined as a shared physical channel which indicates MAP decoding errors from MSs involved with the corresponding persistent allocation. Upon receiving a MAP NACK indication, a BS shall resend an updated persistent allocation IE just in case that the persistent allocation is changed from the previous frame. With implicit indication of no change, the MS shall resume using the previous persistent resource allocation. When MAP NACK channel errors occur in the BS, there is still a possibility of collision on uplink persistent region. Hence, this contribution also considers the MAP IE format to solve the collision problem in uplink sub-frame.

# 2. Proposed Solution

MAP Overhead Reduction

In order to relinquish the allocated radio resources during voice inactive interval, many allocations and de-allocations will take place in VoIP service using persistent allocation scheme. Also, the changes of MCS for link adaptation owing to channel variations need the reassignment of persistent allocation.

Figure 1. De-allocation/allocation for the extension of MS-2’s persistent resource

As one solution to AMC in persistent allocation scheme, we may consider that new persistent allocation will be made according to the updated MCS after the existing persistent resource is released. Figure 1 and Figure 2 show the examples of resource re-assignment in persistent allocation. As shown in Figure 1, in case that the MS-2 requires more slots because of poor channel condition, the MAP should include not only information fields of de-allocation/repacking but also information fields of allocation which appear in a persistent sub-burst IE.

Figure 2. De-allocation/allocation for the shrink of MS-2’s persistent resource

Figure 2 shows the case that some slots allocated to MS-2 is reduced in persistent allocation. Like the preceding example in Figure 1, the fields for both de-allocation/repacking and allocation are needed. These two separate repacking and re-allocation will generate some overheads which can be, however, minimized with careful consideration that both processes and closely coupled.

This contribution proposes a modified persistent sub-burst IE which can perform both reallocation and repacking using one bundle of information fields. Figure 3 shows the re-arrangement of persistent allocation using the proposed IE format.

Figure 3. Reallocation for MS-2’s persistent resource

The proposed IE formats are shown on the session of proposed text in detail. “Delta Duration” field in modified persistent sub-burst IE provides information about reallocation and repacking. In case that RCID of MS is equal to RCID value in the IE relative to the bundle of information fields, the corresponding MS should update its own allocation information using “Delta Duration” field. Otherwise, this field should be also interpreted as repacking by other MSs.

Moreover, 1 MSB of “Delta Duration” field indicates the direction of resource rearrangement. When it is set to 1, the reallocated slots could be increased such as the case of the resource extension of MS-2 in Figure 3. Here, we note that slot offset for repacking should be the sum of the difference of duration (other LSBs of “Delta Duration” field) and its own slot offset, ensuring that the repacked and reallocated region do not cause some collisions with the involved MSs. Otherwise, in case that it is set to 0, reallocated slots could be decreased and the direction of repacking is equal the same as the direction of repacking in de-allocation case.

Accordingly, the proposed persistent sub-burst IE can effectually reduce MAP overhead while the existing persistent scheme is still working in MSs.

Collision Avoidance of Persistent Allocation in UL Sub-frame

According to MAP error detection scheme using MAP NACK channel, a MS will transmit a MAP NACK signal through the Fast Feedback channel region when that it does not successfully receive the MAP in a Persistent allocation frame. However, there can be some ambiguity in persistent allocation in case that the BS fails to receive the MAP NACK signal at frame N and the MS which transmitted MAP NACK signal relevant to the frame N successfully receive the MAP at frame N+Allocation Period. When the MS decodes the MAP without any change in UL persistent allocation at frame N+Allocation Period, the MS resumes its own persistent allocation. However, MS’s burst transmission on the UL persistent allocation can cause a sequential collision with other transmission if the MAP at frame N included some changes. Hence, we focus on the case of UL persistent allocation by reason of possibility that sequential collisions may occur in such a circumstance.

We consider an approach of change counter in order to keep away from the collision problem caused by MAP NACK channel errors. As shown in Figure 4, a BS increases a PCC (Persistent Change Counter) relevant to the persistent allocation group as resource allocation arrangement is changed. It is noted that the PCC value can be increased by one in an allocation period even though the allocation IE includes many reallocations and/or rearrangements. A MS involved in uplink persistent allocation group should store its PCC value and update it when the change of persistent allocation occurs. When decoding the MAP in the next period after MAP error detection, if MS’s own PCC value is not equal to the PCC in persistent allocation IE transmitted from the BS, the MS should transmit a NACK in the shared MAP NACK channel in order to request for a retransmission of the updated persistent allocation IE. The MS shall not transmit any persistently allocated burst until it successfully receives the updated persistent allocation IE.

Figure 4. Persistent Chang Counter in UL sub-frame.

# 3. Proposed Text

[The following modifications are proposed for C802.16maint-08/95r4]

6.3.27.4  MAP NACK Channel

6.3.27.4.1 Downlink Operation

If the MS fails to decode the DL-MAP in a frame which is relevant to a frame in which it has a persistent DL resource allocation (frame K), the MS shall not transmit data bursts or control signals in the relevant UL sub-frame (including CDMA ranging, CQICH, HARQ ACK/NAK or sounding signals) and may send a MAP NACK on its assigned MAP NACK Channel in the frame following the relevant UL sub-frame (K+2) as shown in Figure YYY.  However, if the MS fails to decode UL-MAP in frame K+1, the MS shall not send a MAP NAK in frame K+2 on its assigned MAP NACK Channel.

Figure YYY: Example MAP NACK Relevance

Upon reception of the MAP NACK indication, the BS should determine if the persistent allocation needs to be updated.  The MS that sent the NACK corresponding to a persistent allocation in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true:

- The MS receives a Persistent HARQ DL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ DL MAP IE, which has the same Change Counter as the MS has.

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Counter set to (N+1) mod 4 and the Retransmission Flag of at least one sub-burst IE is set to 1, when the MS has received a Persistent HARQ DL MAP IE with Change Counter set to N in past frames.

The MS shall transmit the MAP NACK corresponding to a persistent allocation in frame K+ap if the MS receives a Persistent HARQ DL MAP IE with Change Counter set to (N+2) mod 4, or (N+3) mod 4, and the Retransmission Flag set to 1, when the MS has successfully received a Persistent HARQ DL MAP IE with Change Counter set to N.

Otherwise, the MS shall transmit an indication to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

If the MS fails to decode the DL-MAPs in four successive frames K, K + ap, K + 3 x ap, K + 3 x ap, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

6.3.27.4.2 Uplink Operation

If the MS fails to decode the UL-MAP in a frame which is relevant to a frame in which it has a persistent UL resource allocation (frame K), the MS shall not transmit data bursts or control signals in the relevant UL sub-frame (including CDMA ranging, CQICH, HARQ ACK/NAK or sounding signals) and may send a MAP NACK on its assigned MAP NACK Channel in the frame following the relevant UL sub-frame as shown in Figure YYY.

Upon reception of the MAP NACK indication, the BS should determine if the persistent allocation needs to be updated.   The MS that sent the NACK corresponding to a persistent allocation in frame K shall resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true:

- he MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID.

- The MS receives a Persistent HARQ UL MAP IE, which has the same Change Counter as the MS has.

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Counter set to (N+1) mod 4 and the Retransmission Flag of at least one sub-burst IE is set to 1, when the MS has received a Persistent HARQ UL MAP IE with Change Counter set to N in past frames.

The MS shall transmit the MAP NACK corresponding to a persistent allocation in frame K+ap if the MS receives a Persistent HARQ UL MAP IE with Change Counter set to (N+2) mod 4, or (N+3) mod 4, and the Retransmission Flag set to 1, when the MS has successfully received a Persistent HARQ UL MAP IE with Change Counter set to N.

Otherwise, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation and the MAP ACK channel allocation.

If the MS fails to decode the UL-MAPs in four successive frames K, K + ap, K + 3 x ap, K + 3 x ap, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

6.3.27.3 Error Correction Information

If a MAP NACK is detected at the BS, the BS may either make absolute updates to all affected MSs or retransmit de-allocation command of frame K - allocation period by sending sub-burst IE with the Retransmission Flag set to 1. It is left to vendor’s implementation which types of command, i.e., deallocation with retransmission flag=1 or absolute updates to all affected MSs, the base station send. An MS shall be able to recover persistent allocation by both type of commands.

6.3.27.4  Change Counter

A Change Counter is included in the Persistent HARQ DL MAP IE. Change Counter shall be increased by one if the Persistent HARQ DL MAP IE includes any changes in the corresponding persistent allocation. In case that there is no change to DL persistent allocation in the current frame, the Persistent Change Counter shall be not increased.

A Change Counter is included in the Persistent HARQ UL MAP IE. Change Counter shall be increased by one if the Persistent HARQ UL MAP IE includes any changes in the corresponding persistent allocation. In case that there is no change to UL persistent allocation in the current frame, the Persistent Change Counter shall be not increased.

Change Counter in a current frame is used for MS who failed to receive the DL-MAP or UL-MAP at the previous frames. When MS receives a Persistent HARQ DL MAP IE or Persistent UL HARQ MAP IE with the same Change Counter as the MS has, the MS shall continue the DL or UL persistent allocation.

[Add new section to 8.4.5.3.28]

8.4.5.3.28 HARQ DL MAP Persistent Allocation IE

Table XXX: Persistent HARQ DL MAP allocation IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent HARQ DL MAP IE () { | -- | -- |
| Extended-2 DIUC | 4 | Persistent HARQ DL MAP IE = 0D |
| Length | 8 | Length in bytes |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this Persistent HARQ DL MAP (FDD/H-FDD Only) |
| Change Counter | 2 | Change Counter. CC value is increased by one if the persistent IE includes any changes in the corresponding group persistent allocation. If there is no change, the CC value does not increase. If a MS’s CC is not equal to this value, the MS shall transmit a MAP NACK signal in a relevant MAP NACK channel. |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| OFDMA Symbol offset | 8 | Offset from the start of DL sub-frame |
| Subchannel offset | 7 |  |
| Number of OFDMA symbols | 7 |  |
| Number of subchannels | 7 |  |
| Rectangular sub-burst indication | 1 | Indicates sub-burst allocations are time-first rectangular. The duration field in each sub-burst IE specifies the number of subchannels for each rectangular allocation. This is only valid for AMC allocations and all allocations with dedicated pilots. When this field is clear, sub-bursts shall be allocated in frequency-first manner and the duration field reverts to the default operation |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition TLV in DCD |
| } | -- | -- |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to the first allocation instance only; |
| Mode | 4 | Indicates the mode in this HARQ region 0b0000: Persistent Chase HARQ 0b0001: Persistent Incremental redundancy HARQ for CTC 0b0010: Persistent Incremental redundancy HARQ for Convolutional Code 0b0011: Persistent MIMO Chase HARQ 0b0100: Persistent MIMO IR HARQ 0b0101: Persistent MIMO IR HARQ for Convolutional Code 0b0110: Persistent MIMO STC HARQ 0b0111 to 0b1111 : reserved |
| Subburst IE Length | 8 | Length, in nibbles, to indicate the size of the subburst IE in this HARQ mode. The MS may skip DL HARQ Subburst IE if it does not support the HARQ mode. However, the MS shall decode NACK Channel field from each DL HARQ Subburst IE to determine the UL ACK channel it shall use for its DL HARQ burst. |
| if (Mode == 0b0000) { | -- | -- |
| Persistent Chase HARQ sub-burst IE | variable | -- |
| } else if (Mode == 0b0001) { |  |  |
| Persistent Incremental redundancy HARQ for CTC sub-burst IE | variable | -- |
| } else if (Mode == 0b0010) { | -- | -- |
| Persistent Incremental redundancy HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0011) { | -- | -- |
| Persistent MIMO Chase HARQ | variable | -- |
| } else if (Mode == 0b0100) { |  |  |
| Persistent MIMO IR HARQ | variable | -- |
| } else if (Mode == 0b0101) { | -- | -- |
| Persistent MIMO IR HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0110) { |  |  |
| Persistent MIMO STC HARQ | variable |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| } |  |  |

Change Counter

Change Counter shall be increased by one if the Persistent HARQ DL MAP IE includes any changes in the corresponding persistent allocation. In case that there is no change to DL persistent allocation in the current frame, the Change Counter shall be not increased.

[Add new section to 8.4.5.4.29]

8.4.5.4.29 HARQ UL MAP Persistent Allocation IE

- Table YYY - Persistent HARQ UL MAP IE

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| Persistent HARQ_UL-MAP_IE() { |  |  |
| Extended 2- UIUC | 4 | Persistent HARQ_UL-MAP_IE() = 0x0B |
| Length | 8 | Length of the IE |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| Change Counter | 2 | Change Counter. CC value is increased by one if the persistent IE includes any changes in the corresponding group persistent allocation. If there is no change, the CC value does not increase. If a MS’s CC is not equal to this value, the MS shall transmit a MAP NACK signal in a relevant MAP NACK channel. |
| Reserved | 1 |  |
| While (data remains) { | - | - |
| Mode | 3 | Indicates the mode of this IE: 0b000: Persistent Chase HARQ 0b001: Persistent Incremental redundancy HARQ for CTC 0b010: Persistent Incremental redundancy HARQ for convolutional code 0b011: Persistent MIMO Chase HARQ 0b100: Persistent MIMO IR HARQ 0b101: Persistent MIMO IR HARQ for convolutional code 0b110: Persistent MIMO STC HARQ 0b111: Reserved |
| Allocation Start Indication | 1 | 0: No allocation start information 1: Allocation start information follows |
| If (Allocation Start Indication ==1) { | - | - |
| OFDMA Symbol offset | 8 | This value indicates start symbol offset of subsequent subbursts in this Persistent HARQ UL MAP IE with reference to the start of the UL sub-frame. |
| Subchannel offset | 7 | This value indicates start Subchannel offset of subsequent subbursts in this Persistent HARQ UL MAP IE |
| Reserved | 1 | Shall be set to zero |
| } | - | - |
| N sub-bursts | 4 | Number of sub-bursts in this Persistent HARQ UL MAP IE is this field value plus 1. |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (i=0;i<Number of sub-burst; i++) { |  |  |
| If (mode == 000) { |  |  |
| Persistent UL HARQ Chase Sub-Burst IE () |  |  |
| } else if (mode == 001) { |  |  |
| Persistent UL HARQ IR CTC Sub-Burst IE () |  |  |
| } else if (mode == 010) { |  |  |
| Persistent UL HARQ IR CC Sub-Burst IE () |  |  |
| } else if (mode == 011) { |  |  |
| Persistent MIMO UL Chase HARQ Sub-Burst IE () |  |  |
| } else if (mode == 100) { |  |  |
| Persistent MIMO UL IR HARQ Sub-Burst IE () |  |  |
| } else if (mode == 101) { |  |  |
| Persistent MIMO UL IR HARQ for CC Sub-Burst IE () |  |  |
| } else if (mode == 110) { |  |  |
| Persistent MIMO UL STC HARQ Sub-Burst IE () |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to byte; shall be set to 0 |
| } |  |  |

- Resource Shifting Indicator

If the resource shifting indicator is set to 1, the MS shall shift its persistent resource position by the accumulated slots as indicated by de-allocation commands with slot offsets smaller than its own.

Change Counter

Change Counter shall be increased by one if the Persistent HARQ UL MAP IE includes any changes in the corresponding persistent allocation. In case that there is no change to UL persistent allocation in the current frame, the Change Counter shall be not increased.