| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fix for ss request/grant flow chart |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Li Wang, ZTE Corporation | Voice: E-mail: wangli@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16Rev2/D3, Letter Ballot 26a. Technical Comments |  |
| Abstract | This contribution proposed clarification for flow chart of ss request/grant. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fix for ss request/grant flow chart

Li Wang

ZTE Corporation

# Problem description

In the current 802.16 standard , section 6.3.6, Figure 55, when MS processes UL-MAP and finds no grant for basic CID, MS proceeds to the next step to await SDU arrival. We think that is error. We propose that MS should proceeds to the next step to begin contention-based CDMA BRs when MS finds no grant for basic CID.

Proposed changes

We propose that MS should proceeds to the next step to begin contention-based CDMA BRs when MS finds no grant for basic CID.

Change #1:

Replace Figure 55 in 6.3.6 with the follow figure:

Figure 55—SS Request/Grant flow chart

# References

IEEE Std 802.16Rev2-D3 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” January, 2008