| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixes for Lost of MOB_HO-IND (Cancel) Messages |  |
| Date Submitted | 2008-03-19 |  |
| Source(s) | Chi-Chen Lee, I-Kang Fu MediaTek Inc. No. 1, Dusing Rd. 1 Science-Based Industrial Park, Hsinchu, Taiwan 300 Nadav Lavi Alvarion | chichen.lee@mediatek.com IK.Fu@mediatek.com Nadav.Lavi@alvarion.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26b, on P802.16Rev2/D3, as announced in IEEE 802.16-08/006. |  |
| Abstract | In IEEE 802.16 Rev2/D3, there is no definition on MS’s/BS’s behavior when the message MOB_HO-IND message with the HO Cancel commend is lost. This contribution explains the potential problems and proposes a resolution to resolve this confusion. |  |
| Purpose | Accept the proposed text modification to IEEE 802.16 Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fixes for Lost of MOB_HO-IND (Cancel) Messages

Chi-Chen Lee, I-Kang Fu

MediaTek Inc.

Nadav Lavi

Alvarion

# Problem Statement

There is no ACK from BS when MS tries to cancel HO by sending MOB_HO-IND message. Thus, if the MOB_HO-IND with HO_IND_type=0b01 (cancel) is lost, the MS and serving BS will at asynchronous state since MS intends to stay on serving BS after HO cancel; and further, there is no way for the MS to know whether the MOB_HO-IND message is lost. The problematic scenarios are described in Figure 1 to Figure 7.

Figure 1 – Lost of MOB_HO-IND (cancel) message after MOB_MSHO-REQ before MOB_BSHO-RSP message

Figure 2 – Lost of MOB_HO-IND (cancel) message after MOB_BSHO-RSP message

Figure 3 – Lost of MOB_HO-IND (cancel) message after MOB_HO-IND (release) message during MS initiated HO

Figure 4 – Lost of MOB_HO-IND (cancel) message after MOB_BSHO-RSP message

Figure 5 –Lost of MOB_HO-IND (cancel) message after MOB_HO-IND (release) message during BS initiated HO

Figure 6 – Lost of MOB_HO-IND (cancel) message when MS detects drop

Figure 7 –MOB_HO-IND (cancel) message is received but when MS context was deleted at serving BS

Figure 7 points out the issue that drop detection at BS and MS may be different and even the MOB_HO-IND message is received correctly at serving BS it still may be discard due to the MS context no longer exists. In this case, the MS has no idea that SBS has deleted its context and may continue requesting bandwidth.

Figure 8 shows the potential security issue if the MOB_HO-IND (cancel) message is lost. If the serving BS that received MOB_HO-IND (release) message from the MS before but accept bandwidth request from the MS that should perform handover, it creates a security hole that a malicious MS can cancel HO operation on behave of any MS. However, if the serving BS ignores the bandwidth request, the MS will be blocked until it detects the asynchronous situation.

Figure 8 – Lost of MOB_HO-IND (cancel) message when MS detects drop

Figure 9 – HO operation state machine – MS side

# Suggested Remedy

There are two remedies, remedy one is to clarify that MS should be able to detect the lost MOB_HO-IND (cancel) message and remedy two is to clarify how should MS and BS behave after lost of MOB_HO-IND (cancel) message.

The suggested remedy two as described in Figure 9 is once the MS detects that MOB_HO-IND (cancel) message is lost, it should behave as it detects drop and take action that described in section 6.3.22.2.6 Drops during HO. If the MS decides to return serving BS and the resource retain timer is not expired, it should perform network reentry with serving BS.

Figure 10 –[Remedy 2] Network reentry after lost of MOB_HO-IND (cancel) message

# Suggested Changes in Rev2/D3

-------------------------------------------------------------Start of the Text------------------------------------------------------------

[InRev2/D3, line 27 on page 434, section 6.3.22.2.3, insert the following text]

After sending the MOB_HO-IND message with HO_IND_type = 0b01, the MS shall be able to detect the lost of MOB_HO-IND message. When the MS detects the lost of MOB_HO-IND message it shall react as if it detects drop during HO (6.3.22.2.6). In this case, if MS decides to return to serving BS, it should perform network reentry with serving BS as described in section 6.3.22.2.6.

[InRev2/D3, line 42 on page 435, section 6.3.22.2.6, modify the text as follows]

When the MS has detected a drop during network reentry with a target BS, it may attempt network reentry with its preferred target BS as through Cell Reselection (see 6.3.22.2.1), and which may include resuming communication with the serving BS by sending MOB_HO-IND message with HO_IND type = 0b01 (HO cancel)., or may include performing network reentry with serving BS if it detects (see 6.3.22.2.3) that MOB_HO-IND message with HO_IND_type = 0b01 is lost.

[InRev2/D3, line 45 on page 435, section 6.3.22.2.6, modify the text as follows]

MS shall perform CDMA ranging with target BS or with serving BS if MOB_HO-IND message (HO_IND_type = 0b01) is lost using codes from HO codes domain.

[InRev2/D3, line 55 on page 435, section 6.3.22.2.6, modify the text as follows]

If MS attempts network reentry with the serving BS, the MS shall transmit RNG-REQ with Ranging Purpose Indication TLV Bit#0=1, CMAC_KEY_COUNT TLV with current CMAC KEY COUNT and Serving BS ID TLV the ID of current serving BS. If the serving BS retains the MS context, it shall interpret the MS had sent MOB_HO-IND message with HO_IND_type = 0b01 but the message was lost, and recover the connection to the MS through HO process optimization TLV. If the serving BS has discarded the MS context, the network reentry procedure shall be the same as initial network entry.

-------------------------------------------------------------End of the Text-------------------------------------------------------------