| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixes for Lost of MOB_HO-IND Messages |  |
| Date Submitted | 2008-03-17 |  |
| Source(s) | Chi-Chen Lee, I-Kang Fu MediaTek Inc. No. 1, Dusing Rd. 1 Science-Based Industrial Park, Hsinchu, Taiwan 300 | chichen.lee@mediatek.com IK.Fu@mediatek.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26b, on P802.16Rev2/D3, as announced in IEEE 802.16-08/006. |  |
| Abstract | In IEEE 802.16 Rev2/D3, there is no definition on MS’s/BS’s behavior when the message MOB_HO-IND message with the HO Reject commend is lost. This contribution explains the potential problems and proposes a resolution to resolve this confusion. |  |
| Purpose | Accept the proposed text modification to IEEE 802.16 Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fixes for Lost of MOB_HO-IND (reject) Messages

Chi-Chen Lee, I-Kang Fu

MediaTek Inc.

# Problem Statement

There is no ACK from BS when MS tries to cancel HO by sending MOB_HO-IND message. Thus, if the MOB_HO-IND with HO_IND_type=0b01 (cancel) is lost, the MS and serving BS will at asynchronous state since MS intends to stay on serving BS after HO cancel; and further, there is no way for the MS to know whether the MOB_HO-IND message is lost. The problematic scenarios are described in Figure 1 to Figure 7 in [1].

Also, current standard is not clear about what should MS behave after expiration of T42 when rejecting HO. For example, Figure 149 indicates that the HO exchange ends without any further action at MS after expiration of T42. However, the MS should decide to send the MOB_MSHO-REQ message to start another HO negotiation or send MOB_HO-IND (release) message to perform HO in case of the harsh link quality. The problematic scenarios are described in Figure 1.

Figure 1 – Lost of MOB_HO-IND (reject) message

# Suggested Remedy

There are two remedies, remedy one is to define a method to detect the lost MOB_HO-IND (cancel) message and remedy two is to clarify the MS behavior after expiration of T42 when rejecting HO.

The suggested remedy one is illustrated in Figure 2. An MS shall start T42 after sending MOB_HO-IND (cancel) message and wait for RNG-RSP message with Success status without any other TLV encoding, e.g. PHY adjustment, except CMAC as the acknowledgement of reception of MOB_HO-IND message from serving BS. The BS receives MOB_HO-IND (cancel) message with valid CMAC should send RNG-RSP message with Success status without any other TLV encoding except CMAC to the MS. If the MS does not receive the RNG-RSP message before the expiration of T42, it should consider the MOB_HO-IND (cancel) message is lost.

Figure 2 – [Remedy 1] Detection of lost of MOB_HO-IND (cancel) message

# Suggested Changes in Rev2/D3

-------------------------------------------------------------Start of the Text------------------------------------------------------------

Remedy two text changes:

[InRev2/D3, line 45 on page 433, section 6.3.22.2.2, modify the text as follows]

If the MS signals rejection of serving BS instruction to HO through HO_IND_type field in the MOB_HO-IND set value of 0b10 (HO reject option), the BS may reconfigure the neighbor BS list and retransmit MOB_BSHO-RSP message including a new neighbor BS list. MS shall start T42 after sending MOB_HO-IND message with HO_IND_type = 0b10. When T42 is expired and MS does not receive another MOB_BSHO-REQ or MOB_BSHO-RSP message from BS, MS may either transmit MOB_MSHO-REQ message to renegotiate target BS or transmit MOB_HO-IND message to perform HO.

Remedy one text changes:

[InRev2/D3, line 27 on page 434, section 6.3.22.2.3, insert the following text]

MS shall start T42 immediately after sending the MOB_HO-IND message (HO_IND_type = 0b01). If T42 is expired and MS does not receive RNG-RSP message with Success status without any other TLV encoding except CMAC, it shall consider the MOB_HO-IND message is lost and reacts as if it detects drop during HO (6.3.22.2.6). In this case, if MS decides to return to serving BS, it should perform network reentry with serving BS as described in section 6.3.22.2.6. The BS shall send RNG-RSP message over MS Basic CID with Success status without any other TLV encoding except CMAC when it receives a MOB_HO-IND message with valid CMAC.

[InRev2/D3, on page 1031, section 10.1, modify the following entry in Table 524]

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MS | T42 | MOB_HO-IND timeout when sent with HO_IND_type = 0b10 or 0b01. | - | 150 ms for HO_IND_type = 0b01. | - |

--------------------------------------------------------------End of the Text------------------------------------------------------------

# References

1. “Fixes for Lost of MOB_HO-IND (Cancel) Messages”, C80216maint-08_106r2