| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixes for Lost of MOB_HO-IND Messages |  |
| Date Submitted | 2008-04-18 |  |
| Source(s) | Chi-Chen Lee, I-Kang Fu, Kelvin Chou MediaTek Inc. No. 1, Dusing Rd. 1 Science-Based Industrial Park, Hsinchu, Taiwan 300 | chichen.lee@mediatek.com IK.Fu@mediatek.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26c, on P802.16Rev2/D4, as announced in IEEE 802.16-08/018. |  |
| Abstract | In IEEE 802.16 Rev2/D4, there is no description on how to detect the message MOB_HO-IND message is lost. This contribution explains the potential problems and proposes a resolution to resolve this problem. |  |
| Purpose | Accept the proposed text modification to IEEE 802.16 Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fixes for Lost of MOB_HO-IND Messages

Chi-Chen Lee, I-Kang Fu, Kelvin Chou

MediaTek Inc.

# Problem Statement

There is no ACK from BS when MS tries to cancel HO by sending MOB_HO-IND message. Thus, if the MOB_HO-IND with HO_IND_type=0b01 (cancel) is lost, the MS and serving BS will at asynchronous state since MS intends to stay on serving BS after HO cancel; and further, there is no way for the MS to know whether the MOB_HO-IND message is lost. The problematic scenarios are described in Figure 1 to Figure 7 in [1].

# Suggested Remedy

The suggested remedy illustrated in Figure 2 is to define a method to detect the lost MOB_HO-IND (cancel) message.

An MS shall start T42 after sending MOB_HO-IND (cancel) message and wait for RNG-RSP message with Success status without any other TLV encoding, e.g. PHY adjustment, except CMAC as the acknowledgement of reception of MOB_HO-IND message from serving BS. The BS receives MOB_HO-IND (cancel) message with valid CMAC should send RNG-RSP message with Success status without any other TLV encoding except CMAC to the MS. If the MS does not receive the RNG-RSP message before the expiration of T42, it should consider the MOB_HO-IND (cancel) message is lost.

Figure 1 – Detection of lost of MOB_HO-IND (cancel) message

# Suggested Changes in Rev2/D4

-------------------------------------------------------------Start of the Text------------------------------------------------------------

[InRev2/D4, line 58 on page 442, section 6.3.22.2.6, insert the following text]

The behaviors of MS and BS will depend on the status of Resource Retain Timer.

Case 1. If the Resource Retain Timer is not expired at MS,

Case 1-1. If the target BS is the same as the previous serving BS,

MS behaviors: If MS resumes communication with serving BS by sending MOB_HO-IND (HO_IND_type = 0b01, HO cancel) message, it shall start the timer T42 immediately after sending MOB_HO-IND message and wait for RNG-RSP message over Basic CID of the MS with success status and a valid CMAC.

BS behaviors: When the serving BS receives MOB_HO-IND (HO_IND_type = 0b01, HO cancel) message, it shall send a RNG-RSP message to the MS with success status and valid CMAC. The RNG-RSP message shall not include any TLV encoding related to PHY adjustment.

Thereafter, if the MS does not receive a RNG-RSP message before the expiration of T42, the MS may perform network reentry to serving BS by sending HO ranging codes.

Case 1-2. If the target BS is not the same as the previous serving BS,

MS behaviors: The MS may attempt to network reentry with the target BS by sending HO ranging codes.

Case 2. If the Resource Retain Timer is expired at MS,

MS behaviors: The MS shall perform initial network entry the target BS by sending initial ranging codes.

[InRev2/D4, on page 1031, section 10.1, modify the following entry in Table 524]

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MS | T42 | MOB_HO-IND timeout when sent with HO_IND_type = 0b10 or 0b01. | - | 150 ms for HO_IND_type = 0b01. | - |

--------------------------------------------------------------End of the Text------------------------------------------------------------

# References

1. “Fixes for Lost of MOB_HO-IND (Cancel) Messages”, C80216maint-08_106r5