| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fix for RNG-REQ retransmission in network entry process |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | Li Wang, ZTE Corporation | Voice: E-mail: wangli@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16Rev2/D4, Letter Ballot 26b. Technical Comments |  |
| Abstract | This contribution proposed clarification for RNG-REQ retransmission during network entry. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Fix for RNG-REQ retransmission in network entry process

Li Wang

ZTE Corporation

# Problem description

In the section 6.3.10.3.1 of the current standard IEEE P802.16Rev2 D4, for figure 102, when BS waits for a SBC-REQ message, BS shall handle the RNG-REQ retransmission because RNG-RSP message with matching MAC address may be lost.

In figure 102, when BS sends RNG-RSP with status=1(continue) and basic + primary CIDs, BS proceeds to next step to wait for CDMA initial ranging code. This is error, and the correction of the next step should be waiting for SBC-REQ. It’s just similar to figure 101.

Proposed changes

Replace Figure 102 in 6.3.10.3.1 with the follow figure:

Figure102—Handle RNG-REQ retransmission after MS T3 timeout (OFDMA PHY only)

# References

IEEE Std 802.16Rev2-D3 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” January, 2008