| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for Intercell Interference Mitigations |  |
| Date Submitted | 2008-03-18 |  |
| Source(s) | Srikanth Gummadi, Lei Wang, Wee Peng Goh, Erik Colban Yair Bourlas, NextWave Wireless Jeff Zhuang Motorola Itay Lusky Altair-Semiconductor | Voice: E-mail: lwang@nextwave.com; ybourlas@nextwave.com; wgoh@nextwave.com; ecolban@nextwave.com Jeff.Zhuang@motorola.com Itay@altair-semi.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Working Group Letter Ballot #26b |  |
| Abstract | This contribution proposes a few minor and backward compatible changes into the 802.16 Rev2/D3 that will provide the required supports to aid interference mitigation schemes for the OFDMA PHY based systems. |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support for Intercell Interference Mitigations

Srikanth Gummadi, Lei Wang, Wee Peng Goh, Erik Colban, Yair Bourla,

NextWave Wireless

Jeff Zhuang, Motorola

Itay Lusky, Altair-Semiconductor

# Introduction

Enabling frequency reuse-1 is very important for a wide adoption of 802.16e OFDMA PHY based systems, as spectrum efficiency is one of the most critical performance measures for competing wireless technologies. Intercell interference has been identified as the major problem for effectively achieving frequency reuse-1, therefore, managing interference becomes one of the most important elements to improve the system performance and hence competitive advantage of 802.16e based systems.

Interference management is achieved by a combination of interference avoidance and interference cancellation techniques. Most interference mitigation techniques are based on the knowledge of the interference properties. However, the current 802.16e does not provide adequate supports for making the interference property knowledge available for effective implementations of interference mitigation schemes. This contribution proposes a few minor and backward compatible changes into the 802.16 Rev2/D3 that will provide the required supports to aid interference mitigation schemes for the OFDMA PHY based systems.

# Proposed Solution

In order to provide the required supports to aid the implementations of the interference mitigation schemes, we propose to introduce a concept, called coordinated DL zone, which is a DL zone coordinated between the servicing BS and all its neighbor BSs, including:

- the same zone boundary;

- the same zone permutation type, e.g., PUSC, STC PUSC, AMC, STC AMC, and FUSC;

- no dedicated pilots;

In such a coordinated DL zone, the same zone boundary and the same zone permutation type allow for the interference to be stable, and also allow the interference properties of the neighbor BSs to be learnt by using pilot subcarriers, as the pilot subcarriers from all neighbor BSs are transmitted at the same time and the same locations.  By gaining the knowledge of the interference properties from all neighbor BSs, the interference mitigation schemes can be effectively implemented.

A frame can have zero, one, or multiple coordinated DL zones. The first PUSC zone can also be a coordinated DL zone.

Permutation types, PUSC, STC-PUSC, AMC, STC-AMC, and FUSC are allowed to be used in a coordinated DL zone.  Each permutation type can have zero or one coordinated zone in a frame, except for the PUSC. For the PUSC, in addition to the first DL PUSC zone, there can be another coordinated DL PUSC zone.

To enable the interference mitigation schemes based on the coordinated DL zone concept, the supports are needed at air interface to signal the presences of such coordinated DL zones and some PHY properties. The current 802.16 Rev2/D3 spec does not provide adequate supports for such coordinated DL zone based signaling, thus the following few minor changes are needed in the 802.16 Rev2/D3:

- add a TLV in SBC-REQ and SBC-RSP messages to signal the BS’s supports for the coordinated DL zones and MSS’s interference cancellation capability in the coordinated DL Zones;

- add a TLV in DCD message to signal that the first DL PUSC zone is a DL Coordinated zone;

- use 1 currently reserved bit in the DL zone switch IE to signal that the zone is a DL Coordinated zone;

- add a TLV in the MOB_NBR-ADV message for each neighbor BS to specify its DL_PermBase and PRBS_ID.

In summary, this contribution proposes few minor and backward compatible changes to the 802.16 Rev2/D3, to provide the required supports for the implementation of interference mitigation schemes.

# Suggested Changes in Rev2/D3

Suggested change #1:

In Rev2/D3, page 1126, line58, insert the following subsection, where the new text is marked by blue and underlined.

11.8.16 DL Coordinated Zone capability

The “DL coordinated zone capability” field specifies that MS can exploit the knowledge of interference if the zone is coordinated between BSs (i.e., the MS in the serving sector will experience interference from coordinated BS transmission that start from the same symbol, with the same zone type, and with the same pilot positions). This field is used only for the WirelessMANOFDMA PHY. A bit value of 0 indicates “not supported” while 1 indicates “supported”.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| DL Coordinated Zone Capability | 185 | 1 | Bit#0: Support DL coordinated zone for non-STC PUSC Bit#1: Support DL coordinated zone for STC PUSC if all bursts uses Matrix A Bit#2: Support DL coordinated zone for STC PUSCif all bursts uses Matrix B Bit#3: Support DL coordinated zone for AMC Bit#4: Support DL coordinated zone for STC AMC Bit#5: Support DL coordinated zone for FUSC Bit#6 to Bit #7 : reserved | SBC-REQ SBC-RSP |

Suggested change #2:

In Rev2/D3, page 129, line54, insert the following line, where the new text is marked by blue and underlined.

DL Coordinated Zone capability (see 11.8.16)

Suggested change #3:

In Rev2/D3, page 131, line 8, replace line 8 to 10 by the following text, where the new text is marked by blue and underlined.

The following parameters may be included:

MIH Capability Supported (see 11.8.10)

Extended capability (see 11.8.15)

DL Coordinated Zone capability (see 11.8.16)

Suggested change #4:

In Rev2/D3, page 681, table 311 (“OFDMA DL Frame Prefix Format for all FFT sizes except 128”), make the following highlighted changed:

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| DL_Frame_Prefix_format() { | - | - |
| Used subchannel bitmap | 6 | Bit #0: Subchannel group 0 Bit #1: Subchannel group 1 Bit #2: Subchannel group 2 Bit #3: Subchannel group 3 Bit #4: Subchannel group 4 Bit #5: Subchannel group 5 |
| Reserved Synchronized_Zone | 1 | Shall be set to zero If 1: The duration of the first PUSC zone of this sector is the same or shorter than the first PUSC zones of all 1’st tier BSs (as seen by MS in desired cell). Also: All 1'st tier BSs partition the subchannels among their segments in the same way as indicated in the above bitmap |
| Repetition_Coding_Indication | 2 | 0b00: No repetition coding on DL-MAP 0b01: Repetition coding of 2 used on DL-MAP 0b10: Repetition coding of 4 used on DL-MAP 0b11: Repetition coding of 6 used on DL-MAP |
| Coding_Indication | 3 | 0b000: CC encoding used on DL-MAP 0b001: BTC encoding used on DL-MAP 0b010: CTC encoding used on DL-MAP 0b011: ZT CC encoding used on DL-MAP 0b100: CC encoding with optional interleaver 0b101: LDPC encoding used on DL-MAP 0b110 to 0b111: Reserved |
| DL-Map_Length | 8 | - |
| Reserved | 4 | Shall be set to zero |
| } |  |  |

Suggested change #5:

In Rev2/D3, page 704, line 49, Table 325 (STC zone IE), make the following changes, where the new text is marked by blue and underlined.

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Reserved | 4 3 | Shall be set to zero |
| Coordinated_zone_indication | 1 | If set to 1, indicates this zone is a coordinated zone, in which the servicing BS coordinates with its neighbor BSs to have the same starting zone boundary, the same permutation type and the same pilot position. If switching to STC zone setting the bit to 1 also means that neighboring BSs use same STC pattern (i.e. Matrix A or Matrix B) as SBS |

Suggested change #6:

In Rev2/D3, page 1189, line 20 (in MOB_NBR-ADV of Sec. 11.18.1), append the following rows in Table 576, where the new text is marked by blue and underlined.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Coordinated PUSC zone parameters | 5 | 1 | Specify the DL_PermBase and PRBS_ID for the coordinated DL PUSC zone that is not the first DL zone: Bit#0 to Bit #4: DL_PermBase Bit#5 to Bit #6: PRBS_ID Bit #7 : reserved |
| Coordinated STC PUSC zone parameters | 6 | 1 | Specify the DL_PermBase and PRBS_ID for the coordinated DL STC PUSC zone: Bit#0 to Bit #4: DL_PermBase Bit#5 to Bit #6: PRBS_ID Bit #7 : reserved |
| Coordinated AMC zone parameters | 7 | 1 | Specify the DL_PermBase and PRBS_ID for the coordinated DL AMC zone: Bit#0 to Bit #4: DL_PermBase Bit#5 to Bit #6: PRBS_ID Bit #7 : reserved |
| Coordinated STC AMC zone parameters | 8 | 1 | Specify the DL_PermBase and PRBS_ID for the coordinated DL STC AMC zone: Bit#0 to Bit #4: DL_PermBase Bit#5 to Bit #6: PRBS_ID Bit #7 : reserved |
| Coordinated FUSC zone parameters | 9 | 1 | Specify the DL_PermBase and PRBS_ID for the coordinated DL FUSC zone: Bit#0 to Bit #4: DL_PermBase Bit#5 to Bit #6: PRBS_ID Bit #7 : reserved |

Suggested change #7:

In Rev2/D3, page 196, line 63, insert the following text, where the new text is marked by blue and underlined.

Coordinated PUSC zone parameters (see 11.18.1).

Coordinated STC PUSC zone parameters (see 11.18.1).

Coordinated AMC zone parameters (see 11.18.1).

Coordinated STC AMC zone parameters (see 11.18.1).

Coordinated FUSC zone parameters (see 11.18.1).