| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | # An Enhanced cell Scanning method using FCH /DL MAP for IEEE 802.16e |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Kiran Thakare, Sten Sjoberg Ericsson AB SE-164 80 Stockholm, Sweden | Voice: +46 8 58532591 E-mail: kiran.thakare@ericsson.com |
| Re: | IEEE 802.16 Rev2 |  |
| Abstract | Within the framework of IEEE standard 802.16 Rev2we propose an enhanced scanning method. |  |
| Purpose | In this contribution, we propose two solutions to enhance the scanning method. This solution requires modification of the structure of the FCH or DL MAP. We propose that both these solutions be discussed and one solution be adopted by the working group. These solutions provide faster scanning of base stations, reduce MS battery consumption and reduce time needed for the MS to lock on to the BS. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# An Enhanced cell Scanning method using FCH /DL MAP

Kiran Thakare, Ericsson AB Sweden

# Introduction

In this contribution, we propose two solutions to enhance the scanning method.  This solution requires modification of the structure of the FCH or DL MAP.  We propose that both these solutions be discussed and one solution be adopted by the working group.  These solutions provide faster scanning of base stations, reduce MS battery consumption and reduce time needed for the MS to lock on to the BS.

# Background

When a user terminal (MS) wants to join the WiMax network [1], it follows the network entry procedures as specified in [1].  During the network entry/ cell reselection process, MS scans the possible channels of the DL frequency band of operation in the defined frequency list.  On finding a DL channel, MS then acquires the preamble and synchronizes with BS.  MS then reads FCH.  The FCH contains the DL Frame Prefix (DLFP) which provides frame configuration information, such as the modulation and coding scheme, the length of one or several DL bursts immediately following the FCH and the usable sub-carriers.  This information is used to decode the DL MAP /UL MAP and DCD/UCD if available.  UL then reads and decodes the DL and UL MAP which is further followed by DCD, UCD information.  The DCD message contains system information such as frame number, frame duration, TTG/RTG for TDD operation, DL Center frequency, MAC Version, HO Support Parameters which includes Hysteresis Margin, Time to Trigger, Trigger Type and Paging Group ID etc. UCD contains system information such as ranging parameters, modulation profile etc.  Downlink Channel Descriptor (DCD) and UL Channel Descriptor (UCD) is used to broadcast the DL/UL system information periodically.  This is shown in the figure 1.  Reference [1] specifies the maximum value of the time between two consecutive DCD/UCD messages as 10s.  MS further monitors the DL MAP/UL MAP to acquire and maintain the synchronization at the MAC level after decoding the DCD/UCD.

# Problem Statement

An MS in an active data transfer or a MS doing the initial entry to the network, needs to performs repeated scanning to acquire and maintain the connectivity to the network.

For example MS begins scanning a base station BS1.  First it synchronizes with BS1 by decoding its preamble.  It then decodes FCH and then the DL MAP.  MS then must wait for the DCD/UCD message to show up and this may take maximum 10second if it is lucky it make wait for 50ms by decoding the DCD/UCD count if it included and has changed.  However, for a MS that is just entering the network is not aware of DCD/UCD count, thus waiting for DCD up to max 10s.  If the neighbor list contains large number of BSs then MS must, sequentially scan all these base stations.   Each requiring MS to lock to BS up to 10s or more to fully receive and decode DCD.  Thus acquiring the system information on DCDs of each BSs may take too long time for MS.  For ten BS this leads in worst case to 100second.

For a UE that is in a connection, UE scans the neighboring cells to maintain the connection.  Two possibilities exists.

The BS that supports mobility functionality includes the MOB-NBR-ADV message at a periodic interval for a MS in a dedicated connection to speed up the HO process.  This is to avail the MS with neighboring cell (BS) characteristics in case of cell re-selection or HO purpose.  The neighbor information includes the DCD/UCD of the neighboring cell. The nominal time between the transmission of MOB-NBR-ADV messages could be as high as 30s [2].  But it is to be noted that sending the MOB-NBR-ADV message takes up the bandwidth and is a considerably higher system overhead as the message containing all the DCD/UCD of the neighbor could consume many frames.  For ten neighbor BSs this is as around 4k-bytes for one UE.

Additionally, Serving BS may send the MS with selected list of neighboring BS to scan.  Serving BS may schedule the scanning interval for MS to conduct the scan. MS may additionally request for scanning interval.  It is well known fact that the scanning interval means degradation in quality and end user quality perception.  Lessening the number of BSs to scan (for MS contemplating a HO) may resolve the issue to some extent but does not fully resolve as for example shortening the list BSs to scan may not be feasible in all deployment scenarios (especially in a dense urban deployment).  For a high-speed moving mobile, this list change very frequently increasing the system overhead due to requirement of updating the list almost every couple of frames.  Other solution could be that the BS may increase the periodicity of the DCD messages say every 10th frame.  This has consequences in terms of overhead (precious radio resource consumption).

Thus, all these patches to system improvement hardly improve the system performance and radio resource use.  Thus the degradation in network entry performance and the handover performance of the WiMax system.  Additionally this lead to extra battery consumption at MS.

# Concept

The proposal allows to improve the system performance without having to increase the periodicity of the broadcast information by reducing the time for acquisition of DCD/UCD broadcast information.  Additionally it allows for improving the handover performance, network entry performance by reducing the MS locking time to the BS and DCD/UCD waiting time during BS scanning process.  This results in enhanced BS scanning process.   This additionally leads to reduction in mobile battery usage, thus saving the mobile power.

The proposal allows to reduce the MS locking time to the BS, to acquire DCD/UCD broadcast information.  This is achieved by including DCD-UCD-offset/frame offset fields just after the frame preamble.  The DCD-UCD-offset /DCD-UCD-frame-offset is the offset from the current time/frame number and indicates the duration to the next DCD/UCD message.  Reading this offset MS can derive the arrival of system broadcast information.

Depending upon the DCD-UCD-offset value MS need not lock to the BS indefinitely for DCD-UCD to arrive, thus reducing the locking time.  MS then can go and search for other BS. And come to previous BS to read DCD/UCD nearer the DCD/UCD arrival time.

For example if DCD-frame-offset value is larger (in seconds say10seconds) then MS can go away and scan other BSs and if DCD transmission is just couple of frames away then it locks (wait) to this BS and decodes the DCD.  Thus, the waiting duration is considerably reduced resulting in lesser scanning duration and saving battery consumption.  The scanning algorithm at MS can be a proprietary algorithm and need not be specified by the standards.  Nevertheless, the information needed for this shall be supported by the system

The solution can be achieved two ways:

- The DCD/UCD-offset fields is sent just after the frame preamble.  This information could be sent by modifying the current FCH.

- The information DCD/UCD-offset field could be sent by modifying the current DL MAP.

# Proposed solution 1: FCH Structure

# 5.1 Existing (16e) FCH Structure

Frame preamble is followed by FCH over two symbols.  FCH is QPSK modulated with ½ FEC code rate and four repetitions.  PUSC permutation is applied for the FCH.  FCH is sent on 4 sub-channels. Each sub-channel comprises of 48 data sub-carrier over two symbols.  Thus for all the FFT size 512, 1024, 2024 the total available data sub-carriers for FCH are 4 * 48 = 192 sub-carriers per segment.

The FCH contains the DL Frame Prefix (DLFP) which provides frame configuration information, such as the modulation and coding scheme, the length of one or several DL burst immediately following the FCH and the usable sub-carriers.  DLFP mapped on FCH is shown in table 1 below.

| FCH |  |  |
| --- | --- | --- |
|  | DLFP | 24 bit |

Table 1: DLFP Mapping on FCH

Table 2 shows the details of DLFP, a 24bit data structure transmitted at the beginning of each frame.

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| DLFP() { |  |  |
| Used Sub-channel Bitmap | 6 |  |
| Reserved | 1 |  |
| Repetition Coding Indication | 2 |  |
| Coding Indication | 3 |  |
| DL MAP Length | 8 |  |
| Reserved | 4 |  |
| } |  |  |

Table 2: Details of DLFP Mapped on FCH

# 5.2 Solution based on FCH Structure

This solution only uses the 4 reserved bits of the DLFP field can be used as follows, shown in table 3

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| DLFP() { |  |  |
| Used Sub-channel Bitmap | 6 |  |
| reserved | 0 |  |
| Repetition Coding Indication | 2 |  |
| Coding Indication | 3 |  |
| DL MAP Length | 8 |  |
| DCD-UCD offset | 4 bit |  |

Table 3: showing the modified FCH/DLFP.

In this case the DCD-UCD- offset can be represented as 4 bits, representing 16values.  For new MSs, 0000 bit represent a DCD-UCD value for older MS they can ignore this.  Following table is defined to consider the 4 bits DCD-UCD-offset values.  Another possibility is to use an exponential scale for the consecutive levels for the DCD-UCD-offsets, i.e. level_n = level_0 * k^n.  However other possibilities are open.

| Field value for 4 reserved bits (decimal) | Description |
| --- | --- |
| 0 | Used for Reference WirelessMAN OFDMA systems to indicate current default. |
| 1-11 | Base-2 logarithm of the number of frames before next DCD message; e.g. a value 1 will imply that the message is expected in less than 250 ms, a value 2 will indicate duration between 250 ms and 500 ms and a value 11 will indicate arrival of the DCD/UCD in a duration between 5 and 10 s. All other allowed values n indicate arrival between (2^(n-1) * 5) ms and (2^n * 5) ms. The choice of the multiplicand 5 is prompted by parameters prevalently used (frame time of 5 ms) in the reference WirelessMAN OFDMA system. |
| 12-15 | Reserved for future use |

Table 4: Mapping of the reserved bits in the FCH to an expected duration for reception of the DCD/UCD message

# 6.  Proposed solution 2: DL MAP Structure

The information DCD/UCD-frame-offset field is sent just after the frame preamble.  This information is sent by modifying the current DL MAP.  The reserved bit of physical synchronization field could be used for this and or separate field in the DL MAP message or even a separate MAP IE.  DCD-UCD-frame-offset, a 12 bit value, can be sent in separate DL MAP IE or a separate as a field for the DL MAP.

# 8. Proposal

We propose to discuss the two solutions described in this document.  Additionally we propose that one of the solution proposed here be adapted by the maintenance group group.  We prefer solution number 2, since adding a new field in DL MAP does not rely on using up the few reserved bits in the FCH.  Moreover, solution 2 allows for 12bit frame-offset allowing for finer granularity level to be sent within the IE.  Both the solution provides superior benefits in terms of faster network entry procedure, reduction in MS power consumption and efficient use of radio resources of FCH.

# 8.1 Text Proposal

# 8.1.1 Solution using 1 reserved field of FCH

Replace the table 311 on page 611 with table 4

Add following text on page 682, at line 6

DCD-UCD-offset: Defines the offset in times indicating around what time the system broadcast information will be available.  The granularity is as per the table 4.

Add the table 5 after the above text.

# 8.2 Text Proposal for solution 2 using DL MAP

Add at line 56 of table 38 page 84 following entry

| If DCD-UCD-Frame-offset == 1 { | 1 | 1 bit Indicator value 1or 0 |
| --- | --- | --- |
| DCD-UCD-frame-offset } | 12 bit | Defines the offset from the current frame number to the frame number at which the system broadcast information will be available |

# 9. References

[1] IEEE Std 802.16e-2005, “IEEE Standard for local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands.’’

[2] wimax_forum_mobile_system_profile_v1_40