| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Configuration Timer in H-FDD Operation |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Pallav Sudarshan Jeff Zhuang Mark Cudak Motorola, Inc. | E-mail: Pallav.Sudarshan@motorola.com Jeff.Zhuang@motorola.com Mark.Cudak@motorola.com <http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #26b |  |
| Abstract | Clarifications and signaling mechanisms are provided for efficient operation of H-FDD in 802.16e. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Frame Configuration Timer in H-FDD Operation

# Introduction

In H-FDD operation, the frame configuration information regarding the boundaries of the two DL partitions and two UL partitions can be changed dynamically. An indication by the BS on the expected behavior of boundary change can be useful for sleep mode operation of the MS.

In this contribution, we propose a mechanism that helps the sleep mode operation of MS when the frame configuration is changed dynamically.

# 2. Proposed Text

I: Modify Table 40 pg 86 Section 6.3.2.3.4 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 | — |
| Reserved | 8 | Shall be set to zero |
| FDD Partition Change Flag | 1 | For FDD only. Indicates the next possible partition change 0: Possible partition change in next frame 1: Minimum number of frames (excluding current frame) before next possible change is given by TLV ‘FDD Frame Change Partition Timer’ |
| Reserved | 7 | Shall be set to zero |
| UCD Count | 8 | — |

II: Modify Table 429 pg 831 Section 8.4.5.6.1 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compressed_DL_MAP() { |  | — |
| Compressed map indicator | 3 | Set to binary 110 to indicate a compressed map format |
| UL-MAP appended | 1 | — |
| Reserved | 1 | Shall be set to zero |
| FDD Partition Change Flag | 1 | For FDD only. Indicates the next possible partition change 0: Possible partition change in next frame 1: Minimum number of frames (excluding current frame) before next possible change is given by TLV ‘FDD Frame Change Partition Timer’ |
| Map message length | 11 | — |

III: Add the row shown in Red to Table 543 Section 11.4.1 pg 1067

| Name | Type | Length | Value(variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| Available DL Radio Resources | 23 | 1 | Indicates the average ratio of non-assigned DL radio resources to the total usable DL radio resources. The average ratio shall be calculated over a time interval defined by the DL_radio_resources_window_size parameter (Table 524). The reported average ratio will serve as a relative load indicator. This value can be biased by the operator provided it reflects a consistent representation of the average loading condition of BSs across the operator network. 0x00 : 0% 0x01 : 1% ... 0x64 : 100% 0x65 - 0xFE : reserved, 0xFF indicates no information available | All |
| FDD Frame Partition Change Timer (M) | 26 | 1 | Minimum number of frames before next possible partition change = M+1 frames | OFDMA |