| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification to STC-AMC pilot structure |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Ron Porat Wee Peng Goh Erik Colban Nextwave Wireless* | E-mail: rporat@nextwave.com wgoh@nextwave.com ecolban@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #26b as announced in IEEE 802.16-08/006 |  |
| Abstract | This contribution recommends that the STC-AMC pilot structure to be modified for performance improvement |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification to  STC-AMC pilot structure

Ron Porat, Wee Peng Goh, Erik Colban  – Nextwave Wireless

# Purpose

This document describes a modification to the STC-AMC pilot structure for 2 physical or virtual transmit antennas.

# Reasons:

- The current pilot structure spans 6 symbols which increases the memory size for channel estimation and the performance in medium to high mobility

- The new pilot structure has the following benefits:

  - Simplification of the channel estimation by consolidating pilots in one symbol per slot while maintaining the same pilot density.

  - Improving channel estimation performance by improving simple two 1D MMSE algorithms

- Power variation between symbols is also reduced with the new structure from 1.8dB to 1dB

# Text Changes

Modified 8.4.8.3.1.1 Allocation of pilot subcarriers as follows:

For 2-antenna BS, all pilots in the even symbols shall be allocated for antenna 0; whereas, all pilots in the odd symbols shall be allocated for antenna 1. The positions of pilots in the odd symbols are further switched with those of data subcarriers whose locations coincide with pilots in the previous symbol. Pilots for antenna 0 and antenna 1 are assigned to OFDM symbol of index m=3s+1 where s is the running 2x3 slot number in the STC AMC zone. The pilot positions for antenna 0 and antenna 1 is alternated in adjacent slots. This is shown in Figure 272.

Pilot Location for Antenna #0 = 6k’+1 (m=3s+1) 9k+3[m mod 3]+1 (m = even)

Pilot Location for Antenna #1 = 6k’+4 (m=3s+1) 9k+3[(m – 1) mod 3]+1 (m = odd)

for m = [symbol index], symbol index 0 is the first symbol (except midamble) in which the STC

Zone is applied, k’ =max(k) * 3/2  where k is defined in 8.4.6.1.2.3.

In other words, symbol index shall be reset to 0 when a new STC Zone is applied.

Change figure 272 in section 8.4.8.3.1.1 to

Pilot for Ant 0

Pilot for Ant 1

Change page 923, line 52 as follows:

For 4-antenna BS, pilot pattern for the first 2 antenna shall first be changed as the same as in the 2-antenna BS case, and applied to the same symbols as in the 2-antenna BS case. The pilot pattern for the 3rd and 4th antenna shall be applied to symbols m=3s where s is the running 2x3 slot number in the STC AMC zone.  and then the neighboring two subcarriers shall be further punctured for antenna 2 and 3 as is shown in Figure 274. Pilot Location for Antenna #0 = 9k+3[m mod 3]+1 (m = even) Pilot Location for Antenna #1 = 9k+3[(m–1) mod 3]+1 (m = odd) Pilot Location for Antenna #2 = 9k+3[m mod 3]+2 (m = even) Pilot Location for Antenna #3 = 9k+3[(m–1) mod 3]+2 (m = odd) for m = [symbol index], symbol index 0 is the first symbol (except midamble) in which the STC Zone is applied, k is defined in 8.4.6.1.2.3. In other words, sSymbol index shall be reset to 0 when a new STC Zone is applied.

Change figure 274 in section 8.4.8.3.1.1 to

Pilot for Ant 0

Pilot for Ant 1

Pilot for Ant 2

Pilot for Ant 3