| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Slot structures for AMC permutation in STC zones |  |
| Date Submitted | 3/10/2008 |  |
| Source(s) | Gokhan Korkmaz, Arvind Raghavan, Dave Pechner ArrayComm LLC | Voice: +1-408-9521856 E-mail: gkorkmaz@arraycomm.com arvind@arraycomm.com dpechner@arraycomm.com |
| Re: | LB26 – IEEE802.16 REV2/D3. |  |
| Abstract | This contribution proposes modifications to the slot format for AMC permutation in STC zone. |  |
| Purpose | Adopt the proposed text changes into IEEE802.16 REV2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Slot structures for AMC permutation in STC zones

Gokhan Korkmaz, Arvind Raghavan, Dave Pechner

ArrayComm LLC

Purpose

In 802.16, 2x6 slot structure is defined for all AMC allocations in DL STC zone. However, for matrix B, a slot structure with shorter duration is sufficient and desirable. In this document, we propose to use 2 x 3 (2 bins for 6 symbols) slot structure for matrix B.

Since we are proposing to use only one type of slot structure for the matrix type, there is no need to do further modifications for signaling the slot structure. Matrix type is enough to specify the format.

# Text Changes

8.4.8.3.1.2.1 STC Mapping for optional AMC permutation

[Change the first paragraph of the subclause as indicated]:

For the optional AMC permutation in STC zone, the data slots shall take 2x6 (2 bins for 6 symbols) or 2x3 according to the matrix type. The subcarrier permutation represented by Equation (88) in 8.4.6.3 shall not be applied for the optional AMC permutation within STC zones.

[Change the third paragraph of the subclause as indicated]:

For 2 antenna matrix B in the optional AMC permutation, data slots shall take 2 x 3 format. For 2-antenna vertically encoded matrix B in the optional AMC permutation, the modulated data symbols shall be sequentially mapped for two Tx antennas along the subcarriers of the first symbol. The mapping continues in an ascending manner in subchannels first and then proceeds to the next symbol in time. An illustration of the mapping rule for the antenna #0 is shown in Figure 276, assuming 2 Tx with vertically encoded matrix B for a block of 2 slots. Figure 276 also shows the mapping rule for 2-antenna horizontally encoded matrix B in the optional AMC permutation, where each encoded stream is separately mapped to the corresponding antenna.