| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Error recovery during H-FDD operation in OFDMA physical layer |  |
| Date Submitted | 2008-03-17 |  |
| Source(s) | Andrea Bacioccola, Zexian Li, Roberto Albanese Nokia Aik Chindapol NSN | andrea.bacioccola@nokia.com zexian.li@nokia.com roberto.albanese@nokia.com aik.chindapol@nsn.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #26b |  |
| Abstract | The contribution proposes an error recovery mechanism for H-FDD operation when the boundary is changed and MS loses the updated boundary information. |  |
| Purpose | Adopt and incorporation in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Error recovery during H-FDD operation in OFDMA physical layer

Andrea Bacioccola, Zexian Li, Roberto Albanese

Nokia

Aik Chindapol

NSN

# Introduction

In H-FDD operation, a group of user terminals transmit in the temporal region where another group of user terminals receive. This divides the user terminals into two groups, and divides any particular frame in two temporal regions or partitions. However, the boundary between the two temporal regions can be changed dynamically by the BS in order to utilize system resource more efficiently. When the boundary is changed, there might be problems for H-FDD MSs which were not able to detect the boundary change. This contribution is aimed to provide a text change to the standard in order to efficiently support boundary error recovery in H-FDD operations without service interruption.

# Proposed text

[add the following paragraph at the end of section 6.3.21.1]

In OFDMA H-FDD operation, due to a possible boundary change during unavailability interval, an H-FDD MS shall listen to MAP1 in the first frame of each availability interval to get the updated boundary information.

[add the following paragraph at the end of section 6.3.24]

In OFDMA H-FDD operation, due to a possible boundary change during idle mode unavailability interval, an H-FDD MS shall listen to MAP1 in the first frame of each listening interval to get the updated boundary information.

[add the following paragraph at the end of section 8.4.4.1]

When an H-FDD MS detects that the boundary information is not anymore valid, the MS shall listen to MAP1 in the subsequent frame to get the updated boundary information.