| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixes for lost of RNG-REQ message during HO with fast ranging |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Li Wang, ZTE Corporation | Voice: E-mail: wangli@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16Rev2/D3, Letter Ballot 26a. Technical Comments |  |
| Abstract | This contribution proposed a solution for lost of RNG-REQ message during HO with fast ranging. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fixes for lost of RNG-REQ message during HO with fast ranging

Li Wang

ZTE Corporation

# Problem description

In the current 802.16 standard(P802.16 Rev2 D3) , HO process, section 6.3.22.2.4:

For the purpose of expediting NW re-entry of the MS with the target BS, the serving BS may negotiate with target BS allocation of a non-contention-based ranging opportunity for the MS, i.e., an unsolicited UL allocation for transmission of RNG-REQ message.

MS should send RNG-REQ to target BS and start timer T3 when MS receives a Fast_Ranging_IE.

If RNG-REQ message is lost, MS should perform CDMA ranging with target BS or network reentry with preferred target BS when T3 is timeout. It's not efficient that MS waits T3 timeout and perform the next step.

Proposed changes

We propose that target BS should set a timer T59 for waiting RNG-REQ from MS, target BS should allocate a new Fast_Ranging_IE and send to MS again when T59 is timeout.  Target BS should start T59 when it sends the Fast_Ranging_IE allocation to MS. Time reference of T59 is 10ms as default value and 20ms as maximum value. And Fast_Ranging_IE retransmission retries is 3.

Add the follow definition to table 524 in section 10 of P802.16 Rev2 D3.

| System | Name | Time reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| BS | T59 | RNG-REQ reception timeout following the transmission of a fast ranging IE during HO |  | 10ms | 20ms |
| BS | Fast_Ranging_IE retries | Number of retries on allocation of fast ranging IE at target BS during HO | 3 | 3 | 16 |

Following is the Flow chart of target BS process when wait for RNG-REQ during HO:

# References

IEEE Std 802.16Rev2-D3 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” January, 2008