| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO Support for MBS |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Sang Youb Kim, Mo-Han Fong Nortel Networks 2221 Lakeside Blvd. Richardson, TX 75082 | E-mail: sangyoub@nortel.com |
| Re: | IEEE 802.16Rev2/D4, Letter Ballot 26c. Technical Comments |  |
| Abstract | Current Standards specifies MIMO to unicast and supporting MIMO for MBS is missed. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MIMO support for MBS

Sang Youb Kim, Mo-Han Fong

Nortel Networks

# 1.	 Introduction

In 802.16e-2005, MIMO application is specified for unicast and MIMO support for multicast is missed. However, if a BTS and MS implement MIMO, there is no reason that it should not transmit in MIMO format for MBS.

## Simulation

In the single frequency network (SFN), the spatial multiplexing can further improve the cell-edge coverage. An achievable MBS throughput is analyzed through simulation.

2.1	 Simulation Condition

| Parameters | Value | Comments |
| --- | --- | --- |
| Number of Cells (3 sectors) | 19 |  |
| Bandwidth | 10 MHz |  |
| Operating Frequency | 2500 MHz |  |
| Minimum Mobile-to-BS Distance | 35 m |  |
| Test Sector | Centre Cell any sector |  |
| Sector Orientation | Bore-Sight Pointing |  |
| Antenna Pattern | 70° (-3dB) with 20 dB front-to-Back, ( =70°, and =20dB) |  |
| BS Height | 32 m |  |
| MS Height | 1.5 m |  |
| Propagation Model | ITU | 128+ 37.6*log10(d) (frequency adjustment factor included for 2.5GHz) |
| Log-Normal Shadowing Standard Deviation | 8 dB |  |
| BS Shadowing Correlation | 0.5 |  |
| MS Noise Figure | 7 dB |  |
| Thermal Noise Density | -174 dBm/Hz |  |
| BS Antenna Gain | 15 dB |  |
| CPE Antenna Gain | -1 dBi |  |
| BS Maximum PA Power | 46 dBm |  |
| Other Loss | 20 dB for 500m ISD 10 dB for 1500 ISD | Including cable loss and penetration losses |
| Frequency Reuse Factor | 1:1 |  |
| Antenna Configuration | BS: 2tx, 2rx MS: 1tx, 2rx |  |
| FFT Size | 1024 |  |
| Cyclic Prefix Duration | 11.4 μs |  |
| MS Rx Combining Scheme | MMSE, and MMSE-SIC |  |
| Channel Estimation | Realistic |  |
| FEC Scheme | Turbo Code |  |
| Modulation and Coding | QPSK ¼, QPSK ½, QPSK ¾, 16QAM ½, 16QAM¾ , 64QAM Rate ½, 64QAM Rate 4/5 | No Adaptation |
| Effective SINR method | EESM | See Equation-2 |
| Maximum SINR | 30 dB |  |
| MS Rx Combining Scheme | MMSE, and MMSE-SIC |  |
| Minimum Simulation Time Per Drop | Each Location (MS location and shadowfading) was simulated many times with different Fast fading realizations, until the variance of the average goodput was less than 1%. |  |
| Overhead considered | 28.6% | Pilots 1/8 CP 1/8 Spectral Usage ≈94% |
| Fast Fading Model | For ISD of 1500m: 60 % PedB 3km/h, 30% VehA 30km/h, 10% Veh A 120km/h For ISD of 500m: Ped B 3km/h |  |

2.2	 Goodput vs. Coverage for Inter Site Distance (ISD) of 500 meter

2.3	 Goodput vs. Coverage for Inter Site Distance (ISD) of 1500 meter

## With 1x2 configuration and including estimated MAP overhead of 4 symbols, the achievable MBS throughput is 1.7 bps/Hz for 1.5km ISD and 3 bps/Hz for 500m ISD.

## With 2x2 MMSE configuration and including estimated MAP overhead of 4 symbols, the achievable MBS throughput is 1.7 bps/Hz for 1.5km ISD and 3.6 bps/Hz for 500m ISD.

## With 2x2 and advanced receiver, and including estimated MAP overhead of 4 symbols, the achievable MBS throughput is 2 bps/Hz for 1.5km ISD and 4 bps/Hz for 500m ISD.

## 3.	 Proposal text change

Modifiy Table 324—Extended-2 DIUC code assignment for DIUC = 14 on page 717

| Extended-2 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | MBS MAP IE |
| 01 | HO Anchor Active DL MAP IE |
| 02 | HO Active Anchor DL MAP IE |
| 03 | HO CID Translation MAP IE |
| 04 | MIMO in Another BS IE |
| 05 | Macro-MIMO DL Basic IE |
| 06 | Skip IE |
| 07 | HARQ DL MAP IE |
| 08 | HARQ ACK IE |
| 09 | Enhanced DL MAP IE |
| 0A | Closed-loop MIMO DL Enhanced IE |
| 0B | MIMO DL Basic IE |
| 0C | MIMO |
| 0D | Persistent HARQ DL MAP IE |
| 0E | AAS SDMA DL IE |
| 0F | Reserved MIMO MBS MAP IE |

Insert the following section

8.4.5.3.30	MIMO MBS MAP IE

In the DL-MAP, a BS may transmit DIUC = 14 with the MIMO MBS_MAP_IE() to indicate when the next data for a multicast and broadcast service flow will be transmitted using STC transmission in the format specified by STC and Matrix Indicator fields. The usage of this MAP IE shall follow the description of 8.4.5.3.12

Table XXX – MIMO MBS MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO_MBS_MAP_IE() { | - | - |
| Extended-2 DIUC | 4 | MIMO MBS MAP IE = 0x0F |
| Length | 8 | - |
| MBS Zone identifier | 7 | MBS Zone identifier corresponds to the identifier provided by the BS at connection initiation |
| Macro diversity enhanced | 1 | 0: Non Macro-Diversity enhanced zone; 1: Macro-Diversity enhanced zone |
| If(Macro diversity enhanced = 1){ | - | - |
| Permutation | 2 | 0b00: PUSC permutation 0b01: FUSC permutation 0b10: Optional FUSC permutation 0b11: Adjacent subcarrier permutation |
| DL_PermBase | 5 | - |
| PRBS_ID | 2 | - |
| OFDMA Symbol Offset | 7 | The offset of the OFDMA symbol measured in OFDMA symbols from beginning of the DL frame in which the DL-MAP is transmitted. Counting from the frame preamble and starting from 0 |
| STC | 2 | 0b00: No STC 0b01: STC using 2/3 antennas 0b10: STC using 4 antennas 0b11: FHDC using 2 antennas |
| Matrix Indicator | 2 | STC matrix (see 8.4.8.1.4) if (STC == 0b01 or STC == 0b10) { 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Reserved } Else if (STC == 0b11) { 0b00 = Matrix A 0b01 = Matrix B 0b10 – 11 = Reserved } |
| MIMO MBS MAP message allocation included indication | 1 | Used to indicate if the MIMO MBS MAP message allocation parameters are included |
| Reserved | 3 | Shall be set to zero |
| if (MIMO MBS MAP messsage allocation included = 1) { | - | - |
| Reserved | 3 | - |
| Boosting | 3 | Refer to Table 318 |
| DIUC | 4 | - |
| No. Subchannels | 6 | Indication of burst size of MIMO MBS MAP message with the number of subchannels |
| NO. OFDMA symbols | 6 | Indication of burst size of MIMO MBS MAP message with the number of OFDMA symbols |
| Repetition Coding Indication | 2 | 0b00—No repetition coding 0b01—Repetition coding of 2 used 0b10—Repetition coding of 4 used 0b11—Repetition coding of 6 used |
| } | - | - |
| } else { | - | - |
| DIUC | 4 | - |
| CID | 16 | CID for Single BS MBS service |
| OFDMA Symbol Offset | 8 | The offset of the first OFDMA symbol of the MBS region measured in OFDMA symbols from beginning of this DL frame. |
| Subchannel offset | 6 | The lowest index OFDMA subchannel used for carrying the burst, starting from subchannel 0. |
| Boosting | 3 | Refer to Table 318 |
| SLC_3_indication | 1 | Used to notify sleep mode class 3 is used for single BS MBS service |
| NO. OFDMA Symbols | 6 | - |
| NO. Subchannels | 6 | - |
| Repetition Coding Indication | 2 | 0b00—No repetition coding 0b01—Repetition coding of 2 used 0b10—Repetition coding of 4 used 0b11—Repetition coding of 6 used |
| if (SLC 3_indication = 1) { | - | - |
| Next MIMO_MBS_MAP_IE Frame Offset | 8 | The Next MIMO MBS_MAP_IE Frame Offset value is lower 8 bits of the frame number in which the BS shall transmit the next MIMO MBS MAP IE frame. |
| } | - | - |
| } | - | - |
| if !(byte boundary) { | - | - |
| Padding Nibble | variable | Padding to reach byte boundary |
| } | - | - |
| } | - | - |

6.3.2.3.XX	MIMO_MBS_MAP (multicast and broadcast service map) message

The MIMO_MBS_MAP message defines the MBS transmission using STC. The BS shall send an MIMO_MBS_MAP message on the Broadcast CID to specify the location and size of multi-BS MBS data bursts which are located in DL permutation zones for MBS in frames that are from 2 to 5 frames in the future from the frame containing the MIMO_MBS MAP message. If present, an MIMO_MBS_MAP message shall be located in the first data region in the DL permutation zone for MBS. In terms of multi-BS MBS, a DL permutation zone for MBS is considered an MBS portion. The MIMO_MBS_MAP message format is presented in Table YYY. This message includes the MIMO_MBS_DATA_IE, Extended_MIMO_MBS_DATA_IE and MIMO_MBS_DATA_Time_Diversity_IE which define the access information for the MBS burst. See Table YYY.

Table YYY – MIMO MBS MAP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO_MBS_MAP Message format (){ | - | - |
| Management Message Type = 70 | 8 |  |
| MBS_DIUC_Change_Count | 8 | - |
| #MIMO_MBS_DATA_IE | 4 | The number of included MIMO_MBS DATA IEs |
| for (i = 0; i < n; i++){ | - | n = #MBS DATA IEs |
| MIMO_MBS_DATA_IE | variable | - |
| } | - | - |
| #Extended_MIMO_MBS_DATA_IE | 4 | The number of included Extended MIMO_MBS DATA IEs |
| for(i = 0; i < k; i++) { | - | The number of included Extended MIMO_MBS DATA IEs |
| Extended_MIMO_MBS_DATA_IE() | variable | - |
| } |  |  |
| #MIMO_MBS_DATA_Time_Diversity_IE | 4 | The number of included MIMO_MBS DATA Time Diversity IEs |
| for(i = 0; i < m; i++){ |  | m = #MIMO_MBS DATA Time Diversity IEs |
| MIMO_MBS_DATA_Time_Diversity_IE() | variable | - |
| } |  |  |
| if(!byte boundary){ | - | - |
| Padding Nibble | 4 | - |
| } |  |  |
| TLV encoding element | - | - |
| } | - | - |

Table ZZZ – MIMO MBS MAP types

| MIMO_MBS_MAP type | Description |
| --- | --- |
| 0 | MMO_MBS_DATA_IE |
| 1 | MIMO_MBS_DATA_Time_Diversity_IE |
| 2 | Extended_MIMO_MBS_DATA_IE |
| 3 | Reserved |

Table AAA – MIMO MBS DATA IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO_MBS_DATA_IE(){ | - | - |
| MIMO_MBS_MAP Type = 0 | 2 | MIMO_MBS_DATA_IE |
| MBS Burst Frame Offset | 2 | This indicates the burst located by this IE will be shown after MBS Burst Frame Offset + 2 frames. |
| Next MIMO_MBS MAP change indication | 1 | This indicates whether the size of MIMO MBS MAP message of next MB S frame for these Multicast CIDs included this IE will be different from the size of this MIMO MBS MAP message. |
| No. of Multicast CID | 3 | - |
| for(i = 0; i < No. of Multicast CID; i++){ | - | - |
| Multicast CID | 12 | 12 LSBs of CID for multicast. |
| } | - | - |
| MBS DIUC | 4 | - |
| OFDMA Symbol Offset | 8 | OFDMA symbol offset with respect to start of next (MBS Burst Frame offset + 2)th frame. |
| Subchannel Offset | 6 | OFDMA subchannel offset with respect to start of the next (MBS Burst Frame offset +2)th frame. |
| Boosting | 3 | Refer to Table 318. |
| No. OFDMA Symbols | 7 | The size of MBS data. |
| No. Subchannels | 6 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 - Repetition coding of 2 used 0b10 - Repetition coding of 4 used 0b11 - Repetition coding of 6 used |
| Matrix Indicator | 2 | STC matrix (see 8.4.8.1.4) if (STC == 0b01 or STC == 0b10) { 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Reserved } Else if (STC == 0b11) { 0b00 = Matrix A 0b01 = Matrix B 0b10 – 11 = Reserved } |
| Next MIMO_MBS Frame Offset | 8 | A relative value from the current frame number in which the next MIMO_MBS MAP message will be transmitted. |
| if (Next MIMO MBS MAP change indication == 1){ | - | - |
| Next MIMO MBS No. OFDMA Symbols | 6 | It is to indicate the size of MIMO_MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| Next MIMO MBS No. OFDMA Subchannels | 6 | It is to indicate the size of MIMO_MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| } | - | - |
| } | - | - |

Table BBB – Extended MIMO MBS DATA IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended_MIMO_MBS_DATA_IE(){ | - | - |
| MIMO_MBS_MAP Type = 2 | 2 | MIMO_MBS_DATA_IE |
| MBS Burst Frame Offset | 2 | This indicates the burst located by this IE will be shown after MBS Burst Frame Offset + 2 frames. |
| Next MIMO_MBS MAP change indication | 1 | This indicates whether the size of MIMO MBS MAP message of next MB S frame for these Multicast CIDs included this IE will be different from the size of this MIMO MBS MAP message. |
| No. of Multicast CID | 3 | - |
| for(i = 0; i < No. of Multicast CID; i++){ | - | - |
| Multicast CID | 12 | 12 LSBs of CID for multicast. |
| No. of Logical Channel ID | 4 | - |
| for(j = 0; j < No. of Logical Channel ID; j++) { | - | - |
| Logical Channel ID | 8 | - |
| } | - | - |
| } | - | - |
| MBS DIUC | 4 | - |
| OFDMA Symbol Offset | 8 | OFDMA symbol offset with respect to start of next (MBS Burst Frame offset + 2)th frame. |
| Subchannel Offset | 6 | OFDMA subchannel offset with respect to start of the next (MBS Burst Frame offset +2)th frame. |
| Boosting | 3 | - |
| No. OFDMA Symbols | 7 | The size of MBS data. |
| No. Subchannels | 6 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 - Repetition coding of 2 used 0b10 - Repetition coding of 4 used 0b11 - Repetition coding of 6 used |
| Matrix Indicator | 2 | STC matrix (see 8.4.8.1.4) if (STC == 0b01 or STC == 0b10) { 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Reserved } Else if (STC == 0b11) { 0b00 = Matrix A 0b01 = Matrix B 0b10 – 11 = Reserved } |
| Next MIMO_MBS Frame Offset | 8 | A relative value from the current frame number in which the next MIMO_MBS MAP message will be transmitted. |
| Next MIMO_MBS OFDMA Symbol Offset | 8 | The offset of the OFDMA symbol in which the next MBS portion starts, measured in OFDMA symbols from the beginning of the DL frame in which the MIMO_MBS_MAP is transmitted. |
| if (Next MIMO MBS MAP change indication == 1){ | - | - |
| Next MIMO MBS No. OFDMA Symbols | 6 | It is to indicate the size of MIMO_MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| Next MIMO MBS No. OFDMA Subchannels | 6 | It is to indicate the size of MIMO_MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| } | - | - |
| } | - | - |

Table CCC – MIMO MBS DATA Time Diversity IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO_MBS_DATA_Time_Diversity_IE() { | - | - |
| MIMO_MBS_MAP Type = 1 | 2 | - |
| MBS Burst Frame Offset | 2 | This indicates the burst located by this IE will be shown after MBS Burst Frame offset + 2 frames |
| OFDMA symbol offset | 8 | This indicates starting position of the region of MBS Bursts with respect to start of the next (MBS Burst Frame offset + 2)-th frame. |
| # of Data Sub-bursts | 4 | n = # of Data Sub-bursts with the same frame offset |
| for( i = 0; i < n; i++ ){ | - | - |
| Multicast CID | 12 | 12 LSBs of CID for multicast |
| N_EP code | 4 | - |
| N_SCH code | 4 | - |
| AI_SN | 1 | - |
| SPID | 2 | - |
| ACID | 4 | - |
| Matrix Indicator | 2 | STC matrix (see 8.4.8.1.4) if (STC == 0b01 or STC == 0b10) { 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Reserved } Else if (STC == 0b11) { 0b00 = Matrix A 0b01 = Matrix B 0b10 – 11 = Reserved } |
| Next MIMO MBS MAP change indication | 1 | This indicates whether the size of MIMO_MBS MAP message of next MBS frame for these multicast CIDs included this IE will be different from the size of this MIMO MBS MAP message. |
| Next MIMO_MBS frame offset | 8 | - |
| Next MIMO_MBS OFDMA Symbol offset | 8 | - |
| if (Next MIMO_MBS MAP change indication == 1) { | - | - |
| Next MIMO_MBS No. OFDMA symbols | 6 | It is to indicate the size of MIMO_MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| Next MIMO_MBS No. OFDMA subchannels | 6 | It is to indicate the size of MIMO_MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| } | - | - |
| } | - | - |
| } | - | - |