| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SDU MTU Capability Declaration |  |
| Date Submitted | 2008-3-17 |  |
| Source(s) | David Johnston Intel Corporation 2111 NE 25th Avenue Hillsboro, OR, USA | Voice: 503 629 0238 E-mail:dj.johnston@intel.com david.johnston@ieee.org *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Revision 2 |  |
| Abstract | The MTU of transport SDUs is unbounded in 802.16. All implementations will have some limit. A method to discover the upper bound of the SDU MTU between SS and BS during SBC is proposed. |  |
| Purpose | To prevent inadvertent packet loss due to exceeding supported MTU sizes. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SDU MTU Capability Declaration

David Johnston

Intel Corporation

# Explanation

The {first, middle, middle, … , last} structure of 802.16 fragmentation leads to a hypothetically unbounded MTU size of transport SDUs on fragmentable connections.

This leads to at least two failure scenarios:

1) All implementations will have some implementation defined MTU limit. In the absence of some specified MTU, MTU negotiation or MTU declaration, it is possible for an 802.16 entity (SS or BS) to transmit SDUs with a greater SDU size than is supported by the receiving entity, leading to undefined behavior.

2) 802.1D specifies that when a packet is received for forwarding to a bridge port, where the packet size is greater than the MTU of the bridge port, the packet shall be “silently discarded”. This leads to an unreported packet loss scenario, E.G. when bridging 802.16 to Ethernet.

The proposed text adds an optional TLV to SBC-REQ and SBC-RSP that declares the upper bound for the transport SDU MTU of the sending entity. Where both SS and BS send this TLV, they are both able to identify the smaller of the two values as the safe upper MTU limit. This does not impose any MTU size, but instead indicates a safe upper limit for an MTU. Implementations are free to choose a smaller MTU than is discovered through this mechanism but are afforded the opportunity to avoid an unsupportably large MTU.

WiMAX for example specifies a 1400 byte MTU for IP CS SDUs, to accommodate the IP packet and other headers (GRE, IPSec, etc.) within 1522 byte ethernet packets. However bridged scenarios might want to advertise a smaller MTU when bridging to a medium on which a smaller MTU is implemented.

# Proposed Text Changes

The text below defines a TLV for inclusion in SBC-REQ and SBC-RSP (11.8.x) and text to permit its inclusion (6.3.2.3.23 and 6.3.2.3.24). This TLV declares the upper bound supportable for the transport SDU MTU of the sender (SS in the SBC-REQ case and BS in the SBC-RSP case).

[Add a line to the list in 6.3.2.3.23]

The following parameters may be included:

Capabilities for construction and transmission of MAC PDUs (see 11.8.2)
Security Negotiation Parameters (see 11.8.4)
Service Information Query (see 11.8.9)
Visited NSP ID (see 11.8.11)
Auth Type for EAP (see 11.8.12)
MIH Capability Supported (see 11.8.10)
Extended capability (see 11.8.15)
SDU MTU capability (see 11.8.16)

[Add text after the mandatory TLV list in 6.3.2.3.24]

A BS shall generate SBC-RSP messages in the form shown in Table 91, including both of the following parameters:

CID (in the MAC header) The connection identifier in the MAC header is the Basic CID for this SS, as appears in the RNG-REQ message.

The following parameter may be included in the SBC-RSP.

SDU MTU capability (see 11.8.16)

[Insert new subclause 11.8.16, at the end of page 1126, inserting the sequentially appropriate type value]

11.8.16 SDU MTU capability

The maximum MTU capability TLV specifies the upper bound for the SDU MTU size supportable on transport connections.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Maximum MTU capability | xxx | 2 | An unsigned 16 bit integer. When in SBC-REQ: The maximum SDU MTU, in bytes, supported by the SS on transport connections. When in SBC-RSP: The maximum SDU MTU, in bytes, supported by the SS on transport connections. | SBC-REQ SBC-RSP |