| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Adding PSC Assignment TLV to DSA and DSC protocol messages. |  |
| Date Submitted | 2008-03-15 |  |
| Source(s) | Henri Moelard Floyd Simpson Joe Schumacher Phil Barber | E-mail: moelard@motorola.com E-mail: Floyd.Simpson@motorola.com E-mail: j.schumacher@motorola.com E-mail: pbarber@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #26b |  |
| Abstract | Instructions to add a new TLV to DSA-REQ/RSP and DSC-REQ/RSP protocol messages to allow immediate addition of the associated CID to an existing Power Saving Class. |  |
| Purpose | Adopt for P802.16 Rev2 as replacement for original resolution proposed by comment #2037 in LB26b |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Adding PSC Assignment TLV to DSA and DSC protocol messages

Floyd Simpson, Joe Schumacher, Henri Moelard –Motorola
Phil Barber - Huawei

# Explanation

The current condition that Number_of_CIDs = 0 and CID=0 in the PSC definition applies only to CIDs associated with the MS at the time the MOB_SLP-REQ/RSP message is transmitted is unnecessarily restrictive. This requires that the MS has to stay awake after a new service flow has been activated till the (typically one) PSC (that is meant to include all CIDs) has been re-defined to include the new CID and activated. This has a negative impact on the MS’s power consumption and Sleep Mode-based coexistence schemes.

LB26b comment #2037 proposes to completely remove the restriction so that CIDs of service flows, activated after a PSC has been defined, can be added automatically. However, comments have been raised this may have long-lived effects and may not be a preferred design.

This contribution proposes an alternative mechanism to add a CID to an existing PSC by means of the DSA/DSC protocol, thus allowing a CID to be added to a PSC without interrupting the Sleep Mode windows, but yet provide control over this per CID.

# Proposed Rev2D3 Text Changes

[Insert the following new subclause at the end of section 11.13; fill in the subclause number and the next available type number.]

11.13.xx PSC Assignment

The PSC Assignment value specifies the Power_Saving_Class_ID of an existing PSC to which the CID of the service flow shall be assigned. This TLV may be included only when the DSA or DSC protocol message activates the service flow (i.e. includes an active QoS parameter set).

If the DSA or DSC protocol activates the service flow and the PSC Assignment TLV is present in the BS’s DSA/DSC-REQ/RSP and specifies the Power_Saving_Class_ID of an existing PSC, then the CID of the service flow is added immediately to the PSC at the time the service flow is activated. No explicit PSC re-definition is needed.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].tbd | 1 | Bit #0-5 The Power_Saving_Class_ID of the PSC to which the CID of this service flow is to be added. Bit #6-7 Reserved | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP |

[Modify page 188, lines 18-24 in section 6.3.2.3.40 MOB_SLP-RSP (sleep response) message]

CID

CIDs of all connections comprising the Power Saving Class. If Basic CID is included, it means that all MS management connections are included in the Power Saving Class. If CID = 0 is included, that means all current transport connections are added to the Power Saving Class. If Defintion = 1, then the service flow management encoding PSC Assignment shall be set to the value of the Power_Saving_Class_ID in this message corresponding to the transport connection identified in the CID information element.