| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |  |
| --- | --- | --- | --- |
| Title | FDD/H-FDD Group Switching Signaling (Harmonized) |  |  |
| Date Submitted | 2008-03-18 |  |  |
| Sources: | Pallav Sudarshan, Jeff Zhuang, Gerrit Hiddink, Mark Cudak; | Motorola, Inc | Pallav.Sudarshan@motorola.com, Jeff.Zhuang@motorola.com, ghiddink@motorola.com Mark.Cudak@motorola.com |
|  | Dan Gal | Alcatel-Lucent | dgal@alcatel-Lucent.com |
|  | Sylvain Labonte | Sequans | sylvain@sequans.com |
|  | Anuj Puri | Beceem | apuri@beceem.com |
|  | Peretz Shkalim | Runcom | peretzs@runcom.co.il |
|  | Sean Mcbeath | Huawei | smcbeath@huawei.com, |
|  | Jiyun Seol | Samsung | Jiyun.seol@samsung.com |
|  | Yaghoobi, Hassan | Intel | hassan.yaghoobi@intel.com |
|  | Andrea Bacioccola Aik Chindapol | Nokia, NSN | andrea.bacioccola@nokia.com, aik.cindapol@nsn.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #26b |  |  |
| Abstract | Harmonized OFDMA FDD/H-FDD Group Switching Signaling for 802.16Rev2 |  |  |
| Purpose | Adopt and incorporate in IEEE P802.16Rev2. |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |  |

# H-FDD Group-Switching Signaling

# 1. Introduction

In H-FDD operation, a group of user terminals transmit in the temporal region where another group of user terminals receive. This divides the user terminals into two groups, and divides any particular frame in two temporal regions or partitions. The assignment of user terminals to the groups can be done dynamically or statically, depending on BS implementation and deployment preferences. For load-balancing or to enable advanced grouping of users, the BS may require switching a user terminal from one group to the other. A low-overhead mechanism is needed to enable fast switching of a user terminal from one group to the other. This contribution provides details of the signaling mechanism.

# 2. Proposed Solution

In H-FDD operation, for a HARQ-enabled MS in non-MIMO mode, the BS shall signal the group association to the MS using a new “Group Indicator” field in DL HARQ Chase Subburst IE or DL HARQ IR Subburst IE, using one of the reserved bits. In the case of a group switch, BS may request an acknowledgement by allocating a one-time additional CQI channel to the MS for transmitting a special CQICH code. After the transmission of the special acknowledgement, the MS will switch to the other group. If BS chooses not to request any acknowledgement, no CQI feedback channel shall be assigned. If the BS does not detect the MS acknowledgement CQI, it could re-transmit the group switch message. In case a HARQ-enabled MS does not have any DL allocation, the BS may issue a DL HARQ Chase Subburst IE with the Group Indicator and “null” subchannel allocation (i.e., zero subchannel) for that MS.

The BS may signal the group association by using the H-FDD Group Switch IE. If the MS also gets a CQICH channel assignment in the same frame of group switch command, which means that the BS requests an explicit acknowledgement, the SS shall transmit a special code in the fast feedback region as instructed.

# 3. Proposed Text

{note to the editor: Please insert the following new and changed texts in the appropriate locations in 802.16Rev2 draft}

8.4.4.1.1 Group Switching

In FDD, for H-FDD MSs, the BS shall be able to switch a user (MS) from group-1 to group-2, or vice versa at its discretion. To effectuate a group switch, the BS shall use either

- the H-FDD Group Switch IE (8.4.5.3.29), or

- the DL HARQ Chase Subburst IE, or

- the DL HARQ IR CTC Subburst IE, or

- the DL HARQ IR CC Subburst IE.

When using the H-FDD Group Switch IE method, the BS shall use the Group Indicator field to signal the H-FDD group index that the MS should be associated with. If the Group Indicator field is not equal to the current H-FDD MS’s group index, the mobile station shall switch to the group whose index is indicated by the Group Indicator field.

When an MS is instructed to switch group, the MS shall also deem any existing periodic CQICH allocations as being de-allocated by the BS. The BS may also request the MS to explicitly acknowledge a group switching instruction, in which case the BS shall assign a one-time CQICH channel. When either the DL HARQ Chase Subburst IE or the DL HARQ IR CTC Subburst IE, or the DL HARQ IR CC Subburst IE is used for signaling group switch, the one-time CQICH channel shall be allocated via setting the LSB #0 of the Dedicated DL Control Indicator to 1 and using the Allocation Index field to indicate the location.

When H-FDD Group Switch IE is used for signaling group switch, a one-time CQICH channel is assigned in that IE (see 8.4.5.3.29). The MS shall acknowledge the group switch when the BS includes a one-time CQICH allocation in the IE that contains the group switching instruction.

The MS shall acknowledge - with a MAP ACK command as described in section 8.4.5.4.10.16 - in the assigned CQICH channel

(Note to editor: section number to be confirmed by related Persistent Scheduling comments).

After the BS initiates a group switching, if there is any UL transmission such as the explicit acknowledgement requested by BS and ACK to HARQ traffic, the MS shall switch to the destination group as soon as its UL transmission in the original groups ends. If there is no UL transmission after the BS imitates the group switching, the MS shall switch destination group as soon as the DL transmission in the original groups ends.

8.4.5.3.29: H-FDD Group Switch IE

In FDD, for H-FDD MS, H-FDD Group Switch IE, as shown in Table YYY, may be used by the BS to signal one or more MS to switch H-FDD groups.

Table YYY – H-FDD Group Switch IE Format

| Syntax | Size | Notes |
| --- | --- | --- |
| HFDD_Group_Switch_IE() { | - | - |
| Extended DIUC | 4 | HFDD_Group_Switch_IE() = 0x0D |
| Length | 4 |  |
| RCID_Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| While (data remains){ |  |  |
| RCID | variable |  |
| Group Indicator | 1 | Indicates the group assignment of the MS (see 8.4.4.1 for FDD frame structure and group definition) 0b0: Group #1 0b1: Group #2 |
| CQICH Allocation Included | 1 | 0b0: CQICH Allocation Not Included 0b1: CQICH Allocation Included |
| if (CQICH Allocation Included==1) { |  |  |
| Allocation Index | 6 | Index to the channel in a frame the CQI code should be transmitted by the SS |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to byte; shall be set to 0 |
| } | - | - |

Group Indicator

The MS shall compare the Group Indicator field to its current H-FDD group index and if the values are not identical, the MS shall switch to the group as indicated by the Group Indicator field (see 8.4.4.1.1)

CQICH Allocation Included

If the CQICH Allocation Included field is set to 1, the MS shall respond with an acknowledgement of the group change, using the assigned CQICH channel indexed by the Allocation Index (see 8.4.4.1.1)

{ Make the following changes in Section 8.4.5.3.2.2 Table 321, Pg. 699 as indicated:}

| Extended DIUC (Hexadecimal) | Usage |
| --- | --- |
| 0D-0E | Reserved |
| 0D | H-FDD Group Switch IE |
| 0E | Reserved for possible Extended-3 DIUC |

{Change two rows of Table 346 on page 732 Section 8.4.5.3.21 as indicated in red:}

Table 346 -- DL HARQ Chase Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL_HARQ_Chase_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| N ACK channel | 8 | 4 Number of HARQ ACK enabled subbursts in the 2D region. |
| For (j = 0; j < Number of sub-bursts; j++) { — — | — | — |
| RCID_IE() | variable | — |
| Duration | 10 | Duration in slots |
| Sub-Burst DIUC Indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst If j is 0 then this indicator shall be 1. Reserved 1 Shall be set to zero. |
| Reserved | 1 |  |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If( Sub-Burst DIUC Indicator == 1){ |  |  |

{Add the following paragraph after line 64 Section 8.4.5.3.21 on page 733}

Group Indicator

If the Group Indicator field is not equal to the current H-FDD group index that the MS is associated with, the MS shall switch to the group index indicated by the Group Indicator field. BS can request explicit acknowledgement from MS by setting the LSB #0 of the Dedicated DL Control Indicator to 1 in this IE, in which case MS shall use the assigned CQICH channel indicated in Allocation Index field (see 8.4.4.1.1).

[Change three rows of Table 347 on page 734 Section 8.4.5.3.21  as indicated in red:]

Table 346 -- DL HARQ IR CTC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL_HARQ_IR_CTC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| N ACK channel | 4 | 4 Number of HARQ ACK enabled subbursts in the 2D region. |
| For (j = 0; j < Number of sub-bursts; j++) { — — | — | — |
| RCID_IE() | variable | — |
| NEP | 4 | — |
| NSCH | 4 | — |
| SPID | 2 | — |
| ACID | 4 | — |
| AI_SN | 1 | — |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit |
| Reserved | 2 |  |
| Reserved | 1 |  |
| Group Indicator | 1 | Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| Dedicated DL Control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE |

{Add the following paragraph after line 26 of Section 8.4.5.3.21 on page 735 (after table 347)}

Group Indicator

If the Group Indicator field is not equal to the current H-FDD group index that the MS is associated with, the MS shall switch to the group index indicated by the Group Indicator field. BS can request explicit acknowledgement from MS by setting the LSB #0 of the Dedicated DL Control Indicator to 1 in this IE, in which case MS shall use the assigned CQICH channel indicated in Allocation Index field (see 8.4.4.1.1).

[Change two rows of Table 346 on page 732 Section 8.4.5.3.21 as indicated in red:]

Table 346 -- DL HARQ IR CC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL_HARQ_IR_CC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| N ACK channel | 8 | 4 Number of HARQ ACK enabled subbursts in the 2D region. |
| For (j = 0; j < Number of sub-bursts; j++) { — — | — | — |
| RCID_IE() | variable | — |
| Duration | 10 | Duration in slots |
| Sub-Burst DIUC Indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, the this subburst will use the same DIUC as the previous subburst If j is 0 then this indicator shall be 1. Reserved 1 Shall be set to zero. |
| Reserved | 1 |  |
| Group Indicator | 1 | Used for FDD/H-FDD case only to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If( Sub-Burst DIUC Indicator == 1){ |  |  |

{Add the following paragraph after table 348 of Section 8.4.5.3.21 on page 736}

Group Indicator

If the Group Indicator field is not equal to the current H-FDD group index that the MS is associated with, the MS shall switch to the group index indicated by the Group Indicator field. BS can request explicit acknowledgement from MS by setting the LSB #0 of the Dedicated DL Control Indicator to 1 in this IE, in which case MS shall use the assigned CQICH channel indicated in Allocation Index field (see 8.4.4.1.1).