| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |  |
| --- | --- | --- | --- |
| Title | FDD Frame Structure Signaling (Draft 1 for Harmonization) |  |  |
| Date Submitted | 2008-03-18 |  |  |
| Sources: | Pallav Sudarshan, Jeff Zhuang, Gerrit Hiddink, Mark Cudak; | Motorola, Inc | Pallav.Sudarshan@motorola.com, Jeff.Zhuang@motorola.com, ghiddink@motorola.com Mark.Cudak@motorola.com |
|  | Dan Gal | Alcatel-Lucent | dgal@alcatel-Lucent.com |
|  | Sylvain Labonte | Sequans | sylvain@sequans.com |
|  | Anuj Puri | Beceem | apuri@beceem.com |
|  | Peretz Shkalim | Runcom | peretzs@runcom.co.il |
|  | Sean Mcbeath | Huawei | smcbeath@huawei.com, |
|  | Jiyun Seol | Samsung | Jiyun.seol@samsung.com |
|  | Yaghoobi, Hassan | Intel | hassan.yaghoobi@intel.com |
|  | Andrea Bacioccola Aik Chindapol | Nokia, NSN | andrea.bacioccola@nokia.com, aik.cindapol@nsn.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #26b |  |  |
| Abstract | OFDMA FDD Frame Structure Signaling for 802.16Rev2 |  |  |
| Purpose | Adopt and incorporate in IEEE P802.16Rev2. |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |  |

# FDD Frame Structure Signaling

Part 1: MAP-Based Signaling

# Introduction

The default UL transmission is “time-first”, as opposed to default “frequency-first” allocation in DL. The UL transmission for a user starts after the end of a previous allocation.  The temporal duration of the UL transmission for a user usually spans the entire zone. This is the so-called “snake-like” allocation, which means essentially that in order to enable H-FDD SS with both UL and DL traffic in the same frame, uplink subframe needs to be divided into partitions. Given that a partition can not be defined on a per-SS basis, the more common case is that each partition contains allocation to a group of users.

In the downlink, the start time of subframe2 is referred to as the “FDD frame partition boundary”. The BS indicates the size of the each DL subframe in its DL-MAP, using the parameter “No. OFDMA Symbols”.

# 2. Proposed Text

{note the editor: insert the following text in section 8.4.4.1}

8.4.4.1.2  Frame Partition Signaling

The BS shall indicate the size (in symbols) of DL1 of the current frame using the “No. of OFDMA symbols” field (see 6.3.2.3.2) in DL-MAP1 whereas the same field in DL-MAP2 shall indicate the size of DL2 of the next frame.

For the UL, the BS shall indicate, in the “No. OFDMA symbols” in UL-MAP1 and UL-MAP2, the size of UL1 of the next frame and the size of UL2 of the next-next (n+2) frame respectively.

II. Modify Table 38 pg 84 Section 6.3.2.3.2 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL-MAP_Message_Format() { |  |  |
| Management Message Type = 2 | 8 | — |
| PHY Synchronization Field | variable | See appropriate PHY specification. |
| DCD Count | 8 | — |
| Base Station ID | 48 | — |
| Begin PHY-specific section { | — | See applicable PHY subclause |
| if (WirelessMAN-OFDMA) { | — | — |
| No. OFDMA symbols | 8 | For TDD, Number of OFDMA symbols in the DL subframe including all AAS/permutation zone and including the preamble. For FDD, see Section 8.4.4.1.2 |
| } | — | — |
| for (i = 1; i <= n; i++){ | — | — |

III. Modify Table 40 pg 86 Section 6.3.2.3.4 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 | — |
| Reserved | 8 | Shall be set to zero |
| UCD Count | 8 | — |
| Allocation Start Time | 32 | — |
| Begin PHY-specific section { | — | See applicable PHY subclause |
| if (WirelessMAN-OFDMA) { | — | — |
| No. OFDMA symbols | 8 | For TDD, Number of OFDMA symbols in the UL subframe. For FDD, see Section 8.4.4.1.2 |
| } | — | — |
| for (i = 1; i <= n; i++){ | — | For each UL-MAP element 1 to n. |

IV: Modify Table 429 pg 831 Section 8.4.5.6.1 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compressed_DL_MAP() { |  | — |
| Compressed map indicator | 3 | Set to binary 110 to indicate a compressed map format |
| UL-MAP appended | 1 | — |
| Reserved | 1 | Shall be set to zero |
| Map message length | 11 | — |
| PHY Synchronization Field | 32 | See appropriate PHY specification. |
| DCD Count | 8 | — |
| Operator ID | 8 | — |
| Sector ID | 8 | — |
| No. OFDMA symbols | 8 | For TDD, Number of OFDMA symbols in the DL subframe including all AAS/permutation zone and including the preamble. For FDD, see Section 8.4.4.1.2 |
| DL IE count | 8 | — |

V: Modify Table 430 pg 832 Section 8.4.5.6.2 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compressed_UL-MAP(){ |  |  |
| UCD Count | 8 | — |
| Allocation Start Time | 32 | — |
| No. OFDMA symbols | 8 | For TDD, Number of OFDMA symbols in the UL subframe. For FDD, see Section 8.4.4.1.2 |
| while (map data remains){ | — | — |

VI. Add text following table 430 as indicated:

When a H-FDD MS no longer has valid partition information, the MS shall listen to MAP1 in order to receive updated partition information.

Part 2: MAP-Based Signaling with Timer Restrictor

# Timer-Restricted, Frame Partition Change Mechanisms

# 1. Introduction

The FDD frame partition information, conveyed in the No. OFDAM symbols in MAP1 and MAP2 can be changed, in principle, in every frame. Restricting the changes in a way that would be predictable to the MS can be useful for sleep mode.

The timer restriction, defined in this proposed change, indicates to the MS whether a frame partition change may take place as soon as the next frame, or not earlier than M future frames from the current frame. The indication would be provided in a flag in the UL-MAP. The number M would be indicated in a new TLV.

# 2. Proposed Text

I: Modify Table 40 pg 86 Section 6.3.2.3.4 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 | — |
| Reserved | 8 | Shall be set to zero |
| FDD Partition Change Flag | 1 | For FDD only. Indicates the next possible partition change 0: Possible partition change in next frame 1: Minimum number of frames (excluding current frame) before next possible change is given by TLV ‘FDD Frame Change Partition Timer’ |
| Reserved | 7 | Shall be zero |
| UCD Count | 8 | — |

II: Modify Table 429 pg 831 Section 8.4.5.6.1 as shown

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compressed_DL_MAP() { |  | — |
| Compressed map indicator | 3 | Set to binary 110 to indicate a compressed map format |
| UL-MAP appended | 1 | — |
| Reserved | 1 | Shall be set to zero |
| FDD Partition Change Flag | 1 | For FDD only. Indicates the next possible partition change 0: Possible partition change in next frame 1: Minimum number of frames (excluding current frame) before next possible change is given by TLV ‘FDD Frame Change Partition Timer’ |
| Map message length | 11 | — |

III: Add the row shown in Red to Table 543 Section 11.4.1 pg 1067

| Name | Type | Length | Value(variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| Available DL Radio Resources | 23 | 1 | Indicates the average ratio of non-assigned DL radio resources to the total usable DL radio resources. The average ratio shall be calculated over a time interval defined by the DL_radio_resources_window_size parameter (Table 524). The reported average ratio will serve as a relative load indicator. This value can be biased by the operator provided it reflects a consistent representation of the average loading condition of BSs across the operator network. 0x00 : 0% 0x01 : 1% ... 0x64 : 100% 0x65 - 0xFE : reserved, 0xFF indicates no information available | All |
| FDD Frame Partition Change Timer (M) | 26 | 1 | If the current frame is n, the next possible frame partition change shall not take place before frame number n+M+1. | OFDMA |

Part 3: DCD-Based Signaling

# DCD-Based Frame Partition Signaling

# 1. Introduction

The DCD-based signaling of FDD frame partition may be used to disable the MAP-based signaling and indicate the frame partition and keep it stable for a relatively long time (even permanently) and freeing the MS from constantly monitoring the frame partition conveyed in the MAPs.

# 2.  Proposed Text

I: Add the row shown in Red to Table 543 pg 1067

| Name | Type | Length | Value(variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| Available DL Radio Resources | 23 | 1 | Indicates the average ratio of non-assigned DL radio resources to the total usable DL radio resources. The average ratio shall be calculated over a time interval defined by the DL_radio_resources_window_size parameter (Table 524). The reported average ratio will serve as a relative load indicator. This value can be biased by the operator provided it reflects a consistent representation of the average loading condition of BSs across the operator network. 0x00 : 0% 0x01 : 1% ... 0x64 : 100% 0x65 - 0xFE : reserved, 0xFF indicates no information available | All |
| FDD Partition TLV | 25 | 2 | Bit #0 = 1: DCD-based signaling Bit #0 = 0: MAP-based signaling DCD-based signaling (Bit #0 = 1): Bits 1-7: Number of OFDMA Symbols in DL2 Bits 8-15: Frame number LSBs of the first frame in which the DCD-based signaling - with the above parameter- becomes effective. MAP-based signaling (Bit #0 = 0): Bits 1-15: Reserved Note: The DL and UL MAPs convey the sizes of DL1, DL2, UL1 and UL2 (see section 8.4.4.1.2) | OFDMA |