| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Feedback_Polling_IE |  |
| Date Submitted | 2008-03-15 |  |
| Source(s) | Kathiravetpillai Sivanesan and Peiying Zhu Nortel Networks 2221 Lakeside Blvd. Richardson, TX 75082 | E-mail: kathirsi<at>nortel.com |
| Re: | IEEE 802.16Rev2/D3, Letter Ballot 26b. Technical Comments |  |
| Abstract | Proposed a modification to the feedback Polling IE |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Feedback Polling IE

Kathiravetpillai Sivanesan and Peiying Zhu

Nortel Networks

# 1.	 Introduction

For the CL-MIMO operation the CQI report can be based on the Preamble or Midamble.  So, the Feedback Polling IE needs to be modified to support Midamble based CQI reporting as well.

## Proposal

Change Table 424 in Section 8.4.5.4.28 Feedback Polling IE as following

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Feedback_Polling_IE() { | - | - |
| Extended-2 UIUC | 4 |  |
| Length | 8 |  |
| Num_Allocations | 4 |  |
| Dedicated UL Allocation Included | 1 |  |
| Reserved | 3 |  |
| for (i = 0; i < Number of allocated feedback channels; i++) { | - |  |
| Basic CID | 16 |  |
| Allocation Duration (d) | 3 |  |
| If (d != 0b000) { |  |  |
| Feedback type |  |  |
| if (feedback type == 0110) { |  |  |
| Report Type | 2 | 0b00: Report the estimation of CINR measured from preamble for band AMC. 0b01: Report the estimation of CINR measured from mid-amble for band AMC. 0b10: Report the estimation of post pre-coding CINR measured from mid-amble for band AMC. 0b11: Reserved |
| } |  |  |
| } |  |  |
| Frame Offset | 3 |  |
| Period (p) | 2 3 |  |
| …. |  |  |