| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Full Duplex Support for H-FDD Frame Structure |  |
| Date Submitted | 2008-03-17 |  |
| Source(s) | Kamran Etemad, Intel Sean McBeath, Huawei Technologies | Voice: E-mail: kamran.etemad@intel.com smcbeath@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Ballot 26b |  |
| Abstract | Full duplex support in the proposed H-FDD frame structure is inefficient. This contribution proposes to use the new persistent IEs (typically in its non-persistent mode) to allocate resources in both group 1 and group 2. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Full Duplex Support in the H-FDD Frame Structure

Kamran Etemad, Intel

Sean McBeath, Huawei Technologies

# Introduction

Full duplex is not efficiently supported within the constraints of the proposed H-FDD frame structure.  Full duplex support is an important consideration for cell edge mobile stations which are power limited.  In this contribution, we propose modifications to the proposed persistent IEs so that a single UL-MAP can allocate resources in both H-FDD groups.  The text changes in this document are relative to C802.16maint-08/095r2.

# Proposed Text

8.4.5.4.29 Persistent HARQ UL MAP Allocation IE

- Table YYY - Persistent HARQ UL MAP IE

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| Persistent HARQ_UL-MAP_IE() { |  |  |
| Extended 2- UIUC | 4 | Persistent HARQ_UL-MAP_IE() = 0x0B |
| Length | 8 | Length of the IE |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| Change Indicator | 1 | 0: No Change occurred 1: Change occurred |
| Reserved | 1 |  |
| While (data remains) { | - | - |
| Mode | 3 | Indicates the mode of this IE: 0b000: Persistent Chase HARQ 0b001: Persistent Incremental redundancy HARQ for CTC 0b010: Persistent Incremental redundancy HARQ for convolutional code 0b011: Persistent MIMO Chase HARQ 0b100: Persistent MIMO IR HARQ 0b101: Persistent MIMO IR HARQ for convolutional code 0b110: Persistent MIMO STC HARQ 0b111: Reserved |
| Allocation Start Indication | 1 | 0: No allocation start information 1: Allocation start information follows |
| If (Allocation Start Indication ==1) { | - | - |
| OFDMA Symbol offset | 8 | This value indicates start symbol offset of subsequent subbursts in this Persistent HARQ UL MAP IE with reference to the start of the UL sub-frame. |
| Subchannel offset | 7 | This value indicates start Subchannel offset of subsequent subbursts in this Persistent HARQ UL MAP IE |
| Reserved | 1 | Shall be set to zero |
| } | - | - |
| Group 2 Allocation Start Indication | 1 | 0: No allocation start information for group 2 (H-FDD) 1: Allocation start information follows for group 2 (H-FDD) |
| If (Group 2 Allocation Start Indication ==1) { | - | - |
| Group 2 OFDMA Symbol offset | 8 | This value indicates start symbol offset of subsequent subbursts in this Persistent HARQ UL MAP IE with reference to the start of the UL sub-frame for group 2 (H-FDD). |
| Group 2 Subchannel offset | 7 | This value indicates start Subchannel offset of subsequent subbursts in this Persistent HARQ UL MAP IE for group 2 (H-FDD) |
| Reserved | 1 | Shall be set to zero |
| } | - | - |
| N sub-bursts | 4 | Number of sub-bursts in this Persistent HARQ UL MAP IE is this field value plus 1. |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (i=0;i<Number of sub-burst; i++) { |  |  |
| If (mode == 000) { |  |  |
| Persistent UL HARQ Chase Sub-Burst IE () |  |  |
| } else if (mode == 001) { |  |  |
| Persistent UL HARQ IR CTC Sub-Burst IE () |  |  |
| } else if (mode == 010) { |  |  |
| Persistent UL HARQ IR CC Sub-Burst IE () |  |  |
| } else if (mode == 011) { |  |  |
| Persistent MIMO UL Chase HARQ Sub-Burst IE () |  |  |
| } else if (mode == 100) { |  |  |
| Persistent MIMO UL IR HARQ Sub-Burst IE () |  |  |
| } else if (mode == 101) { |  |  |
| Persistent MIMO UL IR HARQ for CC Sub-Burst IE () |  |  |
| } else if (mode == 110) { |  |  |
| Persistent MIMO UL STC HARQ Sub-Burst IE () |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to byte; shall be set to 0 |
| } |  |  |

- Table YYY - Persistent UL HARQ Chase Subburst IE format

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| Persistent UL HARQ Chase Sub-Burst IE { |  |  |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| Full Duplex | 1 | 1 = F-FDD 0 = H-FDD or TDD |
| If (Allocation Flag == 0) { |  |  |
| RCID_IE() | variable | -- |
| if (Resource Shifting Indicator ==1) { |  |  |
| If (Full Duplex == 0) { | variable |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the start of the UL subframe if an allocation start indication is not included in this IE and with respect to OFDM symbol offset and subchannel offset if an allocation start indication is included in this IE OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } else { |  |  |
| Group 2 Duration | variable | Group 2 Duration for F-FDD Support |
| Group 2 Slot Offset | variable | Group 2 Slot Offset for F-FDD Support |
| } |  |  |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID_IE() | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| If (Full Duplex == 0) { | variable |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the start of the UL subframe if an allocation start indication is not included in this IE and with respect to OFDM symbol offset and subchannel offset if an allocation start indication is included in this IE OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| else { |  |  |
| Group 2 Duration | variable | Group 2 Duration for F-FDD Support |
| Group 2 Slot Offset | variable | Group 2 Slot Offset for F-FDD Support |
| } |  |  |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| Dedicated UL Control Indicator | 1 | - |
| If (Dedicated UL Control Indicator ==1) { |  |  |
| Dedicated UL Control IE () | variable |  |
| } | - | - |
| UIUC | 4 |  |
| Repetition Coding Indication | 1 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| ACK Disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the BS in the HARQ ACK BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the subburst in the HARQ ACK BITMAP. For the burst, MS shall not perform HARQ retransmission and ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |

- Full Duplex

The full duplex flag shall be set to 1 if a full duplex mobile station shall transmit a single uplink burst using resources in both group 1 and group 2.  The exact resources are indicated by the the resources indicated in this IE

Group 2 Slot Offset

The group 2 slot offset shall be set to the first slot in the resource assignment.  The group 2 slot offset is defined with respect to the lowest numbered slot of the group 2 UL subframe if an allocation start indication for group 2 is not included in this IE, and the slot offset is defined with respect to the indicated OFDM symbol offset and subchannel offset if an allocation start indication for group 2 is included in this IE.

Group 2 Duration

The group 2 duration shall be set to the duration of the resource assignment in group 2.

[note to the editor: Please insert the following new text in the H-FDD Section]

8.4.4.1.2 Full Duplex Support

The base station may assign full duplex mobile stations to either group 1 or group 2.  If a full duplex mobile station is associated with group 1, and the base stations sets the full duplex flag of the Persistent HARQ UL MAP IE to 1, the mobile station shall transmit a single uplink burst using the resources indicated in the IE.  The total duration for the burst shall be calculated by the MS as the duration field plus the group 2 duration field within the Persistent HARQ UL MAP IE.