| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for a new Extended Subheader to report Persistent Allocation Errors |  |
| Date Submitted | 2008-03-19 |  |
| Source(s) | Yair Bourlas, NextWave Sean McBeath, Huawei Technologies Mo-Han Fong, Nortel | Source(s) ybourlas@nextwave.com smcbeath@huawei.com mhfong@nortel.com |
| Re: | Ballot 26b |  |
| Abstract | This contribution proposes error notification mechanism for persistent allocation procedures described in contribution C80216maint-08_095r6.doc |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for a new Extended Subheader to report Persistent Allocation Errors

Yair Bourlas, NextWave

Mo-Han Fong, Nortel

Sean McBeath, Huawei Technologies

# Introduction

Contribution IEEE C802.16maint-08/095r4 proposes persistent allocation mechanism, but does not cover error notification mechanisms associated with the persistent allocation.  This contribution proposes error notification mechanism for persistent allocation procedures described in contribution C80216maint-08_095r4.doc

# Proposed Text

Add text to IEEE P802.16Rev2/D3 as shown below; these changes must be implemented after the changes of C80216maint-08-095r4

[Make the following changes to section 6.3.27.4.1 Downlink Operation]

Otherwise, the MS shall  transmit an indication to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

Otherwise, the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and shall stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

[Make the following changes to section 6.3.27.4.2 Uplink Operation]

Otherwise, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation and the MAP ACK channel allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

Otherwise, the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation and the MAP ACK channel allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and terminate the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

[Add text to IEEE P802.16Rev2/D3, section 6.3.2.2.7 Extended subheader format table 26]

[Add to Table 26—Description of extended subheaders types (UL)]

| Extended subheader type | Name | Extended subheader body size (byte) | Description |
| --- | --- | --- | --- |
| … | … | … | … |
| 5 | Persistent Allocation Secondary Error | 1 | See section 6.3.2.27.9 |
| 56-127 | Reserved |  |  |

[Add new section after section 6.3.2.2.7.8]

6.3.2.2.7.9 Persistent Allocation Secondary Error

The Persistent Allocation Secondary Event is used by the MS to indicate failure with a persistent allocation.  The fields of the Persistent Allocation Secondary Error extended subheader are defined in Table XXX

Table XXX— Persistent Allocation Secondary Error Event extended subheader

| Name | Size (bit) | Description |
| --- | --- | --- |
| CID | 16 | Basic CID or Transport CID. The Basic CID indicates more than one transport CID is affected |