| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Post Processing SINR Calculation based on Midamble for band AMC |  |
| Date Submitted | 2008-03-19 |  |
| Source(s) | Kathiravetpillai Sivanesan, Peiying Zhu, Wen Tong, and Sang Youb Kim Nortel Networks 2221 Lakeside Blvd. Richardson, TX 75082 | E-mail: kathirsi<at>nortel.com |
| Re: | IEEE 802.16Rev2/D3, Letter Ballot 26b. Technical Comments |  |
| Abstract | Proposed a method to calculate the post processing SINR based on Midamble |  |
| Purpose | Adopt the proposed method in IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Post Processing SINR Calculation based on Midamble for band AMC

- Kathiravetpillai Sivanesan and Peiying Zhu

- Nortel Networks

- Introduction

For the CL-MIMO operation the best band selection, CQI calculation, PMI selection are based on the Midamble. As the Preamble is transmitted from single antenna it cannot be used for the MIMO channel estimation.

- Assumptions

Assume the 2X2 antennas configuration. The channel matrix

,

and the precoding matrix for the rank 1 and 2 are

.

For 3 and 6 bits feedback there are 8 and 64 codewords  available, respectively. The MS assumes the rank 2 reception is done using the MMSE receiver and the rank 1 reception is done using MRC receiver.

- Operation

  - The BS transmits the Midamble to support the band AMC operation

  - MS estimates the channel matrix H on the Midamble for each band.  MS calculates noise+ interference level in the midamble as well.

For the Rank 2 (2-stream) transmission the received signal with the precoding the received signal of the MIMO systems can be written as

- The MMSE filter coefficients, GMMSE is given by

Where  and number of transmit antennas denoted as M.

- The k-th decoded stream post processing SNIR is given by

- For the Rank -1 (1-stream) transmission the received signal is written as

- The post processing SNIR for the MRC combining at the receiver is

Where ,  and the noise plus interference power at the antennas 1 and 2 are  and , respectively.

The average capacity for the subcarriers or streams with SNIR_1, SNIR_2,….. SNIR_N is given by

nats.

- Then, the average post processing SNIR is given by

.

  - MS calculates the capacity for both the Rank 1 and Rank 2 transmission based on the post processing SNIR for all possible precoding codewords and all 12 bands. It chooses the 4 best band, corresponding PMIs and ranks based on the maximum capacity.