| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CINR measurement on Midamble for Band AMC MIMO operation |  |
| Date Submitted | 2008-03-20 |  |
| Source(s) | Peiyiing Zhu Nortel Networks 3500 Carling Ottawa, Ontario, Canada | E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16Rev2/D3, Letter Ballot 26b. Technical Comments |  |
| Abstract | Add support of CINR measurement on Midamble for Band AMC MIMO operation |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CINR measurement on Midamble for Band AMC MIMO operation

Peiying Zhu

Nortel Networks

# 1.	 Introduction

The contribution captured the agreed text in MIMO ad-hoc for comments 2060 and 2173 to add supports for measuring CINR on midamble for band AMC operation.

The highlevel agreement are:

Concept of the changes:

- Add TLV in REP-REQ/RSP for CINR measurement on the midamble (for band AMC measurement type only, no reuse 1/3)

- Add SBC-REQ/RSP bit for support of CINR from midamble

- Add text clarifying the post-precoding CINR calculation (in the MAC section where CINR mechanisms are defined):

## Proposed text changes:

Add the following TLV “Midamble Physical CINR request” to the table as "type 1.9", "length 1", and value in sedction 11.11.

| Midamble Physical CINR request | 1.9 | 1 | Midamble Physical CINR request is used with Channel Type Request = 0b01 (band AMC) to report CINR on the midamble. Bits 0–3: avg in multiples of 1/16 (range is [1/16,16/16]). Bit 4: 0: Report only mean of CINR. 1: Report both mean and standard deviation of CINR. Bit 5-6 0b00: report CINR assuming 1 stream 0b01: report CINR assuming 2 streams 0b10: report CINR using number of streams determined by MS 0b11: reserved Bits 7: reserved; shall be set to zero. |
| --- | --- | --- | --- |

Modify the following text on page 1149, line 45 as following

Measurements according to Zone-specific physical CINR request TLV (1.4) and, Preamble physical CINR request TLV (1.5) and Midamble physical CINR request TLV (1.9) shall use the αavg specified in these TLVs regardless of the value of the αavg in Report Type TLV (1.1).

Add the following row  in the Table on page 1151, line 41 in

| Bits #0-1 = 0b10 | The estimation of physical CINR measured from midamble in an STC zone with dedicated pilot | 2.16 | 5 | The estimation of physical CINR measured from midamble for Band AMC subchannel. Bit #39 ~ Bit #28 : Band Indication Bitmap (Bit #39 for Band with index 11, Bit #38 for Bandwith index 10 .. Bit#28 for Band with index 0) Bit #27 ~ Bit #3 : CINR reports for 5 selected Bands. (5 bits per each band. Band with lower index has lower significant 5 bits) Bit #2 ~ Bit #0 : Reserved |
| --- | --- | --- | --- | --- |

Modify the following text in section 6.3.18.1

6.3.18.1 DL CINR report with REP-RSP MAC message

[add the following text at the end of the subsection]

When calculating CINR from the midamble in an STC zone with dedicated pilots, the MS shall determine the best number of streams and precoder matrix (the choice is implementation specific), and calculate the Physical CINR if rank-1 and Avg_CINR if rank-2 according to 8.4.5.4.10.1, under assumption that the BS applies the chosen precoder and number of streams.