| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title |  |  |
| Date Submitted | 2008-03-20 |  |
| Source(s) | Erik Colban NextWave Wireless* | Voice: E-mail: ecolban@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Working Group Letter Ballot #26b |  |
| Abstract | Updated resolution to comment 2041 |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Update proposed resolution to comment 2041

Erik Colban

NextWave Wireless

[Replace the text in the Notes column for the Trigger Reference Indicator row in Table 143 by:]

0: In addition to triggers defined by Neighbor BS Trigger TLVs, the MS shall apply the triggers for actions 0x1 and 0x2 defined the DCD message that do not have the same type, function and action as a trigger defined in a Neighbor BS Trigger TLV.

1: In addition to triggers defined by Neighbor BS Trigger TLVs, the MS shall apply the triggers that it shall apply to the preceding neighbor BS that do not have the same type, function and action as a trigger defined in a Neighbor BS Trigger TLV.

[On page 425, lines 36 - 38, delete the following sentence, since it is not quite correct and is redundant given the definition of the action behavior (see p. 1172, Table 545):]

When the Trigger Action in the DCD message is encoded as 0x3, on trigger, MS starts neighbor BS scanning

process by sending MOB_SCN-REQ or by initiating Autonomous neighbor cell scanning (see 8.4.13.1.3) or

both.

[On page 425, delete lines 61-65:]

If the trigger type, trigger function and trigger action for a particular

neighbor BS as defined in MOB_NBR-ADV are the same as the neighbor BS trigger type, trigger function

and trigger action defined in the serving BS DCD, the trigger value and trigger averaging duration defined in

the MOB_NBR-ADV shall take precedence.

[On page 1189, lines 36-40, modify as follows:]

Trigger averaging duration is the time measured in

number of frames over which the metric measurements

are averaged. When the mean value of the measurement

meets the trigger condition, the MS reacts

using the specified action.

[Add to page 1190, line 38, the following sentence:]

If Neighbor BS Trigger TLVs are included in the MOB_NBR-ADV message, the MS may ignore Neighbor BS Trigger TLVs having a metric that the MS and BS have not agreed to support during SBC-REQ/RSP message exchange.

When the mean value of the MS's measurements over the averaging interval of a trigger defined by a Neighbor BS Trigger TLV meets the trigger condition as specified by the type, function,  and value of the trigger, the  MS shall invoke the trigger's specified action. If the conditions of more than one trigger are met simultaneously, the MS shall select at least one of the trigger actions.

[On page 1071, lines 31-34, modify as follows:]

Trigger averaging duration is the time measured in number of

frames over which the metric measurements are averaged. When

the mean value of the measurement meets the trigger condition,

the MS reacts using the specified action.

Add to page 1072, line 52:]

If Trigger TLVs are included in the DCD message, the MS may ignore Trigger TLVs having a metric that the MS and BS have not agreed to support during SBC-REQ/RSP message exchange..

When the mean value of the MS's measurements over the averaging interval of a trigger defined by a Trigger TLV meets the trigger condition as specified by the type, function,  and value of the trigger, the  MS shall invoke the trigger's specified action. If the conditions of more than one trigger are met simultaneously, the MS shall select at least one of the trigger actions.

Triggers specified in this TLV may be overridden by triggers specified in the Neighbor BS Trigger TLV (see 6.3.2.3.42, Table 143)

The set of triggers defined by the Trigger TLVs in the DCD_settings compound TLV for a given neighbor BS defines the complete set of triggers in the DCD message of that BS. When this TLV does not occur in the DCD_settings compound TLV for a given neighbor BS, the set of Trigger TLVs  in the neighbor BS's DCD message is identical to the set of Trigger TLVs in the current BS.