| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Introduction of Access Restriction into the RNG-RSP message |  |
| Date Submitted | 2008-05-12 |  |
| Source(s) | David Fox Vodafone Group PLC Newbury, UK | Voice: +44 7919308756 E-mail: david.fox@vodafone.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #26c on IEEE Rev2/D4 |  |
| Abstract | In the current definition of PKMv2 it is not possible to reject the authentication process, as was provided by the Auth Reject message in PKMv1. This type of functionality to force a SS to not attempt authentication again is important to successful operation of a mobile network. For example, it is needed for the BS to inform the SS that the authentication is not possible through this access network or through a particular NSP. This would occur in the case when no roaming agreement exists between the network and home network of the SS. Without this functionality these SS would repeatedly request access from a WiMAX network which could appear like a DDoS to the WiMAX network, and unnecessarily drain the battery of the SS. |  |
| Purpose | Adopt the proposed text and incorporate into 802.16 Rev2/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction of Access Restriction into the RNG-RSP message

David Fox

Vodafone Group PLC

# Problem Description

It is desired that terminals operate assuming that all networks will provide it service, because terminals will be able to connect to new networks without explicitly being pre-provisioned with knowledge of each network, and thereby maximizing connectivity for the terminal.

This model works when roaming agreements exist between the home and visited networks, however in some cases this is not the case, and the visited network needs the ability to inform the device that it should not register on this network.

This problem is most prevalent when national roaming is prohibited, so each time a terminal falls out of coverage of its home network and finds coverage of another network, which happens frequently even in very established networks such as GSM, the device needs to know that it should attempt to access these networks on each occasion of coverage loss.

Within the current specification a network does not have the ability to inform a device it should not attempt to register on this network.

# Proposed Changes

Change #1: Add the following TLVs and its description in “RNG-RSP message”:

Adopt the following changes in section 6.3.2.3.6 RNG-RSP (ranging response) message:

The following TLV parameters may be included in an unsolicited RNG-RSP message:

Rendezvous time

This is the offset, measured in units of frame duration, when the BS is expected to provide a

non-contention-based ranging opportunity for the MS. The offset is calculated from the frame

where RNG-RSP message is transmitted. The BS is expected to provide the non-contention-

based ranging opportunity at the frame specified by the rendezvous time parameter.

CDMA code

A unique code assigned to the MS, to be used for dedicated ranging. The code is from the initial

ranging codeset.

Transmission Opportunity Offset

A unique transmission opportunity assigned to the MS, to be used for dedicated ranging, in

units of symbol duration.

Access Restriction

A parameter included when the BS requires the SS to terminate communication with the BS, and provides a command to the SS regarding when the SS may redo ranging.

Section 6.3.9.5.1, page 311, line 33 insert as:

If Access Restriction flag is set to 1 in HO type support field of DCD message, the BS may include the Access Restriction information in an unsolicited RNG-RSP message. On receiving a RNG-RSP including the Access Restriction TLV the SS shall cease communication with the BS aborting a network entry attempt and shall not redo ranging as follows:

If Access Restriction indicates “No re-entry in BS”, the SS shall not attempt Network Entry on this BS again until the SS has been manually powered down.

If Access Restriction indicates “No re-entry in NSP”, the SS shall not attempt Network Entry on this NSP again until the SS has been manually powered down.

If Access Restriction indicates “No re-entry in NAP”, the SS shall not attempt Network Entry on this NSP again until the SS has been manually powered down.

If Access Restriction indicates “Illegal Device”, the SS shall not attempt Network Entry on this NAP again until the SS has been manually powered down.

Change #2: Add the following TLVs in Table 552 - RNG-RSP message encodings:

Add the following TLV into Table 552:

Table 552—RNG-RSP message encodings

| Name | Type (1 byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| Access Restriction | 39 | 1 | Is sent by BS to command SS to abort the current network entry attempt. This parameter indicates the restriction on network entry for the SS as follows: 0x00 = No re-entry in BS 0x01 = No re-entry in NSP 0x02 = No re-entry in NAP 0x03 = Illegal Device | All |

Change #3: Enhancement of Table 563 “DCD channel encoding”:

Change the following entry in Table 563 as indicated:

Table 563—DCD channel encoding

| Name | Type (1 byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| HO type support | 50 | 1 | Bit 0: HO Bit 1: MDHO Bit 2: FBSS HO Bit 3: Access Restriction Bit 34–7: Reserved | OFDMA |