| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Resolve DSx and HO interfere issues |  |
| Date Submitted | 2008-04-16 |  |
| Source(s) | Lucy Pollak Altair Semiconductor 6 Haharash St. Hod Hasharon 45240, Israel | Voice: 972-9-7403045 E-mail: lucy.pollak@altair-semi.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #26c as announced in IEEE 802.16-08/018 |  |
| Abstract | The DSx transactions, which are not transparent and could not be continued on new serving BS need to be terminated before HO. The proper termination mechanism is not present in the specification. The DSx Transactions Update TLV sent by HO initiator will help to terminate outstanding DSx transactions and provide Service Flow configuration synchronization between all devices involved into HO process. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Resolve DSx and HO interfere issues

Lucy Pollak

Altair Semiconductor

# Background

# This contribution is an alternative way to resolve the problem defined in IEEE C802.16maint-08/135 Contribution:

# “Handovers are not transparent DSx transactions, i.e., a transaction started at the serving BS cannot continue at the target BS, even in the case of a full optimization handover process. First, the standard does not address any synchronization of the transaction state between the serving and the target BS (although it does address many other so-called dynamic state attributes). Secondly, the target BS manages its own resources and may not have the same resources available as the serving BS. The current DSx transactions are not flexible enough to handle such situations. Thirdly, messages sent by the MS or the serving BS may or may not be received by the other party. Therefore, although a transaction may seem to be completed by one entity it may not be completed at the other entity.”

# The proposed mechanism to terminate all DSX transactions before HO at both MS and serving BS is based on the approach that HO negotiation messages is the better place to signal about DSx transaction termination. Both devices will be involved in negotiation process and may terminate the outstanding transactions properly and synchronize SF’s context.

# The mechanism gives the opportunity to start HO execution with static SF context only, which simplify the further application of Connect Info TLV, which may be received in RNG-RSP and which contains DSC-like SF encodings.

# The other advantage of this proposal is that RNG-RSP is not involved into this synchronization mechanism and may be omitted.

# Proposed Changes

The new DSx Transaction Update TLV distributed with HO request message will provide the other device with information, which DSx transaction need to be successfully completed similar to “DSx Ended” event processing or terminated unsuccessfully similar to DSX-RSP/ACK with erred confirmation code processing.

Both mechanisms are already used by DSx SMs.

# Proposed Changes in 802.16Rev2/D4

[On page 450, Section “6.3.22.2.8.1.6 Service flows settings”, add the new subsection at the end of the section:]

6.3.22.2.8.1.6.8 Service flows - dynamic context for SFs in Added and Changed DSx states

MS context with Serving BS: All dynamic context for outstanding DSx transactions is moved to static context in accordance with rules defined below. All Service flows in Added or Changed DSx states return to Nominal state.

MS context with Target BS: Only Service flow static configuration context is in use.

The MS and BS shall not start the DSA and DSC transactions if HO negotiation has already been started. The MS and BS shall postpone this operation until the HO negotiation failure or HO cancellation. When the MS will complete network re-entry on new serving BS it may start postponed DSA or DSC transaction with new serving BS.

If HO will start when there are some outstanding DSA or DSC transactions, the HO initiator (MS or BS) shall place the list of all outstanding DSA and DSC transactions into MOB_MSHO-REQ or MOB_BSHO-REQ respectively with indication for each transaction if it was successfully completed or terminated. The transaction is considered as successfully completed only if correspondent DSx-ACK management message with successful confirmation code was already sent or received. The presence of some transaction in MOB_MSHO-REQ or MOB_BSHO-REQ message shall be considered as DSA-ACK or DSC-ACK, if it was lost or has not been received yet. If some transaction in this list is indicated as terminated, it will cause for MS or BS to terminate transaction regardless of DSx SM state and revert SF configuration information. The successfully completed transaction will be ended at both devices.

The DSx transaction may be started and completed at both devices at a different time. The absence of some transaction in the DSX transaction list will be treaded differently regarding of DSx transaction state and only in “Holding Down” state the transaction shall be considered as finished. The presence of some transaction, which is unknown or completed by other device, will not impact its operations.

When the MS will fail in HO negotiation or HO processing and return to normal operation with its previous serving BS or will successfully complete network re-entry with its target BS, it may repeat the unsuccessfully terminated DSA or DSC transaction.

[On page 223, Section “6.3.2.3.47 MOB_BSHO-REQ (BS HO request) message, apply the following changes:]

The MOB_BSHO-REQ may contain the following TLV:

DSx Transaction Update (see 11.15.2)

The MOB_BSHO-REQ message shall include the following parameter encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

[On page 227, Section “6.3.2.3.48 MOB_MSHO-REQ (MS HO request) message”, , apply the following changes:]

The MOB_MSHO-REQ may contain the following TLV:

DSx Transaction Update (see 11.15.2)

The MOB_MSHO-REQ message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

[On page 1266, Section “11.15 HO management encodings”, add the new subsection at the end of the section:]

11.15.2 DSx Transaction Update

The DSx Transaction Update encoding field is sent by HO initiator to terminate outstanding DSx transactions and provide Service Flow configuration synchronization between all devices involved into HO process.

| Name | Type (1 byte) | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| DSx Transaction Update | 2 | variable | Compound | MOB_BSHO-REQ MOB_MSHO-REQ |

The following TLV elements may appear in a DSx Transaction Update TLV.

| Name | Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- | --- |
| DSx Succeeded TA List | 2.1 | 2*n | List of successfully completed DSA and DSC transaction IDs |
| DSx Failed TA List | 2.2 | 2*m | List of terminated DSA and DSC transaction IDs |