| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Resolve DSx and HO interfere issues |  |
| Date Submitted | 2008-05-15 |  |
| Source(s) | Lucy Pollak Altair Semiconductor 6 Haharash St. Hod Hasharon 45240, Israel Erik Colban Nextwave | Voice: 972-9-7403045 E-mail: lucy.pollak@altair-semi.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #26c as announced in IEEE 802.16-08/018 |  |
| Abstract | The DSx transactions, which are not transparent and could not be continued on new serving BS need to be terminated before HO. The proper termination mechanism is not present in the specification. The DSx Transactions Update TLV sent by HO initiator will help to terminate outstanding DSx transactions and provide Service Flow configuration synchronization between all devices involved into HO process. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Resolve DSx and HO interfere issues

Lucy Pollak

Altair

Erik Colban

NextWave

# Background

# This contribution is an alternative way to resolve the problem defined in IEEE C802.16maint-08/135 Contribution:

# “Handovers are not transparent DSx transactions, i.e., a transaction started at the serving BS cannot continue at the target BS, even in the case of a full optimization handover process. First, the standard does not address any synchronization of the transaction state between the serving and the target BS (although it does address many other so-called dynamic state attributes). Secondly, the target BS manages its own resources and may not have the same resources available as the serving BS. The current DSx transactions are not flexible enough to handle such situations. Thirdly, messages sent by the MS or the serving BS may or may not be received by the other party. Therefore, although a transaction may seem to be completed by one entity it may not be completed at the other entity.”

# Proposed Changes

# Proposed Changes in 802.16Rev2/D4

[On page 450, Section “6.3.22.2.8.1.6 Service flows settings”, add the new subsection at the end of the section:]

6.3.22.2.8.1.6.8 Service flows - dynamic context for SFs in Added and Changed DSx states

MS context with Serving BS: All dynamic context for outstanding DSx transactions is moved to static context in accordance with rules defined below. All Service flows in Added, Deleted, or Changed DSx states return to Nominal state.

MS context with Target BS: Only Service flow static configuration context is in use.

The MS and BS shall not start a DSA, DSC or DSD transaction if a HO has been initiated.

11.7.10 CID Update Encodings field

The CID Update Encodings field provides a translation table that allows an MS to update its service flow and connection information so that it may continue service after an HO to a new serving BS.

| Name | Type (1 byte) | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| CID_update | 24 | variable | Compound | REG-RSP |

These TLV values shall appear in each CID Update TLV.

| Name | Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- | --- |
| New_CID | 24.1 | 2 | New CID after HO to new BS |
| SFID | 24.2 | 4 | Service flow ID |

The following TLV element may appear in a CID Update TLV.

| Name | Type (1 byte) | Length | Value (variable length) |
| --- | --- | --- | --- |
| Connection Info | 24.3 | variable | If any of the service flow parameters change, then those service flow parameter encoding TLVs that have changed will be added. Connection Info is a compound TLV value that encapsulates the service flow parameters that have changed for the service. All the rules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsulated in this TLV. |

The following TLV shall be included if a DSC transaction for the SF was ongoing at the serving BS at the time handover started.

| Name | Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- | --- |
| DSC transaction termination indication | 24.4 | 1 | 0-successfully completed DSC transaction 1-DSC transaction unsuccessfully terminated |

11.7.10.1 Compressed CID Update Encodings field

The Compressed CID Update Encodings field provides a translation table that allows an MS to update its CID. Only CIDs that have no parameter change can be translated by this TLV.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Compressed CID update | 25 | variable | The first byte indicates the length of the following BITMAP in bytes. The n-th bit, starting from the MSB of the BITMAP is set to 1 when the n-th SFID is to be updated to a new CID. Where, the SFIDs are sorted with increasing order. After the BITMAP, a list of new CID follows. The number of new CID is equal to the number of ones in the BITMAP. Optionally, a second and a third bitmap, which are always used together, may be included. Each of them has the same length as the first bitmap. The second bitmap indicates if there was ongoing DSC transaction at the serving BS for the specified SF at the time handover started. For each bit set to 1 in the second bitmap the corresponding bit in third bitmap indicates: 0-successfully completed DSC transaction 1-DSC transaction unsuccessfully terminated | REG-RSP |

[On page 1077, Section “11.5 RNG-REQ management message encodings”, add new row to the end of the table 549:]

| Name | Type (1 byte) | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| HO connection synchronization indication | 22 | 1 | 0 – no connection synchronization problems expected 1- possible connection synchronization problems | _ |

The HO connection synchronization indication TLV may be included if there is a potential synchronization problem of the SF states between the MS and the BS, and indicates a request for the uncompressed CID Update TLV in the RNG-RSP message.