| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Formulas for Determining H-FDD Frame Partitions |  |
| Date Submitted | 2008-04-17 |  |
| Source(s) | Labonte, Sylvain, SEQUANS 19 Le Parvis de la Defense 92073 Paris La Defense CEDEX France | Voice: +33 1 41 02 82 99 E-mail: sylvain@sequans.com |
| Re: | LB#26c concerning IEEE P802.16 Rev2 Draft 4 |  |
| Abstract | This paper presents the formulas MSs may use to determine H-FDD frame partitions. |  |
| Purpose | Adopt the LB#26c comment associated with this contribution, and incorporate the proposed text of this contribution in IEEE P802.16 Rev2 Draft 5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Formulas for Determining H-FDD Frame Partitions

Sylvain Labonte, SEQUANS Communications

# Introduction

In 802.16Rev2/D4, Section 8.4.4.1 describes the FDD frame structure and in particular Figure 223 illustrates the FDD frame.  The text and the figure constitute an implicit description of the manner in which a MS may determine the frame partition.  The proposed text of this contribution removes any potential ambiguity by describing the explicit rules a MS may use to determine the partition.

# Proposed text

[To the editor: Please add following text…]

The MS may assume that the following formulas will yield consistent and accurate results for determining the DL frame partition in Frame n:

First symbol of Group 2 = NS1_DL[n] + TLV24[n](1:2) + 1              in symbols

= NSF[n] - NS2_DL[n-1] + 1                        in symbols

Start of Group 2 = (NS1_DL[n] + TLV24[n](1:2)) * Ts  + Z[n]           in seconds

= (NSF[n] - NS2_DL[n-1] ) * Ts  + Z[n]                   in seconds

Number of symbols in Group 2 = NS2_DL[n-1]

= NSF[n] – NS1_DL[n] – TLV24[n](1:2)

Where:

TLV24[n](7:0) = TLV24 of the DCD message corresponding to the DCD count announced in DL MAP of frame n, see Table 563

NS1_DL[n] = Number of symbols announced in DL MAP of group 1 in frame n (includes preamble)

NS2_DL[n] = Number of symbols announced in DL MAP of group 2 in frame n

Tf[n] = duration of frame n, announced in DL-MAP of frame n see 8.4.5.2

Ts = OFDMA symbol duration see 8.4.2.4

NSF[n]  =  floor(Tf[n]/Ts)                    in symbols

UnusedGap[n] = Tf[n] – NSF[n] * Ts      in seconds

Z[n]  	= 0 when TLV24(0) = 1 (unused gap is at the end of the frame)

= UnusedGap[n] when TLV24[n](0) = 0 (unused gap is between the groups).