| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fast Ranging optimization |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Giovanni Maggi, Aik Chindapol, Achim Von Brandt Nokia Siemens Networks* | Voice: E-mail: Aik.Chindapol@nsn.com, Achim.Brandt@nsn.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26c |  |
| Abstract | This contribution propose a fast ranging optimization for HO across the network |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Fast Ranging optimization

Achim von Brandt

Nokia Siemens Networks

# Problem description

Currently MS may request HO preparation for any number of candidate BSs using MOB_MSHO-REQ. The serving BS coordinates with candiate BSs and sends MOB_BSHO-RSP with one action time. The standard specifies the use of the smallest of the received action times as the action time in MOB_BSHO-RSP. However the smallest value may become invalid if the serving BS has to wait for responses from potential target BSs that cannot respond back in a timely manner. Although the serving BS can exclude these target BSs from the recommended list, MS may not be able to perform fast ranging with these BSs.

Therefore it is necessary to loosen the definition of Action Time.

# Proposed Changes

Change the following sentence in Sec 6.3.22.2.4 as follows:

It is recommended that Action Time is be the earliest time (i.e., smallest value) for allocation of fast ranging opportunity that was committed by the target BSs recommended in the message.

# References

[IEEE802.16-Rev2/D3]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D3 (February 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.