| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Persistent Allocation |  |
| Date Submitted | 2008-04-21 |  |
| Source(s) | Changyoon Oh Samsung Electronics | Voice: E-mail: changyoon.oh@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26c |  |
| Abstract | This contribution proposed the text clarification on persistent allocation. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Persistent Allocation

Samsung Electronics

# 1. Problem description

# In the current 802.16 standard, some operation of persistent allocation are not clear.

# Those includes

# - Submap Operation

# - Error Handling procedure

# Submap Operation

When submap in persistent allocation is supported, it is not clear when MS sends MAP NACK. Several options are possible.

Option I. When MS fails to decode the primary MAP at frame K

- BS deallocates persistent allocation by primary MAP

Option II. When MS fails to decode either primary MAP or the submap, which was used for initial persistent allocation

- BS deallocates persistent allocation by submap, which was used for initial persistent allocation

# Error Handling Procedure

BS shall send change indicator=1 for the assigned HARQ Region ID at frame K+ap (allocation period) when there is a change of HARQ Region or deallocation command for the assigned HARQ Region ID at frame K.

# Current draft does not consider the change of HARQ Region.

# For completeness, the error recovery procedure should reflect the case of flexible HARQ region.

- The MS receives a Persistent HARQ DL/UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the new HARQ Region information (symbol/subchannel offset, Number of symbols/subchannels).

# Proposed Changes:

6.3.27.3.11.2.1 Downlink Operation

If the MS fails to decode the DL-MAP in a frame which is relevant to a frame in which it has a persistent DL resource allocation (frame K), the MS shall not transmit data bursts or control signals in the relevant UL sub-frame (including CDMA ranging, CQICH, HARQ ACK/NAK or sounding signals) and may send a MAP NACK on its assigned MAP NACK Channel in the frame following the relevant UL sub-frame (K+2) as shown in Figure YYY.  However, if the MS fails to decode UL-MAP in frame K+1, the MS shall not send a MAP NAK in frame K+2 on its assigned MAP NACK Channel.

Upon reception of the MAP NACK indication, the BS should determine if the persistent allocation needs to be updated.   The MS that fail to decode the DL-MAP in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.289) if any one of the following conditions is true in frame K + ap:

- The MS receives a Persistent HARQ DL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 0.

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the Retransmission Flag of at least one sub-burst IE is set to 1

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the new HARQ Region information (symbol/subchannel offset, Number of symbols/subchannels).

Otherwise, the MS shall transmit an indication to the BS the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and shall terminate stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and shall terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

6.3.27.3.21.2.2 Uplink Operation

If the MS fails to decode the UL-MAP in a frame which is relevant to a frame in which it has a persistent UL resource allocation (frame K), the MS shall not transmit data bursts or control signals in the relevant UL sub-frame (including CDMA ranging, CQICH, HARQ ACK/NAK or sounding signals) and may send a MAP NACK on its assigned MAP NACK Channel in the frame following the relevant UL sub-frame as shown in Figure YYY.

Upon reception of the MAP NACK indication, the BS should determine if the persistent allocation needs to be updated.   The MS that fail to decode the UL-MAP in a frame K (frameMAP), which is relevant to a frame in which it has a persistent UL resource allocation (frame K), shall resume using the persistent allocation in the frame relevant to frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true in frameMAP + ap:

- The MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 0.

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the Retransmission Flag of at least one sub-burst IE is set to 1

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the new HARQ Region information (symbol/subchannel offset, Number of symbols/subchannels).

Otherwise, the MS shall transmit an indication to the BS the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and shall terminate the persistent assignment including the data allocation, and the MAP NACK allocation and the MAP ACK channel allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS the MS should attempt to send the Persistent Allocation Secondary Error extended subheader to the BS and shall terminate the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

6.3.27.51.4   Change Indicator

A Change Indicator for each HARQ Region is included in the Persistent HARQ DL MAP IE.  The change indicator shall be set to 0 to indicate that there were no changes, i.e., no deallocation/HARQ region change,  to the any DL persistent allocations in frame K - allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.

A Change Indicator for each HARQ Region is included in the Persistent HARQ UL MAP IE.  The change indicator shall be set to 0 to indicate that there were no changes, i.e., no deallocation/HARQ region change, to the any UL persistent allocations in frame K - allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.

Change indicator in frame K is used for MS who failed to receive the DL-MAP or UL-MAP in frame K – allocation period to recover the persistent allocation, i.e., when MS, who failed to receive the DL-MAP or UL-MAP in frame K – allocation period, shall continue the persistent allocation, when the change indicator is set to 0 in frame K.

# References

[IEEE802.16-Rev2/D4]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D4 (April 2008) (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007)