| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Persistent Allocation – Error handling procedures |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | Yair Bourlas, NextWave Mo-Han Fong, Nortel Geunhwi Lim, Samsung Electronics Jianmin Lu, Huawei Technologies Sean McBeath, Huawei Technologies Anuj Puri, Beceem | Voice: E-mail: ybourlas@nextwave.com mhfong@nortel.com geunhwi.lim@samsung.com lujianmin@huawei.com smcbeath@huawei.com apuri@beceem.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Ballot 26c |  |
| Abstract | Error handling procedures can be simplified if we were to tie error procedures of the MS to the allocation of a MAP NACK channel. |  |
| Purpose | Accept the proposed changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Simplified Error Procedures for Persistent Allocations

Yair Bourlas, NextWave

Sean McBeath, Huawei

Mo-Han Fong, Nortel

Geunhwi Lim, Samsung Electronics

Jianmin Lu, Huawei Technologies

Anuj Puri, Beceem

# Introduction

Error handling procedures can be simplified if we were to tie the MS error procedures to the allocation of a MAP NACK channel.

It is noted that R2/D4 allows the base station to optionally allocated persistent resources without allocating a MAP NACK channel, but that the error procedures defined in R2/D4 do not have an adequate description of the MS behavior in the case where MAP NACK channel is not assigned.  Moreover, the MS uses the assigned MAP NACK channel to report MAP decoding errors for the primary MAP and that sub map decoding errors are not reported using the MAP NACK channel.  There is a need to clarify the system behavior in the case the MS is allocated persistent resources using a sub map and the MS has experienced a decoding error in that sub map in subsequent frames

# Proposed Solution

- Allow Base Station to control the type of error procedures the MS should follows: Primary or Secondary

- We observe that error handling procedures are made trivial if it is known that resource shifting is not used or HARQ region has not changed, both of which are under the control of the base station.

- We also observe that some systems would not invoke resource shifting, or would use it sparingly, and therefore would be motivated to take advantage of the simplified error handling procedures.

- It is noted that MAP NACK channel assignment is optional in R2/D4 and that we need to clarify MS operations in the case where MAP NACK channel has not been assigned.

- The benefit of the simplified error procedures is that it will result in a more robust system which is less prone to MAP (and sub MAP) errors.

Simplified Error Procedures – without resource shifting

- MAP ACK for allocation

- MAP ACK for de-allocation

- Change indicator to allow BS to control MS error procedures – primary or secondary

Benefits

- MS can lose multiple MAPs, e.g. K, K+4, K+8, and resume at the next allocation period, in which it successfully decoded the MAP, e.g. K+12

- MS can lose MAPs K and K+1 and resume at K+4, without having to resort to secondary error procedures.

- NO need for MAP NACK; MAP NACK channel allocation set to 1’s

The benefit of the simplified error procedures is that it will result in a more robust system which is less prone to MAP (and sub MAP) errors.

Add clarifications to R2/D4 as follows:

- If MAP NACK channel is not assigned (MAP NACK index = 111111), the MS uses ‘simplified’ error procedures.

- It should be noted that without the MAP NACK channel, resource shifting procedures and HARQ region changes are prone to errors which may go undetected.  Therefore, resource shifting or HARQ region changes should not be used if any of the MSs in a given HARQ region ID is assigned a persistent resource without an associated MAP NACK channel.

# Proposed Text Changes

Remedy 1:

Add text as shown below in blue to section 6.3.27.1.8 to clarify that the error procedures described in that section are applicable to MSs with a MAP NACK channel assignment.  Added text is shown in blue.

6.3.27.1.8 Error Recovery

The following downlink and uplink error procedures apply to MSs with MAP NACK channel assignment, (the MAP NACK field is not set to 0b111111.)

Remedy 2:

Create a new section under section 6.3.27.1 Error Handling procedures describing the error procedures for MS without a MAP NACK channel assignment.

6.3.27.1.8.3 MS Error procedures if MAP NACK channel is not assigned

The following downlink and uplink error procedures apply to persistent allocations with the MAP NACK field set to 0b111111.

Resource shifting and HARQ region changes should not be used if any of the MSs in a given HARQ region are assigned a persistent resource without an associated MAP NACK channel.

6.3.27.1.8.3.1 Downlink Operation

The MS that fail to decode the DL-MAP in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.29) if any one of the following conditions is true in frame K + N*ap:

- The MS receives a Persistent HARQ DL MAP IE for the assigned Persistent Region ID with the allocation flag set to 1, which includes the MS’s RCID.  This is a new persistent allocation which replaces the previous persistent allocation for the assigned Persistent Region ID.

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 0.

If the MS successfully decodes the DL-MAP at frame K + N*ap and none of the conditions above is true, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and shall stop using the persistent assignment including the data allocation, and the ACK channel allocation.

Note that the MS can recover from multiple MAP losses, i.e. loss of MAP in frames K and K+1 or loss of MAP in frames K, K+ap, K+2*ap, etc.

6.3.27.1.8.3.2 Uplink Operation

The MS that fail to decode the UL-MAP in a frame (frameMAP), which is relevant to a frame in which it has a persistent UL resource allocation (frame K), shall resume using the persistent allocation in the frame relevant to frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.4.30) if any one of the following conditions is true in frameMAP + N*ap:

- The MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID. This is a new persistent allocation which replaces the previous persistent allocation for the assigned Persistent Region ID.

- The MS receives a Persistent HARQ UL MAP IE for the assigned Persistent Region ID, which has the Change Indicator for the assigned Persistent Region ID set to 0.

If the MS successfully decodes the UL-MAP at frame K + N*ap and none of the conditions above is true, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and shall stop using the persistent assignment.

Note that the MS can recover from multiple MAP losses, i.e. loss of MAP in frames K and K+1 or loss of MAP in frames K, K+ap, K+2*ap, etc.