| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Persistent Scheduling – Change Indicator |  |
| Date Submitted | 2008-04-22 |  |
| Source(s) | Hua Xu, John Harris, Steven Xu Motorola Sean McBeath Huawei | Voice: E-mail: hua.xu@motorola.com smcbeath@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Ballot 26c |  |
| Abstract | The resource allocation scheme of 802.16e is extended to enable persistent allocations (also referred to as periodic allocations). |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Persistent Scheduling – Change Indicator

Hua Xu, John Harris, Steven Xu,   Motorola

Sean McBeath,   Huawei Technology

# Introduction

Modify the description of the Change Indicator to cover more scenarios.

# Proposed Text

6.3.27.3  MAP NACK Channel

6.3.27.3.1 Downlink Operation

If the MS fails to decode the DL-MAP in a frame which is relevant to a frame in which it has a persistent DL resource allocation (frame K), the MS shall not transmit data bursts or control signals in the relevant UL sub-frame (including CDMA ranging, CQICH, HARQ ACK/NAK or sounding signals) and may send a MAP NACK on its assigned MAP NACK Channel in the frame following the relevant UL sub-frame (K+2) as shown in Figure 154.  However, if the MS fails to decode UL-MAP in frame K+1, the MS shall not send a MAP NAK in frame K+2 on its assigned MAP NACK Channel.

Figure 154: Example MAP NACK Relevance

[Make a correction in the second line of the following paragraph and add a sentence below the bullets.  Insertion is in blue, deletion is in red.]

Upon reception of the MAP NACK indication, the BS should determine if the persistent allocation needs to be updated.  The MS that fail to decode the DL-MAP in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true:

- The MS receives a Persistent HARQ DL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator set to 0.

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator set to 1, and the Retransmission Flag of at least one sub-burst IE is set to 1

Otherwise, the MS shall transmit an indication to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

6.3.27.3.2 Uplink Operation

If the MS fails to decode the UL-MAP in a frame which is relevant to a frame in which it has a persistent UL resource allocation (frame K), the MS shall not transmit data bursts or control signals in the relevant UL sub-frame (including CDMA ranging, CQICH, HARQ ACK/NAK or sounding signals) and may send a MAP NACK on its assigned MAP NACK Channel in the frame following the relevant UL sub-frame as shown in Figure 154.

Upon reception of the MAP NACK indication, the BS should determine if the persistent allocation needs to be updated.   The MS that fail to decode the UL-MAP in frame K shall resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true:

- The MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator set to 0.

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator set to 1, and the Retransmission Flag of at least one sub-burst IE is set to 1

Otherwise, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation and the MAP ACK channel allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

[Delete Section 6.3.27.5 Change Indicator.  Insertion is in blue, deletion is in red.]

6.3.27.5   Change Indicator

A Change Indicator is included in the Persistent HARQ DL MAP IE.  The change indicator shall be set to 0 to indicate that there were no changes to the any DL persistent allocations in frame K - allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.

A Change Indicator is included in the Persistent HARQ UL MAP IE.  The change indicator shall be set to 0 to indicate that there were no changes to the any UL persistent allocations in frame K - allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.

Change indicator in frame K is used for MS who failed to receive the DL-MAP or UL-MAP in frame K – allocation period to recover the persistent allocation, i.e., when MS, who failed to receive the DL-MAP or UL-MAP in frame K – allocation period, shall continue the persistent allocation, when the change indicator is set to 0 in frame K.

[Add a section 6.3.27.5 Error Recovery.  Insertion is in blue, deletion is in red.]

6.3.27.5 Error Recovery

6.3.27.5.1 Downlink Persistent Allocations

The MS that fails to decode the DL-MAP in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true:

- The MS receives a Persistent HARQ DL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator set to 0.

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator set to 1, and the Retransmission Flag of at least one sub-burst IE is set to 1

where a Change Indicator is specified in the Persistent HARQ DL MAP IE and can be set to 0 or 1.

Otherwise, the MS shall transmit an indication to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the MAP ACK channel allocation.

6.3.27.5.2 Uplink Persistent Allocations

The MS that fails to decode the UL-MAP in frame K shall resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true:

- The MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator set to 0.

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator set to 1, and the Retransmission Flag of at least one sub-burst IE is set to 1

where a Change Indicator is specified in the Persistent HARQ UL MAP IE and can be set to 0 or 1.

Otherwise, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation, the MAP NACK allocation and the MAP ACK channel allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and terminate the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

[Modify Table 362 as shown.  Insertion is in blue, deletion is in red.]

8.4.5.3.29 Persistent HARQ DL MAP Allocation IE

Table 362: Persistent HARQ DL MAP allocation IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent HARQ DL MAP IE () { | -- | -- |
| Extended-2 DIUC | 4 | Persistent HARQ DL MAP IE = 0D |
| Length | 8 | Length in bytes |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this Persistent HARQ DL MAP (FDD/H-FDD Only) |
| Change Indicator | 1 | 0: No Change Occurred 1: Change Occurred Can be set to 0 or 1. See section 6.3.27.3.1. |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| OFDMA Symbol offset | 8 | Offset from the start of DL sub-frame |
| Subchannel offset | 7 |  |
| Number of OFDMA symbols | 7 |  |
| Number of subchannels | 7 |  |
| Rectangular sub-burst indication | 1 | Indicates sub-burst allocations are time-first rectangular. The duration field in each sub-burst IE specifies the number of subchannels for each rectangular allocation. This is only valid for AMC allocations and all allocations with dedicated pilots. When this field is clear, sub-bursts shall be allocated in frequency-first manner and the duration field reverts to the default operation |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition TLV in DCD |
| } | -- | -- |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to the first allocation instance only; |
| Mode | 4 | Indicates the mode in this HARQ region 0b0000: Persistent Chase HARQ 0b0001: Persistent Incremental redundancy HARQ for CTC 0b0010: Persistent Incremental redundancy HARQ for Convolutional Code 0b0011: Persistent MIMO Chase HARQ 0b0100: Persistent MIMO IR HARQ 0b0101: Persistent MIMO IR HARQ for Convolutional Code 0b0110: Persistent MIMO STC HARQ 0b0111 to 0b1111 : reserved |
| Subburst IE Length | 8 | Length, in nibbles, to indicate the size of the subburst IE in this HARQ mode. The MS may skip DL HARQ Subburst IE if it does not support the HARQ mode. However, the MS shall decode NACK Channel field from each DL HARQ Subburst IE to determine the UL ACK channel it shall use for its DL HARQ burst. |
| if (Mode == 0b0000) { | -- | -- |
| Persistent Chase HARQ sub-burst IE | variable | -- |
| } else if (Mode == 0b0001) { |  |  |
| Persistent Incremental redundancy HARQ for CTC sub-burst IE | variable | -- |
| } else if (Mode == 0b0010) { | -- | -- |
| Persistent Incremental redundancy HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0011) { | -- | -- |
| Persistent MIMO Chase HARQ | variable | -- |
| } else if (Mode == 0b0100) { |  |  |
| Persistent MIMO IR HARQ | variable | -- |
| } else if (Mode == 0b0101) { | -- | -- |
| Persistent MIMO IR HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0110) { |  |  |
| Persistent MIMO STC HARQ | variable |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| } |  |  |

Change Indicator

The change indicator shall be set to 0 to indicate that there were no changes to any persistent allocation in frame K – allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1. The allocation period is indicated in the subburst IE.

The change indicator can be set to 0 or 1.  It is used to guide the MSs that have DL MAP decoding error to resume their DL persistent operations.  See section 6.3.27.5.1 for details.

8.4.5.4.30 Persistent HARQ UL MAP Allocation IE

[Modify Table 438 as shown.  Insertion is in blue, deletion is in red.]

- Table 438 - Persistent HARQ UL MAP IE

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| Persistent HARQ_UL-MAP_IE() { |  |  |
| Extended 2- UIUC | 4 | Persistent HARQ_UL-MAP_IE() = 0x0B |
| Length | 8 | Length of the IE |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| Change Indicator | 1 | 0: No Change Occurred 1: Change Occurred Can be set to 0 or 1. See section 6.3.27.3.2. |
| Reserved | 1 |  |
| While (data remains) { | - | - |
| Mode | 3 | Indicates the mode of this IE: 0b000: Persistent Chase HARQ 0b001: Persistent Incremental redundancy HARQ for CTC 0b010: Persistent Incremental redundancy HARQ for convolutional code 0b011: Persistent MIMO Chase HARQ 0b100: Persistent MIMO IR HARQ 0b101: Persistent MIMO IR HARQ for convolutional code 0b110: Persistent MIMO STC HARQ 0b111: Reserved |
| Allocation Start Indication | 1 | 0: No allocation start information 1: Allocation start information follows |
| If (Allocation Start Indication ==1) { | - | - |
| OFDMA Symbol offset | 8 | This value indicates start symbol offset of subsequent subbursts in this Persistent HARQ UL MAP IE with reference to the start of the UL sub-frame. |
| Subchannel offset | 7 | This value indicates start Subchannel offset of subsequent subbursts in this Persistent HARQ UL MAP IE |
| Reserved | 1 | Shall be set to zero |
| } | - | - |
| N sub-bursts | 4 | Number of sub-bursts in this Persistent HARQ UL MAP IE is this field value plus 1. |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (i=0;i<Number of sub-burst; i++) { |  |  |
| If (mode == 000) { |  |  |
| Persistent UL HARQ Chase Sub-Burst IE () |  |  |
| } else if (mode == 001) { |  |  |
| Persistent UL HARQ IR CTC Sub-Burst IE () |  |  |
| } else if (mode == 010) { |  |  |
| Persistent UL HARQ IR CC Sub-Burst IE () |  |  |
| } else if (mode == 011) { |  |  |
| Persistent MIMO UL Chase HARQ Sub- Burst IE () |  |  |
| } else if (mode == 100) { |  |  |
| Persistent MIMO UL IR HARQ Sub-Burst IE () |  |  |
| } else if (mode == 101) { |  |  |
| Persistent MIMO UL IR HARQ for CC Sub-Burst IE () |  |  |
| } else if (mode == 110) { |  |  |
| Persistent MIMO UL STC HARQ Sub-Burst IE () |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to byte; shall be set to 0 |
| } |  |  |

- Resource Shifting Indicator

If the resource shifting indicator is set to 1, the MS shall shift its persistent resource position by the accumulated slots as indicated by de-allocation commands with slot offsets smaller than its own.

Change Indicator

The change indicator shall be set to 0 to indicate that there were no changes to persistent allocations in frame relevant to frame K – allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.and allocation period is indicated in the subburst IE.

The change indicator can be set to 0 or 1.  It is used to guide the MSs that have UL MAP decoding error to resume their UL persistent operations.  See section 6.3.27.5.2 for details.