| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Persistent Scheduling – Change Indicator |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Hua Xu, John Harris, Steven Xu Motorola Sean McBeath Huawei Changyoon Oh, Samsung Electronics Vladimir Yanover, Alvarion Yair Bourlas, Nextwave | Voice: E-mail: hua.xu@motorola.com smcbeath@huawei.com changyoon.oh@samsung.com vladimir.yanover@alvarion.com YBourlas@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Ballot 26c |  |
| Abstract | The resource allocation scheme of 802.16e is extended to enable persistent allocations (also referred to as periodic allocations). |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Persistent Scheduling – Change Indicator

Hua Xu, John Harris, Steven Xu,   Motorola

Sean McBeath,   Huawei Technology

Changyoon Oh, Samsung Electronics

Vladimir Yanover, Alvarion

Yair Bourlas, Nextwave

# Introduction

Modify the description of the Change Indicator to cover more scenarios.

# Proposed Text

[The contribution shows the modifications to the accepted contribution C802.16maint-08/182r8.

The color codes used in the accepted contribution C802.16maint-08/182r8 are as follows.

[note to the editor: Make the following changes to IEEE P802.16Rev2/D4 as shown below;

Blue = editorial insertions relative to R2D4

Red = editorial deletions relative to R2D4

Purple = insertions to support this proposal

Green = deletions to support this proposal

Pink = insertion to apply vertical optimization accepted for chase combining mode to other HARQ modes.

[] = notes to editor

]

We use

Brown = insertions relative to contribution C802.16maint-08/182r8

Grey = deletions relative to contribution C802.16maint-08/182r8

]

6.3.27.51.5   Change Indicator

[Replace the entire section 6.3.27.1.5 with the text in Brown.]

A Change Indicator for each HARQ Region is included in the Persistent HARQ DL MAP IE.  The change indicator shall be set to 0 to indicate that there were no changes to the any DL persistent allocations, including HARQ region changes, in comparatively to frame K - allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.

A Change Indicator for each HARQ Region is included in the Persistent HARQ UL MAP IE.  The change indicator shall be set to 0 to indicate that there were no changes to the any UL persistent allocations, including HARQ region changes, in comparatively to frame K - allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1.

Change indicator in frame K is used for by those MSs who which failed to receive the DL-MAP or UL-MAP in frame K – allocation period to recover the configuration of their persistent allocation., For example, i.e., when an MS, who failed to receive the DL-MAP or UL-MAP in frame K – allocation period, should shall continue the persistent allocation, when the change indicator is set to 0 in frame K.

A Change Indicator for each HARQ Region shall be included in the Persistent HARQ DL MAP IE.
A Change Indicator for each HARQ Region shall be included in the Persistent HARQ UL MAP IE.

Change indicator in frame K shall be used by the MSs who failed to receive the DL-MAP or UL-MAP in frame K-ap to decide if they can resume using their persistent allocation in frame K.

MS who failed to receive the DL-MAP or UL-MAP in frame K-ap, shall interpret the change indicator= '0' as there were no changes, such as deallocation, Persistent HARQ Region change in the Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE  in K-ap.

MS who failed to receive the DL-MAP or UL-MAP in frame K-ap,  shall interpret the change indicator='1' as there were change(s) in the Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE  in K-ap.

The base station sets the change indicator to 0 in frame K in case there are sufficient evidences that all the MSs that involved in the changes in frame K-ap have received the change information.

8.4.5.3.29 Persistent HARQ DL MAP Allocation IE

[Modify Table 362 as shown Brown and Grey.]

Table 362: Persistent HARQ DL MAP allocation IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent HARQ DL MAP IE () { | -- | -- |
| Extended-2 DIUC | 4 | Persistent HARQ DL MAP IE = 0D |
| Length | 48 | Length in bytes |
| RCID_Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this Persistent HARQ DL MAP (FDD/H-FDD Only) |
| Change Indicator | 1 | 0: No Change Occurred 1: Change Occurred |
| while (data remainsing) { | - | - |
| Region_ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| Persistent Region ID | 5 |  |
| Change Indicator | 1 | 0: No Change Occurred 1: Change Occurred |
| if (Region ID use indicator ==0) { |  |  |
| OFDMA Symbol offset | 8 | Offset from the start of DL sub-frame |
| Subchannel offset | 7 |  |
| Number of OFDMA symbols | 7 |  |
| Number of subchannels | 7 |  |
| Rectangular sub-burst indication | 1 | Indicates sub-burst allocations are time-first rectangular. The duration field in each sub-burst IE specifies the number of subchannels for each rectangular allocation. The slot offset field in each sub-burst IE specifies the subchannel offset from the first subchannel for each rectangular allocation. This is only valid for AMC allocations and all allocations with dedicated pilots. When this field is clear, sub-bursts shall be allocated in frequency-first manner and the duration field reverts to the default operation |
| } | -- | -- |
| else { |  |  |
| Region_ID | 8 | Index to the DL region defined in DL region definition TLV in DCD |
| } | -- | -- |
| Power boost per sub-burst | 1 | Set to 1 to signal power boost per sub-burst. This field shall be set to 0 if Rectangular sub-burst indication is set to 0. |
| if (Power boost per sub-burst ==0) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each the first allocation instance of the persistent allocation only; |
| } |  |  |
| Mode | 4 | Indicates the mode in this HARQ region 0b0000: Persistent DL Chase HARQ 0b0001: Persistent DL Incremental redundancy HARQ for CTC 0b0010: Persistent DL Incremental redundancy HARQ for Convolutional Code 0b0011: Persistent MIMO DL Chase HARQ 0b0100: Persistent MIMO DL IR HARQ 0b0101: Persistent MIMO DL IR HARQ for Convolutional Code 0b0110: Persistent DL MIMO STC HARQ 0b0111 to 0b1111 : reserved |
| Subburst IE Length | 8 | Length, in nibbles, to indicate the size of the subburst IE in this HARQ mode. The MS may skip DL HARQ Subburst IE if it does not support the HARQ mode. However, the MS shall decode NACK Channel field from each DL HARQ Subburst IE to determine the UL ACK channel it shall use for its DL HARQ burst. |
| if (Mode == 0b0000) { | -- | -- |
| Persistent DL Chase HARQ sub-burst IE | variable | -- |
| } else if (Mode == 0b0001) { |  |  |
| Persistent DL Incremental redundancy HARQ for CTC sub-burst IE | variable | -- |
| } else if (Mode == 0b0010) { | -- | -- |
| Persistent DL Incremental redundancy HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0011) { | -- | -- |
| Persistent MIMO DL Chase HARQ | variable | -- |
| } else if (Mode == 0b0100) { |  |  |
| Persistent MIMO DL IR HARQ | variable | -- |
| } else if (Mode == 0b0101) { | -- | -- |
| Persistent MIMO DL IR HARQ for Convolutional Code | variable | -- |
| } else if (Mode == 0b0110) { |  |  |
| Persistent MIMO DL STC HARQ | variable |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. Padding to byte for the unspecified portion of this IE, i.e., not including the first two fields, “Extended-2 DIUC” and “Length”; shall be set to 0 |
| } |  |  |

Persistent Region ID

The identifier of specific Persistent HARQ region. The operation commanded by the IE is applied to sub-bursts in the region.

Change Indicator

The change indicator shall be set to 0 to indicate that there were no changes to any persistent allocation in frame K – allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1. The allocation period is indicated in the subburst IE

The change indicator can be set to 0 or 1.  It is used by MSs to decide if they can resume using their DL persistent allocations.  See section 6.3.27.1.5 for details.

8.4.5.4.30 Persistent HARQ UL MAP Allocation IE

[Modify Table 438 as shown in Brown and Grey.]

- Table 438 - Persistent HARQ UL MAP IE

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| Persistent HARQ_UL-MAP_IE() { |  |  |
| Extended 2- UIUC | 4 | Persistent HARQ_UL-MAP_IE() = 0x0B |
| Length | 8 | Length of the IE |
| RCID_Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| Change Indicator | 1 | 0: No Change occurred 1: Change occurred |
| Reserved | 1 |  |
| while (data remains) { | - | - |
| Mode | 3 | Indicates the mode of this IE: 0b000: Persistent UL Chase HARQ 0b001: Persistent UL Incremental redundancy HARQ for CTC 0b010: Persistent UL Incremental redundancy HARQ for convolutional code 0b011: Persistent MIMO UL Chase HARQ 0b100: Persistent MIMO UL IR HARQ 0b101: Persistent MIMO UL IR HARQ for convolutional code 0b110: Persistent MIMO UL STC HARQ 0b111: Reserved |
| Allocation Start Indication | 1 | 0: No allocation start information 1: Allocation start information follows |
| If (Allocation Start Indication ==1) { | - | - |
| OFDMA Symbol offset | 8 | This value indicates start symbol offset of subsequent subbursts in this Persistent HARQ UL MAP IE with reference to the start of the UL sub-frame. |
| Subchannel offset | 7 | This value indicates start Subchannel offset of subsequent subbursts in this Persistent HARQ UL MAP IE |
| Reserved | 1 | Shall be set to zero |
| } | - | - |
| N sub-bursts | 4 | Number of sub-bursts in this Persistent HARQ UL MAP IE is this field value plus 1. |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| Persistent Region ID | 5 |  |
| Change Indicator | 1 | 0: No Change Occurred 1: Change Occurred |
| For (i=0;i<Number of sub-burst; i++) { |  |  |
| If (mode == 000) { |  |  |
| Persistent UL HARQ Chase Sub-Burst IE () |  |  |
| } else if (mode == 001) { |  |  |
| Persistent UL HARQ IR CTC Sub-Burst IE () |  |  |
| } else if (mode == 010) { |  |  |
| Persistent UL HARQ IR CC Sub-Burst IE () |  |  |
| } else if (mode == 011) { |  |  |
| Persistent MIMO UL Chase HARQ Sub-Burst IE () |  |  |
| } else if (mode == 100) { |  |  |
| Persistent MIMO UL IR HARQ Sub-Burst IE () |  |  |
| } else if (mode == 101) { |  |  |
| Persistent MIMO UL IR HARQ for CC Sub-Burst IE () |  |  |
| } else if (mode == 110) { |  |  |
| Persistent MIMO UL STC HARQ Sub-Burst IE () |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | To align octet boundary |
| } |  |  |

- Resource Shifting Indicator

If the resource shifting indicator is set to 1, the MS shall shift its persistent resource position by the accumulated slots as indicated by de-allocation commands with slot offsets smaller than its own.

Persistent Region ID

The identifier of specific Persistent HARQ region. The operation commanded by the IE is applied to sub-bursts in the region.

Change Indicator

The change indicator shall be set to 0 to indicate that there were no changes to persistent allocations in the frame relevant to frame K – allocation period, where K is the frame that the change indicator is transmitted. Otherwise, the change indicator shall be set to 1 and The allocation period is indicated in the subburst IE.

The change indicator can be set to 0 or 1.  It is used by MSs to decide if they can resume using their UL persistent allocations.  See section 6.3.27.1.5 for details.