| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Addition of New CQICH Types |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Ron Porat, Wee Peng Goh Nextwave Frank Zhou, Louay Jalloul Beceem | E-mail: rporat,wgoh@nextwave.com fzhou, jalloul@beceem.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Working Group Letter Ballot #26c |  |
| Abstract | This document describes three more CQICH types to be added to the CQICH_Enhanced_Alloc_IE. By extending the CQICH capabilities we allow for an elegant and simple improvement of the UL feedback channel that can in turn be used for other purposes. |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Addition of New CQICH Types

Ron Porat, Wee Peng Goh - Nextwave

# Purpose

This document describes a more efficient CQICH feedback mechanism uniquely suited for feeding back precoding matrix index, rank and CINR of multiple bands to support CL-MIMO in Band AMC. This mechanism uses the CQICH_Enhanced_Alloc_IE by reclaiming several reserved bits.

# Problem Statement:

- The current CQICH mechanism enables very robust feedback of 6 bits per slot.

- For CL-MIMO, the amount of feedback required is higher and there is a need to provide a flexible mechanism that trades robustness with throughput. Users with low UL CINR can feed back less bits and users with higher UL CINR can feed back more bits

- Using UL messages (like feedback header) is inefficient as it requires more than repetition 6 to get to the same level of reliability but will then occupy 6 times more slots

- By extending the CQICH capabilities we allow for an elegant and simple improvement of the UL feedback channel that can in turn be used for other purposes.

# Proposed solution

The underlying idea is to map 6 bits into 3 or 2 tiles (half or one third of a slot) instead of 6. This will double or triple the payload of the CQICH mechanism at the expense of reduced reliability.

The mapping uses the current mapping as in table 389 with small modification for 3 new CQICH types:

- 6 bit CQI (even) – here we map the 6 bits into only the even tiles (half slot) using a pruned mapping from table 389. For example, the mapping of 001000 is 2,4,3,6,7,5 for the current 6 bit CQI and will be 2,3,7 for this new feedback type. (Tiles are numbered 0-5).

- 6 bit CQI (odd) – here we map the 6 bits into only the odd tiles (half slot) using a pruned mapping from table 389. For example, the mapping of 001000 will be 4,6,5 for this new feedback type

- 18 bit CQI – here we use one full slot to transmit 18 bits by mapping each 6 bits to two tiles

  - The first group of 6 bits will be mapped to tiles 0,3 using the pruned 2,6

  - The second group of 6 bits will be mapped to tiles 1,4 using the pruned 4,7

  - The third group of 6 bits will be mapped to tiles 2,5 using the pruned 3,5

Another advantage of mapping to half slot is increased granularity – users can occupy only half a slot or multiples of half slot and allow better utilization of the fast feedback channel by more users.

# Simulation Results:

The following simulation results show the performance of the two new options compared with the current 6bit CQI type.

For comparison, using feedback header message under the same configuration and QPSK rate ½ requires SNR=4dB for payload error rate = 10-2. Note that here we assume that the MS doesn’t transmit anything but the FH in the same OFDM symbol.

Also, it can be noticed that the loss going from 6 bit in full slot to 6 bit in half slot is 3dB which can be gained back if a MS occupies only one half slot due to improved power concentration.

Similar results for a 2-antenna BS are shown as well where for comparison, using feedback header message under the same configuration requires SNR=11dB for payload error rate = 10-2.

# Usage

For CL-MIMO purposes we use the same mapping to 6 bits whether they occupy full slot, half slot or one third of a slot.

If the feedback type is 0b100 (index to precoding matrix in codebook) we map into each group of 6 bits:

- 4 antenna BS – 4 bit PMI as MSB plus 2 bit differential CINR as LSB

- 2 antenna BS – 3 bit PMI as MSB plus 1 bit RI  plus 2 bit differential CINR as LSB

For 2 antenna BS the lowest eigenvalue fluctuates much more than for 4 antenna BS and feeding RI more frequently is more important.

For description of codebooks see contribution 93r3 or a later revision.

PMI – precoding matrix index

RI – rank information

MSB – Most Significant Bits

LSB – Least Significant Bits

# Text Changes

Change CQICH Type in Table 418 CQICH Enhanced Allocation IE to:

Table 418—CQICH Enhanced Allocation IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| CQICH_Enhanced _ALLOC_IE | -- | -- |
| … | … | … |
| CQICH Type | 3 | 0b000: 6-bit CQI Reserved 0b001 0b001: 18-bit CQI 0b010: 3-bit CQI (even) 0b011: 3-bit CQI(odd) 0b100: 6-bit CQI (primary) 0b101: 4-bit CQI (secondary) Reserved b0110-0b111 0b110: 6-bit CQI (even) 0b111: 6-bit CQI(odd) |
| … | … | … |

Add to section 8.4.5.4.10.5 page 846 line 12:

Table 391 defines the mapping between the payload bit sequences and the subcarriers modulation for 6 bit CQI type 0b000 and 0b100.  For CQICH types 0b110, 0b111, and 0b001 the 6 bits CQI shall follow the mapping as described below:

Referring to table 389 – Enhanced fast-feedback channel subcarrier modulation, the mapping of Fast-feedback vector indices per Tile shall be CQICH type dependant as shown below:

- CQICH type 0b110 (even):  Tile(0), Tile(2), Tile(4)

- CQICH type 0b111 (odd):  Tile(1), Tile(3), Tile(5)

- CQICH type 0b001 (18 bit mapping in a full slot):

  - Bit17 – Bit12– Tile (0), tile (3)

  - Bit11 – Bit6 – Tile (1), tile (4)

  - Bit5 – Bit0– Tile (2), tile (5)

Add at the end of section 8.4.5.4.16

Feedback Type

For CQICH type = 0b000, 0b001 or 0b100:

0b000 = Fast DL measurement/Default Feedback with antenna grouping

0b001 = Fast DL measurement/Default Feedback with antenna selection

0b010 = Fast DL measurement/Default Feedback with reduced codebook When the MS transmits the feedback of S/N using 5 ……

For Feedback type = 0b100 (index to precoding matrix in codebook) :

Mapping into each group of 6 bits (CQICH types 0b000, 0b100, 0b110 or 0b111):

- 4 antenna BS – 4 bit PMI mapped to MSB plus 2 bit differential CINR as LSB

- 2 antenna BS – 3 bit PMI mapped to MSB plus 1bit RI  plus 2 bit differential CINR as LSB

Mapping of the 2 bit differential CINR is as follows:

| 00 | -3dB |
| --- | --- |
| 01 | -1dB |
| 10 | 1dB |
| 11 | 3dB |

PMI – precoding matrix index

RI – rank information

MSB – most significant bits

LSB – least significant bits

Replace subsection c) in section 8.4.8.3.6 MIMO precoding with

c) Feeding back multiple precoders for band

AMC operation. For band AMC the BS has the choice to can request a common precoding matrix for all bands or can request a programmable number, N (see Table 386 416) of precoding matrices to be fed back for the N best bands. selected in an ordered fashion. In the latter case, the precoding matrices are associated with the bands with the highest S/N values. As a secondary selection criteria, in case the ordering according to highest S/N is not unique, the bands with the lowest band index are chosen first.  fed back from the lowest AMC band index to the highest index. The index for each precoder is mapped to a CQICH channel of the corresponding size. The precoders for the different bands, in the order described previously, is signaled in the corresponding CQICH channels . See Table 494 and Table 495. For the case where the dedicated pilot bit is set to 1 in the STC Zone IE (8.4.5.3.4) for the zone in which the subburst allocations are made, if the BS requests a common precoding matrix for all bands, a single precoding matrix shall be fed back based on the allocated bands  mid-amble. If the BS requests a programmable number N (see Tables 386  416) of precoding matrices to be fed back for the N bands, then the precoding matrices are associated with the bands following the order in which the

bands were allocated. The precoders for the different bands, in the order described previously, are signaled in the corresponding CQICH channels based on the allocated CQICH types and CQICH_Num  when feedback type is set to 0b100 (8.4.5.4.16).

- If CQICH type = 0b100 and CQICH type = 0b101 (6bit primary / 4 bit secondary),

  - 6 bits carry best common precoder for the 3 selected bands and differential CINR for lowest numbered band

  - 4 bits carry differential CINR for second and third bands.

- If one or multiple CQICH Type = 0b000, 0b110 or 0b111, (6 bit in full or half slots)

  - If N=3, the MS feeds back for all 3 bands in the band bitmap in the order of lowest numbered band to highest

  - If N<3, the MS determines the set S of bands allocated to him that are also part of his last recommended band bitmap. M denotes the number of bands in S

    - If M>=N the MS first feeds back for the best N bands as determined by the last absolute CINR report.

    - If M<N the MS first feeds back for those M bands and then for the remaining (N-M) best bands

- If CQICH Type  = 0b001 (18 bit CQI), the MS feeds back the PMI and differential CINR in the following order:  If 3 bands are feedback(Nr_Precoders_feedback=3), the lowest AMC band is mapped to the first group of 6 bits, the second AMC band is mapped to the second group of 6 bits and the third AMC band is mapped to the third group of 6 bits. If 2 bands are feedback Nr_Precoders_feedback=2), the first group of 6 bits carries the lowest AMC band , the second group of 6 bits carries the second AMC band, and the third group of 6 bits carries the CINR of the 2 AMC band.