| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | TEK generation and update for Handover |  |
| Date Submitted | 2008-04-20 |  |
| Source(s) | Kyeong-Tae Do Eun-Sun Jung Samsung Electronics Co., LTD | Voice: +82-31-279-5748 E-mail: kyeongtae.do@samsung.com esleon.jung@samsung.com |
| Re: | LB26c |  |
| Abstract | The effort to reduce the HO latency is currently being discussed. Main idea is to exchange in the serving BS the information which is required at the target BS. This contribution suggests the way of sharing TEKs. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

TEK generation and update for Handover

Samsung Electronics.

# Introduction – Problem Description

The method to reduce the HO latency is being discussed currently. The MS needs to receive the necessary information to communicate with the target BS after HO. Normally the MS receives it from the RNG-RSP message sent from the target BS as a response to a RNG-REQ message.

One of the requirements to communicate with the target BS is sharing TEKs when the data is encrypted and transmitted. Currently, the TEKs are updated after the MS receives a RNG-RSP message which includes SA-TEK-Update TLV. If the MS is required to receive data without regard to the receipt of the RNG-RSP, the TEKs shall be delivered to the MS before the traffic data and RNG-RSP messages are transmitted. If the TEKs used at the serving BS can continuously be used at the target BS, the TEKs need not to be sent to the MS. But, this method increases the possibility to compromise the connection between the BS and the MS and requires the transmission of the PN windows for AES-CCM encryption. Thus, it is needed to share TEKs, prior to the transmission of data, which will expedite data transmission after HO.

# Proposed Solution