| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for emergency service in IEEE802.16REV2 |  |
| Date Submitted | 2008-04-20 |  |
| Source(s) | Yeongmoon Son, Geunhwi Lim, Brian Shim Samsung Electronics | Voice: +82-31-279-5845 E-mail: ym1004.son @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26c |  |
| Abstract | This contribution defines operation required for emergency service. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for emergency service in IEEE802.16REV2

Yeongmoon Son, Geunhwi Lim, Brian Shim

Samsung Electronics

# Problem description

It is defined in IEEE802.16REV2/D4 that a BS can inform MSs when to broadcast ‘Emergency Service Message’ (i.e. ESM) by using Broadcast Control Pointer IE as shown in the table 358.

Nevertheless, there is no any other description except the above definition regarding Emergency Service at all. Therefore, we have to solve at least the problems, as follows.

- Assignment of CID to Emergency Service:

Unfortunately, the current standard doesn’t define CID used for Emergency Service. It brings ambiguity to operation of Emergency Service. We should assign a CID to Emergency Service so that all the MSs can identify MAC PDU including ESM.

- Broadcast Control Pointer IE(i.e. BCPIE):

All MSs should receive ESM regardless of their status (i.e. Idle Mode, Sleep Mode, or Awake State). The BCPIE is very useful for the BS to indicate the frame in which ESM will be transmitted. However, Broadcast_Sytem_Update_Transmission_Frame in BCPIE is too short (i.e. at most up to 127 frames) while Paging Cycle in Idle Mode has up to 65535 frames and Sleep Window in Sleep Mode has up to 1024 frames. Eventually, it may require the MSs in Sleep Mode or Idle to Mode to unnecessarily wake up several times in order to receive ESM (e.g. 8 times for sleep mode and 509 times for idle mode in worst case). Therefore, we need to enlarge the range of Broadcast_Sytem_Update_Transmission_Frame.

When the MS wakes up in the frame specified by BCPIE, BS may transmit multiple ESMs through several frames. But, the MS doesn’t know how many ESMs the BS transmit. As a result, when the MS receives an ESM in the frame specified by BCPIE, the MS may enter Idle Mode or Sleep Mode even though BS still has pending ESMs to transmit in next frame. Therefore, the BS needs to inform the MS of the number of ESMs which will be transmitted from the frame indicated by BCPIE.

# Proposed Changes

# References

[IEEE802.16-Rev2/D4]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D4 (April 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.