| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of retransmission of DSD-RSP based on T10 timer in response to DSD-REQ |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Yeongmoon Son, Geunhwi Lim, Brian Shim Samsung Electronics | Voice: +82-31-279-5845 E-mail: ym1004.son @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26c |  |
| Abstract | This contribution clarifies DSD operation based on T10 timer. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of retransmission of DSD-RSP based on T10 timer in response to DSD-REQ

Yeongmoon Son, Geunhwi Lim, Brian Shim

Samsung Electronics

# Problem description

As you can know in the figure 141 on page 408, if an MS or a BS receives a DSD-REQ message from a counter entity, it shall perform the following operations in sequence

- It shall respond DSD-RSP message to the remote side.

- It shall start T10 timer.

- It shall save DSD-RSP message to prepare retransmission in future.

- Afterward, it transits to ‘DSD-Remote Holding Down’ state.

If we see the figure 142 on page 409, when the MS or the BS internally finishes deleting the service flow, it stops T10 timer and enters ‘DSD-Remote End’ state. It means that the MS or the BS terminates whole stuff related to the service flow and it doesn’t take care of the deleted service flow any more. Nevertheless, the figure 142 shows when the MS or the BS receives the DSD-REQ message again due to no reception of DSD-RSP message in the remote side, it still sends DSD-RSP message by referring to the ‘ghost’ service flow. Therefore, we need to clarify retransmission of DSD-RSP based on T10 timer in response to a retransmitted DSD-REQ message.

T10 timer is time required for a whole DSx transaction as shown in the following table. Therefore, information of the service flow should be kept during T10 timer for DSx transaction in order to prepare the retransmission of DSx-RSP message in future.

Even though a service flow is deleted as shown in the figure 141, an MS or a BS should keep at least ‘saved DSD-RSP’ in order for retransmission in future until T10 timer expires. In other words, the MS or the BS can send the saved DSD-RSP message in response to the retransmitted DSD-REQ message only while it has valid information of the service flow.

# Proposed Changes

[Modify the figure 142 on page 409, as follows]

# References

[IEEE802.16-Rev2/D4]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D4 (April 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.