| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of load balancing to enhance service availability (using Preamble Index Override & Ranging Abort Timer) |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Jaejeong(Brian) Shim, KiBack Kim, Aeri Lim Samsung Electronics* Vikas Mehrotra, Apurv Mathur, Simon Jing, Rishi Arora, Joseph Schumacher Motorola* | Voice: E-mail: brian.shim@samsung.com Vikas.Mehrotra@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26c |  |
| Abstract | This contribution proposed a load balancing scheme during initial network entry, HO, or re-entry from idle mode. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support of load balancing to enhance service availability (using Preamble Index Override & Ranging Abort Timer)

Jaejeong Shim, KiBack Kim, and Aeri Lim

Samsung Electronics

Vikas Mehrotra, Apurv Mathur, Simon Jing, Rishi Arora, and Joseph Schumacher

Motorola Inc.

# Problem description

# When a BS cannot support an MS trying initial network entry, handover, or reentry from idle mode, it has to move the MS to other BS which can support the MS. However, current specification does not support the operation.

# BS can use RNG-RSP (with ranging status=abort) or DREG-CMD (with action code = 00), but they cannot designate specific target BS to the MS.

# BS can use the ‘Downlink frequency override’ TLV. But the TLV cannot designate a specific target BS and it is not available when reuse factor = 1.

# Even though MS can obtain the Service level prediction of each target BS during handover, the system load of target BS may change when the MS moves to the BS.

# MS could get the loading information (Available DL/UL Radio Resources) in DCD/UCD during network entry. But the information is only for the serving BS. Generally, the TX period of NBR-ADV message is much larger than that of DCD/UCD. Thus, MS may not get the NBR-ADV message during network entry. MS cannot perform load balancing based on the loading information of only the serving BS. In case of re-entry from idle-mode, MS may not obtain latest DCD, UCD, and NBR-ADV. The Configuration Change Count of DCD and UCD does not count in the Available DL Radio Resource and Available UL Radio Resource respectively. That means MS mostly have loading information which is not valid any more during re-entry from idle mode.

# Draft specification [IEEE802.16-Rev2/D4] allows the MS to scan and to initiate at any time handover to target cells different from the ones indicated by the serving BS. Moreover MS can execute handover without even notifying the target BSID to the BS. Such behavior may lead to conflicting decisions between MS and the BS. MS initiated HO primarily aims at individual MS link optimization by taking into account channel quality situation. BS initiated Load Balancing may in general pursue overall network resources optimization by taking into account Serving BS and network load conditions.

# Proposed Changes

Change #1: Add the following TLVs and its description in “RNG-RSP message”:

Adopt the following changes in section 6.3.2.3.6 RNG-RSP (ranging response) message:

The following parameters may be included in the RNG-RSP message:

Timing Adjust Information

If this field is not included, no adjustment shall be made

Power Adjust Information

If this field is not included, no adjustment shall be made

Downlink Frequency Override

Uplink Channel ID Override

Downlink Operational Burst Profile

Basic CID

A required parameter if the RNG-RSP message is being sent on the Initial Ranging CID in response to a RNG-REQ message that was sent on the Initial Ranging CID.

SS MAC Address (48-bit)

A required parameter when the CID in the MAC header is the Initial Ranging CID.

Frequency Adjust Information

AAS Broadcast Permission

Preamble Index Override

Preamble Indices of new target BS(s) where the MS should redo ranging. If the TLV includes two or more Preamble Indices, the first one in the list is the most preferable and the second is the next preferable. When the TLV is used with Downlink frequency override TLV, the MS should redo ranging on the new DL channel identified by the Preamble Indices.

Ranging Abort Timer

Timer defined by a BS to prohibit the MS from attempting network entry at this BS, for a specific time duration.

At the end of Section 6.3.9.5.1, page 316, line 33 insert as:

If the Ranging Abort Timer TLV encoding is included in RNG-RSP, the MS shall abort the current network entry attempt and shall not redo ranging to the current BS until the Ranging Abort Timer expires.

Change #2: Add the following TLVs in Table 572 - RNG-RSP message encodings:

Add the following TLVs into Table 572:

Table 572—RNG-RSP message encodings

| Name | Type (1 byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| Preamble Index Override | 39 | Length is defined as: (Num of Preamble Index) * 1 | Preamble Indices of new target BS(s) where the MS should redo ranging. If this TLV is used, the Ranging Status value shall be set to 2. This TLV shall be used for licensed bands only. | All |
| Ranging Abort Timer | 40 | 1 | 0-255: In units of seconds | All |

Change #3: Rejection of service flow creation

6.3.14.7.1.1 Dynamic service flow creation—SS-initiated

Creation of service flows may be initiated by either BS (mandatory capability) or by SS (optional capability). The SS-initiated protocol is illustrated in Figure 111 and described in detail in 6.3.14.9.3.1. A DSA-REQ from an SS contains a QoS parameter set (marked either for admission-only or for admission and activation). BS responds with DSA-RSP indicating acceptance or rejection. In the case when rejection was caused by presence of nonsupported parameter of nonsupported value, specific parameter may be included into DSA-RSP. If the BS rejects a service flow due to absence of sufficient resources to admit the service flow (see Table 574, CC = 3), the BS may initiate procedures to move the SS to a different BS (see 6.3.22).

# References

[IEEE802.16-Rev2/D4]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D4 (April 2008) (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007)