| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improve Scanning HO Context management |  |
| Date Submitted | 2008-07-17 |  |
| Source(s) | Lucy Pollak Altair Semiconductor 6 Haharash St. Hod Hasharon 45240, Israel Navid Ehsan Nextwave Christopher Cushing Motorola | Voice: 972-9-7403045 E-mail: lucy.pollak@altair-semi.com nehsan@nextwave.com Christopher.Cushing@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d, on P802.16Rev2/D5, as per IEEE 802.16-08/029 |  |
| Abstract | The scanning context and scanning operations during HO are not clearly defined in the spec. The proposed clarification defines the scanning termination details for HO operations. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Improve Scanning HO Context management

Lucy Pollak, Altair Semiconductor

Navid Ehsan, Nextwave

Christopher Cushing, Motorola

# Background

# The scanning context and operation during HO are not clearly defined in the spec. There is only one reminding that scanning should be interrupted by MS–initiated HO negotiation. When the BS-initiated HO will terminate scanning is not defined in the spec. The scanning termination by MOB_MSHO-REQ is used in the case of MS-initiated HO to speed up the HO negotiation and provide fast opportunity for BS to transmit MOB_BSHO-RSP. In the case of BS-initiated HO the BS will send its MOB_BSHO-REQ at interleaving interval, and scanning need to be terminated immediately to support fast HO negotiation and Action time scheduling of HO process.

# Proposed Changes

The proposed clarification defines the scanning termination details for HO operations.

# Proposed Changes in 802.16Rev2/D5

[On page 440, Section “6.3.22.1.2  MS scanning of neighbor BSs”, modify paragraph 12:]

An MS may terminate scanning and return to Normal Operation anytime that may be indicated to the BS by sending a MAC PDU (for example, Bandwidth request) during any scanning interval. If a serving BS receives a MAC PDU message during any scanning interval from an MS that is supposed to be in Scanning Mode, the BS shall assume that the MS is no longer in Scanning Mode. The group of intervals is terminated at any time if the MS sends MOB_SCN-REQ message with Scan Duration set to zero or MOB_MSHO-REQ or serving BS sends MOB_SCN-RSP message with Scan Duration set to zero during any interleaving interval. Upon reception of the MOB_SCN-REQ message with zero Scan Duration, the BS shall respond with a MOB_SCN-RSP message with Scan Duration set to zero and Report Mode set to 0b00 (No report). If HO negotiation is started at the time of ongoing scanning process the group of scanning intervals shall be terminated when the MS transmits MOB_MSHO-REQ for MS-initiated HO and when the BS transmits MOB_BSHO-REQ for BS-initiated HO. The MS may renegotiate scanning intervals with the Serving BS upon transmission of HO-IND with HO_IND_type = 0b01 or HO_IND_type = 0b10.

[On page 458, Section “6.3.22.2.8.1 Context management during optimized HO”, add the new subsection at the end of the section:]

6.3.22.2.8.1.7 Scanning – dynamic and static context

MS scanning context with Serving BS: Reset.

MS scanning context with Target BS: Reset.

MS shall terminate all scanning activity including the outstanding scanning negotiation and group of scanning and/or reporting intervals to perform HO.