| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover Rejection and Target BS indication from MS in MOB_HO-IND |  |
| Date Submitted | 2008-05-12 |  |
| Source(s) | Shahab Sayeedi Joe Schumacher Motorola | Voice: +1 (847) 632-3877 Email: shahab.sayeedi@motorola.com |
| Re: | 802.16 Rev2, LB26c |  |
| Abstract | Correction to MOB_HO-IND message. This message may include Target_BS_ID when HO_IND_type field is set to 0b00 or 0b10. This contribution corrects this oversight which was missed in the original contribution approved from D3. |  |
| Purpose | Review and approve. |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MOB_HO-IND Message Correction

Shahab Sayeedi, Joe Schumacher

Motorola

# Change request

6.3.2.3.50 MOB_HO-IND (HO indication) message

An MS shall transmit a MOB_HO-IND message for final indication that it is about to perform an HO. When the MS cancels or rejects the HO, the MS shall transmit a MOB_HO-IND message with appropriate HO_IND_type field. If the MS rejects the target BS(s) offered by the Serving BS for handover, the MS may include its preferred target BS(s) in the message sent to the serving BS. The message shall be transmitted on the Basic CID.

Table 152—MOB_HO-IND message format

Syntax Size Notes

MOB_HO-IND_Message_format() { — —

Management Message Type = 59 8 bits —

Reserved 6 bits Reserved; shall be set to zero

Mode 2 bits 0b00: HO

0b01: MDHO/FBSS: Anchor BS update

0b10: MDHO/FBSS: Diversity Set update

0b11: Reserved

if (Mode == 0b00) { — —

HO_IND_type 2 bits 0b00: serving BS release

0b01: HO cancel

0b10: HO reject

0b11: Reserved

Ranging_Params_valid_indication 2 bits 0b00: No indication. BS ignores this field

(Default)

0b01: MS ranging parameters for Target BS,

which is specified in this message are valid

0b10: MS has no valid ranging parameters for

Target BS, which is specified in this message

0b11: Reserved

Reserved 4 bits Shall be set to zero.

if (HO_IND_type == 0b00) { — —

Target_BS_ID 48 bits Applicable only when HO_IND_type is set to

0b00 or ob01.

} — —

} — —

if (Mode == 0b01) { — —

MDHOFBSS_IND_Type 2 0b00: Confirms anchor BS update

0b01: Anchor BS update cancel

0b10: Anchor BS update reject

0b11: Reserved

if (MDHOFBSS_IND_Type == 0b00) { — —

Anchor BSID 3 TEMP_BSID of the anchor BS

Action time 8 Action time when the MS shall update the anchor

BS by itself.

} — —

} — —

if (Mode == 0b10) { — —.

MDHOFBSS_IND_Type 2 0b00: Confirms diversity set update

0b01: Diversity set update cancel

0b10: Diversity set update reject

0b11: Reserved

if (MDHOFBSS_IND_Type == 0b00) { — —

Diversity Set Included Indicator 1 1: Final decision of diversity set members

included in the message

0: Diversity set members are as specified in

MOB_BSHO_RSP message. No diversity set

information included in this message.

if (Diversity Set Included Indicator

== 1) {

3 —

Anchor BSID 3 TEMP_BSID of the anchor BS.

N_BSs 3 Number of BS in the diversity set, excluding the

anchor BS.

for (j = 0; j < N_BSs; j++) { — —

Temp BSID 3 Diversity set member ID assigned.

} — —

} — —