| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Initiated Handover |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Shahab Sayeedi Joe Schumacher Motorola | Email : shahab.sayeedi@motorola.com j.schumacher@motorola.com |
| Re: | 802.16 Rev2 LB26c |  |
| Abstract | When an MS rejects the Target BS(s) offered by the Serving BS for the MS to handover to, the MS may optional include its preferred TBS for handover in the MOB_HO-IND message so that the SBS can notify the MS preferred TBS of a potential handover coming in from the mobile and maximize the chances of a controlled handover from the MS. This contributions addresses network initiated handovers. When a a SBS initiates a handover by sending a MOB_BSHO-REQ message without including recommended TBSs for the handover, the MS must respond to the SBS with a message which includes it’s preferred target BSs. Handover may then proceed for the MS to one of the recommended TBS in accordance with the existing handover protocol. |  |
| Purpose | Review and approve. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Initiated Handover

Shahab Sayeedi, Joe Schumacher

Motorola

# Introduction

An SBS may initiate a handover by sending a MOB_BSHO-REQ message to an MS. This may occur for load balancing, maintenance, or other reasons. SBS may not include any recommended TBS when it initiated the handover.

# Summary of Changes

When a SBS initiates a handover by sending MOB_BSHO-REQ message without including any recommended target BSs, MS responds to the message with either a MOB_HO-IND message with HO_IND-type set to 0b10 and its preferred target BS included in the message, a MOB_MSHO-REQ message with its preferred target BSs included in the message, or MOB_HO-IND message the target BS it has selected to handover to to notify the SBS..

# Change request

6.3.22.2.2 HO decision and initiation

A handover begins with a decision for an MS to handover from a serving BS to a target BS. The decision

may originate either at the MS, the serving BS, or on the network. The HO may proceed with a notification

through either MOB_MSHO-REQ or MOB_BSHO-REQ messages. The HO notification is recommended,

but not required. Acknowledgement of MOB_MSHO-REQ with MOB_BSHO-RSP is required. After MS

transmits MOB_MSHO-REQ, MS shall not transmit any MOB_MSHO-REQ prior to expiration of timer

MS_handover_retransmission_timer. MS shall deactivate timer MS handover retransmission timer on MS

transmit of MOB_HO-IND or MS receipt of MOB_BSHO-RSP.

If an MS that transmitted a MOB_MSHO-REQ message detects an incoming MOB_BSHO-REQ message,

it shall ignore that MOB_BSHO-REQ message. A BS that transmitted a MOB_BSHO-REQ message and

detects an incoming MOB_MSHO-REQ message from the same MS shall ignore its MOB_BSHO-REQ. A

BS that transmitted a MOB_BSHO-REQ message and detects an incoming MOB_HO-IND message from

the same MS shall ignore its own previous request.

If the Serving BS sends a MOB_BSHO-REQ message to MS without including any handover target BSIDs in the message, the MS may respond by sending a MOB_HO-IND message with HO_IND_type set to 0b10 and its preferred target BS included in the message, by sending MOB_MSHO-REQ message with its preferred target BS included in the message, or by sending MOB_HO-IND message with HO_IND_type  set to 0b00 and include the target BS it has selected to move to so that the serving BS can be notified before it leaves.

When MOB_MSHO-REQ is sent by an MS, the MS may indicate one or more possible target BS. When

MOB_BSHO-REQ is sent by a BS, the BS may indicate one or more possible target BSs. MS may evaluate

possible target BS(s) through previously performed scanning and Association activity.

Serving BS criteria for recommendation of target BS may include factors such as expected MS performance

at potential target BS and MS QoS requirements. Serving BS may obtain expected MS performance at

potential target BS through the exchange of backbone messages with that BS. Serving BS may negotiate

location of common time interval where dedicated initial ranging transmission opportunity for the MS will

be provided by all potential target BSs. This information may be included into MOB_BSHO-RSP message.

Dedicated allocation for transmission of RNG-REQ means that channel parameters learned by the MS during

Association of that BS are considered valid during sufficient time and can be reused for actual Network

Re-entry without preceding CDMA Ranging. Information such as indicators of link quality in the UL direction

learned by the MS during Association may be provided to the Serving BS over the backbone.

If Network Assisted HO supported flag is set to “1” in MOB_BSHO-REQ message, MS may perform a handover

to any BS among the recommended BSs in MOB_BSHO-REQ without notifying the serving BS of a

selected target BS. As an acknowledgement to the MOB_BSHO-REQ message, the MS may send a

MOB_HO-IND message with its target BSID set to “0x00000000”.

When the serving BS, transmitted MOB_BSHO-REQ with Network Assisted HO supported flag = “1”,

receive MOB_HO-IND with target BS ID = “0x00000000”, it may neglect target BS ID included in

MOB_HO-IND message.

MS actual pursuit of handover to one of BSs specified in MOB_BSHO-RSP is recommended, but not

required. MS may decide to attempt handover to a different BS that may or may not have been included in

MOB_BSHO-RSP. If the MS signals rejection of serving BS instruction to HO through HO_IND_type field

in the MOB_HO-IND set value of 0b10 (HO reject option), the BS may reconfigure the neighbor BS list and

retransmit MOB_BSHO-RSP message including a new neighbor BS list.

In some instances, the BS may need to force the MS to conduct handover. The BS shall include a value of

HO operation mode = 1 in either the MOB_BSHO-REQ or MOB_BSHO-RSP to signal to the MS that the

MS must conduct handover. Upon receiving a message with HO operation mode = 1, the MS should treat the

handover request as required and shall respond with a HO-IND. MS should send HO-IND with option

HO_IND_type = 0b00 indicating commitment to HO unless MS is unable to handover to any of the recommended

BSs in the message, in which case MS may respond with HO-IND with option

HO_IND_type=0b10 indicating HO reject. An MS required to conduct handover is not restricted to conducting

handover to those BS included in the notifying message. In other words, the MS may attempt handover

to a different BS that may or may not have been included in either the MOB_BSHO-REQ or MOB_BSHO.