| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SBC-REQ contents for NSP discovery |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Chris Stanaway Ajay Idnani Chris Cushing Mark Marsan Joe Schumacher Motorola* | Voice: +1(847) 632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Revision 2 / Letter Ballot 26c |  |
| Abstract | The SBC-REQ message may be sent during NSP discovery. When it is sent during NSP discovery, it need not contain the same information that is included when the SBC-REQ is sent during network entry. |  |
| Purpose | Approve for inclusion in 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# SBC-REQ contents for NSP discovery

Chris Stanaway, Ajay Idnani, Chris Cushing, Mark Marsan, Joe Schumacher

Motorola

# Problem Statement

The SBC-REQ is used not only for network entry, but to also perform NSP discovery.  During NSP discovery, the only TLV required is the Service Information Query TLV.  Inclusion of other TLVs in the SBC-REQ requires those TLVs to also be included in the SBC-RSP.  Rev2/D4 does not allow fragmentation of the SBC-RSP. However, when the SBC-RSP is sent in support of NSP discovery, there may be a large amount of NSP information.  Eliminating the need to include TLVs in the SBC-RSP would provide additional room in the message to convey the requested NSP information.

# Proposed Text Changes

[Change subclause 6.3.2.3.23, page 130, lines 37-63 as indicated]

Basic Capability Requests contain the SS Capabilities Encodings (11.8) that are necessary to acquire NSP information and for effective communication with the SS during the remainder of the initialization protocols.

The following parameter may be included in the Basic Capabilities Request unless the Basic Capabilities Request is sent during any network entry or network re-entry:

Service Information Query (see 11.8.9)

The following parameter shall be included in the Basic Capabilities Request only if the Service Information Query TLV is omitted from the Basic Capabilities Request and shall otherwise be omitted:

Physical Parameters Supported (see 11.8.3)

The following parameters may be included only if the Service Information Query TLV is omitted from the Basic Capabilities Request and shall otherwise be omitted:

Capabilities for construction and transmission of MAC PDUs (see 11.8.2)

Security Negotiation Parameters (see 11.8.4)

Service Information Query (see 11.8.9)

Visited NSP ID (see 11.8.11)

Auth Type for EAP (see 11.8.12)

MIH Capability Supported (see 11.8.10)

Extended capability (see 11.8.15)

SDU MTU capability (see 11.8.16)

The Basic Capabilities Request shall include the following parameter encoded as a TLV tuple if authentication has been completed:

HMAC/CMAC Tuple (see 11.1.2)

Either the HMAC Tuple or the CMAC Tuple shall be the final attribute in the message’s TLV attribute list. This attribute should be included in the message during HO reentry.

For FDD systems, the following parameter shall be included in the Basic Capabilities Request only if the Service Information Query TLV is omitted from the Basic Capabilities Request and shall otherwise be omitted:

Bandwidth Allocation Support (see 11.8.1)

[Change subclause 6.3.2.3.24, page 130-131, lines 33-64, 1-10 as specified]

If the SIQ TLV (11.8.9) is not found in the SBC-REQ, then the The following parameters shall be included in the SBC-RSP if found in the SBC-REQ; otherwise the following parameters are not required to be included in the SBC-RSP:

Physical Parameters Supported (see 11.8.3)

Bandwidth Allocation Support (see 11.8.1)

The BS response to the subset of SS capabilities present in the SBC-REQ message. The BS responds to the SS capabilities to indicate whether they may be used. If the BS does not recognize an SS capability, it may return this as “off” in the SBC-RSP.

Only capabilities set to “on” in the SBC-REQ may be set “on” in the SBC-RSP, as this is the handshake indicating that they have been successfully negotiated.

Security Negotiation Parameters (see 11.8.4)

HMAC/CMAC Tuple

Either the HMAC Tuple or the CMAC Tuple shall be the final attribute in the message’s TLV attribute list. This attribute should be included in the message during HO reentry (see 11.1.2).

If the SIQ TLV is not found in the SBC-REQ, then the The capabilities for construction and transmission of MAC PDUs (see 11.8.2) include the following parameter; otherwise, the following parameter is not required to be included in the SBC-RSP:

Maximum number of supported security association (see 11.8.4.6)

The following parameters may be included when If NSP information is solicited in the SBC-REQ message, unless there are no NSP IDs to be included in the NSP List TLV. If and the BS is configured with a list of NSP IDs and the SBC-REQ message included an SIQ TLV (11.8.9), then either the NSP List TLV and, if requested, the Verbose NSP Name List TLV shall be included unless the message includes an or the SII-ADV Message Pointer TLV (providing a pointer to an SII-ADV message in which these TLVs are sent) shall be included:

NSP List (see 11.1.11.1)

Verbose NSP Name List (see 11.1.11.2)

Verbose NSP Name List shall only be included in the message if NSP List TLV is also included in the message.

SII-ADV Message Pointer (see 11.8.14)

If the SIQ TLV is not found in the SBC-REQ, then the following parameters may be included when solicited in the SBC-REQ message:

Visited NSP Realm (see 11.8.13)

SII-ADV Message Pointer (see11.8.14)

MIH Capability Supported (see 11.8.10)

Extended capability (see 11.8.15)