| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications for use of the SII-ADV MAC management message |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Chris Stanaway Ajay Idnani Chris Cushing Mark Marsan Joe Schumacher Motorola* | Voice: +1 (847) 632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Revision 2 / Letter Ballot 26c |  |
| Abstract | P802.16 Rev2/D4 requires clarifications regarding the use of SII-ADV. |  |
| Purpose | Adopt for inclusion in 802.16 Revision 2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarifications for use of the SII-ADV MAC management message

Chris Stanaway, Ajay Idnani, Chris Cushing, Mark Marsan, Joe Schumacher

Motorola

# Problem Statement

Clarification is required regarding BS transmission of the SII-ADV MAC management message. Existing text suggests that the BS either sends the SII-ADV periodically, in response to an SBC-REQ with the SIQ TLV or in response to an SBC-REQ with the SIQ TLV. The text should state that both options may be supported simultaneously.

Clarification is also required regarding BS behavior in response to receiving a SBC-REQ message that includes the SIQ TLV. Existing text suggests that the SS can force the BS to respond with SII-ADV message containing NSP information. This is not the case. The SS cannot force the BS to send the SII-ADV message. It can request NSP information, but the BS can choose to send the information using either the SII-ADV message or in a SBC-RSP message.

# Other Clarifications

As identified in a different comment, the SII-ADV should use the fragmentable broadcast CID, not the broadcast CID.

The SS is not notified of the periodicity (if any) for which the SII-ADV may be broadcast.

# Proposed Text Changes

(Change subclause 6.3.2.3.58, pages 252-253, lines 56-62, 1-2 as indicated)

A BS may use the SII-ADV message to broadcast a list of Network Service Provider (NSP) Identifiers. The message may be broadcast periodically without solicitation. The SII-ADV message or may be sent in response to a SS requesting NSP information solicited by an SS during network entry by including the SIQ TLV in the SBC-REQ message (see section 6.3.2.3.23). This message is sent from the BS to all SSs on the fragmentable broadcast CID. Assignment method, administration, and usage of NSP IDs Ids are outside the scope of this standard. The list of NSP IDs Ids to be included in this message and the message transmission frequency are programmable. The BS may use the SII-ADV message to deliver IEEE 802.21 MIHF frame which carries query response. SII-ADV is broadcasted at pre-determined cycle which was notified to the MS during initial query.