| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SBC-REQ/RSP Fragmentation |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Chris Stanaway Ajay Idnani Chris Cushing Mark Marsan Joe Schumacher Motorola* | Voice: +1 (847) 632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Revision 2 / LB26c |  |
| Abstract | When used in NSP discovery, the SBC-RSP may become large enough to require fragmentation. We propose to allow SBC-REQ/RSP to be transmitted on the Primary CID in addition to the Basic CID |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# SBC-REQ/RSP Fragmentation

Chris Stanaway, Ajay Idnani, Chris Cushing, Mark Marsan, Joe Schumacher

Motorola

# Problem Statement

The SBC-RSP MAC management message can easily be configured to contain more NSP information (NSP ID list and Verbose NSP Name list or NSP realm) than will fit in a single frame.

For instance, for a 5 MHz channel with a 50/50 frame ratio at QPSK ½ and a repetition rate of 6, the SBC-RSP can be no larger than 150 bytes.  This allows a very limited number of NSPs and requires relatively short verbose NSP names.  The problem is even more severe if any of the verbose NSP names use UTF-16 encoding.

# Discussion of Proposed Remedy

The standard should allow the SBC-RSP to be sent on the Primary Management CID so that it can be fragmented. Backward compatibility is maintained:  1) If a SS is compliant with MAC Version 8 or later and the BS is compliant with MAC Version 8 or later, then the SS shall send the SBC-REQ on the Primary Management CID and receive the SBC-RSP on the Primary Management CID; otherwise, it shall use the Basic CID, and 2) The BS determines on which connection to transmit the SBC-RSP based on which connection the SBC-REQ was received.

# Text Changes

[Change Table 37 in subclause 6.3.2.3 (page 80, lines 15-17) as indicated:]

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| 23 | DBPC-REQ | DL burst profile change request | Basic |
| 24 | DBPC-RSP | DL burst profile change response | Basic |
| 25 | RES-CMD | Reset command | Basic |
| 26 | SBC-REQ | SS basic capability request | Basic or Primary management |
| 27 | SBC-RSP | SS basic capability response | Basic or Primary management |
| 28 | CLK-CMP | SS network clock comparison | Broadcast |
| 29 | DREG-CMD | De/Reregister command | Basic |
| 30 | DSX-RVD | DSx received message | Primary management |

[Change subclause 6.3.2.3.23, page 130, lines 27-32 as indicated:]

An SS shall generate SBC-REQ messages including one of the following parameters:

Basic CID (in the MAC header)

The connection identifier in the MAC header is the Basic CID for this SS, as assigned in the RNG-RSP message. A SS includes the Basic CID if it is compliant with MAC version 7 or earlier.

Primary Management CID (in the MAC header)

The connection identifier in the MAC header is the Primary Management CID for this SS, as assigned in the RNG-RSP message. A SS includes the Primary Management CID if it is compliant with MAC version 8 or later.

[Change subclause 6.3.2.3.24, page 131, lines 27-35 as indicated:]

A BS shall generate SBC-RSP messages in the form shown in Table 91, including both of the following parameters:

CID (in the MAC header)

The connection identifier in the MAC header is the Basic CID for this SS, as appears in the RNG-REQ message which was found in the MAC header of the SBC-REQ.