| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SII-ADV Message Pointer TLV size |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Chris Stanaway Ajay Idnani Chris Cushing Mark Marsan Joe Schumacher Motorola* | Voice: +1 (847) 632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Revision 2, LB26c |  |
| Abstract | The SII-ADV Message Pointer TLV places an overly restrictive limit on the SII-ADV message period. |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# SII-ADV Message Pointer TLV size

Chris Stanaway, Ajay Idnani, Chris Cushing, Mark Marsan, Joe Schumacher

Motorola

# Problem Statement

The SII-ADV Message Pointer TLV directs an MS to look for a SII-ADV message in the next frame for which the 8 least significant bits of the frame number are as specified in this TLV.  This requires that a SII-ADV is therefore transmitted within the next 255 frames (or 1.275 seconds).  Given specified default interval for SII-ADV transmissions of 10 seconds, a SII-ADV might not be scheduled to be transmitted in the 255 frames after a SBC-RSP with the SII-ADV Message Pointer TLV is transmitted.

The length of the SII-ADV Message Pointer should be extended to 2 bytes. This enables the MS to be directed to a SII-ADV transmission sometime during the next 65535 frames.

For backward compatibility, the BS should check the MAC version supplied by the MS in the RNG-REQ, and adjust the size of the pointer accordingly.

# Text Changes

[Change subclause 11.8.14, page 1203, lines 11-18 as indicated:]

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| SII-ADV Message Pointer | 183 | 1 2 | The 8 14 least significant bits of the frame number of the frame in which the SII-ADV message with requested information is transmitted. Bit #14-bit#15: Reserved. Shall be set to 0. | SBC-RSP |