| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink Subframe Aggregation |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Sten Sjöberg, Havish Koorapaty Ericsson AB SE-164 80 Stockholm, Sweden | mailto: sten.ingemar.sjoberg@ericsson.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot #26 |  |
| Abstract | This contribution proposes uplink subframe aggregation to improve cell coverage of IEEE 802.16e. |  |
| Purpose | Discuss and Adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Uplink Subframe Aggregation

Sten Sjöberg and Havish Koorapaty

Ericsson AB, Sweden

# Summary

This contribution proposes a logical aggregation of consecutive UL subframes to carry one MAC PDU (medium access control protocol data unit) burst. The benefits of aggregation of UL subframes are:

- Improved coverage with an extension of cell range for Mobile Stations (MS) by allowing the use if the most robust coding and modulation schemes

- Increase of effective data rates at the cell edge

- Ability to maintain a desired overall asymmetric downlink-to-uplink ratio (DL:UL)

- Minimization of overheads caused by fragmentation

The proposal does not impact users that are incapable of the feature.

Aggregation of multiple UL subframes into one logical unit for demodulation and decoding allows one UL MAC PDU that may span over several frames. Consequently, the receiver can accumulate adequate symbol energy to provide better coverage and larger payload can be accommodated for better efficiency. The uplink subframe aggregation logically combines two or more subframe durations into one data burst for an MS at the cell edge. The allocation is done by the mapping messages carried on the downlink.

Multiple alternatives were considered as a means of achieving UL subframe aggregation. A comparison of the proposed method with other alternatives clearly indicates the benefit of the proposal.

Finally, we present a detailed design of the signaling needed to implement the functionality in a flexible and efficient manner.

# Background

IEEE 802.16e accommodates asymmetric traffic by allocating unequal numbers of symbols to downlink and uplink subframes; this is illustrated in Figure 1. For example, DL:UL values of the 5/10MHz reference system range from 35:12 to 26:21, with increments of 1 UL OFDM symbol and the total number of DL and UL symbols being 47 within a frame duration that spans 48.6 symbols, including guard and transceiver switching time.

Figure 1: WirelessMAN OFDMA Reference Systems TDD frame with downlink and uplink subframes

Several inefficiencies can be identified with the existing UL subframe allocation:

- Coverage: The downlink and uplink coverage are largely differentiated by maximum transmitted power allowed for base stations (BS) and MS’s. Typically, there can be differences of two-orders of magnitude, e.g., 20W versus 200mW. For an MS at the cell edge, it is important to impart as much energy as possible to the information bits in order to maximize the coverage. In order to maximize the time of transmission, it is preferable to use as few subchannels as possible. In power-limiting cases, allocation of radio resources on the uplink is limited to a single degree of freedom — time alone enables the receiver to accumulate more energy per information bit. The typically shorter UL duration and the lower maximum transmitted power, further limits the coverage.

- MAC Overhead: A lower UL duration decreases the number of MAC payload bits in relation to the header. Each MAC PDU contains a 6-byte header and a 4-byte CRC for ARQ. Use of HARQ mandates an extra 2-byte CRC at the Physical Layer. Consequently, the throughput seen by upper layers degrades. Detailed examples of MAC PDU efficiency are provided in Table 2. If a VoIP service is carried by a system with a DL:UL ratio of 35:12, the 19.2kbps physical data rate can only support 16 bit vocoder payload every 5 ms, translating to an effective data rate of 3.2 kbps for the service flow. Such a low efficiency will be further reduced if 12 bytes of extra overhead are needed per MAC frame for AES-CCM.

- Robustness of Channel Coding:  The most robust modulation and coding scheme (MCS) provided by the  OFDMA PHY option is rate-1/2 QPSK with 6 repetitions. Given this modulation and code, let us assume that the mobile terminal has already reached its uplink power limit and only one subchannel is allocated for maximum possible received energy per sub-carrier. For a DL:UL ratio 35:12, this code cannot be used with a single subchannel. Indeed, a limitation of one subchannel allows no more than 4 repetitions of the basic rate-1/2 codeword. While HARQ can extend coverage, the effective data rate will then be greatly reduced from 9.6 kb/s to below 3.2 kb/s. Here, we have factored the number of retransmissions into the data rate supported by the first transmission.  The most robust MCS (rate-1/2 QPSK with 6 repetitions) is only supported DL:UL ratio with higher UL allocation, e.g., 29:18.

Table 1: Most robust data rates supported by certain system configurations

| DL:UL ratio | # subchannel | # channel symbols | MCS | payload | repetition factor | data rate |
| --- | --- | --- | --- | --- | --- | --- |
| 35:12 | 1 | 192 | QPSK, rate-1/2 | 48 bits | 4 | 9.6kbps |
| 29:18 | 1 | 336 | QPSK, rate-1/2 | 48 bits | 6 | 9.6kbps |

Table 2: Upper-layer effective data rate and physical data rate

| # required subchannels | total PHY payload bits | MAC overhead | MAC PDU payload | PHY data rate | Effective data rate |
| --- | --- | --- | --- | --- | --- |
| 1 | 96 (12B) | 80 | 16 | 19.2kbps | 3.2kbps |
| 2 | 128 (16B) | 80 | 48 | 25.6kbps | 9.6kbps |
| DL:UL ratio = 35:12 MCS: QPSK rate-1/2, 2 repetitions |  |  |  |  |  |

## Benefit of Uplink Subframe Aggregation

The benefit of uplink subframe aggregation is better coverage for better effective data rate seen by the higher layer. This is clearly demonstrated in Table 3. The effective data rate is defined by dividing the number of MAC PDU payload bits with the total time duration of aggregated frames. As seen in Table 2 and Table 3, to deliver 9.6kbps uplink effective data rate, 2 subchannels must be used to provide sufficient number of channel symbols.

All the configurations shown in Table 3 deliver the same energy per information bit, thus have equivalent coverage performance.

Table 3: Illustration of coverage extension and upper-layer effective data rate improvement by using uplink subframe aggregation under DL:UL ratio = 35:12 with MCS = QPSK rate-1/2, 2 repetitions

| # aggregated subframes | total PHY payload bits | MAC overhead | MAC PDU payload | PHY data rate | Effective data rate |
| --- | --- | --- | --- | --- | --- |
| 1 (1 subchannel) | 96 (12B) | 80 | 16 | 19.2kbps | 3.2kbps |
| 2 | 192 (24B) | 80 | 112 | 19.2kbps | 11.2kbps |
| 3 | 288 (36B) | 80 | 208 | 19.2kbps | 13.86kbps |
| 4 | 384 (48B) | 80 | 304 | 19.2kbps | 15.2kbps |

### Comparison with Other Methods

Hybrid ARQ may be considered as an alternative to the uplink subframe aggregation methods proposed in this contribution. In this case, the improvement in coverage is achieved by the aggregation of energy due to combining of retransmitted blocks. However, HARQ requires the receipt of a NACK before a retransmission can occur, and thus incurs a significant delay compared to the method proposed in this contribution.

Another alternative is the use of pre-emptive retransmissions. In this case, the MS is allocated resources to retransmit the data block multiple times without having to receive a NACK, and thus does not incur the delay due to the NACK messages. However, the effectiveness of this scheme is limited due to the limited number of coding rates available in [1], and due to the limitation of using Chase Combining as specified in [2]. Moreover, the overhead due to the MAC header is present in each transmission and lowers the effectiveness of the method.

Table 4: Illustration of coverage extension and upper-layer effective data rate improvement by using pre-emptive retansmissions under DL:UL ratio = 35:12 with QPSK modulation

| # repeated subframes | total PHY payload bits | MAC overhead | MAC PDU payload | Coding Rate | PHY data rate | Effective data rate |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 192 (24B) | 80 | 112 | 1/2 | 19.2kbps | 11.2kbps |
| 3 | 288 (36B) | 80 | 208 | 3/4 | 19.2kbps | 13.86kbps |
| 4 | 384 (48B) | 80 | 304 | 1 | 19.2kbps | 15.2kbps |

In Table 4, we illustrate the use of pre-emptive retransmissions with Chase combining to achieve comparable data rates to those achieved in Table 3 using uplink subframe aggregation, while keeping the Eb/No value the same. Since repetition by discrete values is achieved by the retransmission, each frame has to carry all the payload bits, thus the coding rate needs to be adjusted accordingly. For the case with two repeated frames, we find that rate-1/2 coding may be used, and the data rate and Eb/No achieved are equivalent compared to the use of uplink subframe aggregation with rate-1/2 coding and two repeats. However, when we go to three repeated frames, we find that a coding rate of 3/4 is needed on the basic transmitted block to achieve the same data rate. With the same Eb/No (which depends only on the number of payload bits, the time of transmission and the power), this will have worse performance than the use of a rate-1/2 code, as used in the uplink subframe aggregation concept. For the case of four repeated frames, we find that pre-emptive retransmission needs to use a code rate of 1 for the same data rate as the uplink subframe aggregation concept (with code rate of 1/2)  at the same Eb/No, and thus will have significantly worse coverage performance.

Thus, it can be reasonably concluded that the use of subframe aggregation, as proposed in this document, has significant advantages over the use of H-ARQ or pre-emptive retransmission for similar purposes.

# Description of Proposed Uplink Subframe Aggregation

The aggregation of UL subframes is illustrated in Figure 2. The slots from all the allocations are combined and then processed in the same way as an allocation in a single frame that contains the total number of slots. The ordering of slots and the mapping of data onto the slots allocated in a single sub-frame remains unchanged. Ordering between sub-frames is done so that data carried by slots from earlier sub-frames appears earlier in the transmitted PDU than data carried by slots from later sub-frames.

Figure 2: Example of a frame structure with aggregated uplink subframes with aggregation over 3 frames is valid for slot allocation in PUSC mode.

### Signalling

Figure 2 also illustrates the signaling changes needed for UL Subframe Aggregation. The UL-MAP message is used with the Extended-2 UIUC IE format which in turn encapsulates an Aggregated UL-MAP IE. The Aggregated UL-MAP IE contains two fields not present in any other kind of UL-MAP IE: the number of slots left in the allocation and a Begin Flag that indicates whether the current frame is the first frame of the transmission or a succeeding frame. A combination of these two flags allows improved PHY performance in certain situations. An extension field allows selective addressing of the same IE to multiple CIDs (e.g. with the UL HARQ MAP IE).

### Effect of UL-MAP Errors

One may reasonably expect UL-MAP error performance to be significantly better than the error performance targeted by data traffic for a single transmission. Consequently, the effect of UL-MAP errors on aggregated frame reception is expected to be negligible. A failure of the first frame of an aggregated transmission will cause failure of the entire transmission due to an inability of the base station to determine how many PHY frames make up the aggregated transmission. However, it is expected that occasional errors in intermediate PHY frames will be handled by declaring the bits transmitted by those frames to be erasures. The overall transmission can then be decoded.

In previous versions of this contribution, we relied on such a rationale to justify the robustness of our design. Still, a concern about the effect of UL-MAP errors on aggregated frames was raised during the meeting in Macau. We have since validated our original rationale. Figure 3 plots the error performance of the aggregated frame in the presence of random errors in the UL-MAP. The UL-MAP error rate in our simulation test has been varied from 0.5% to 5%. The frame error rate for an unaggregated allocation is also plotted for comparison. By example, a target error rate of 10% and a corresponding UL-MAP error rate of 1% leads to a degradation in performance of less than 0.25 dB for the aggregated frame. The error performance without subframe aggregation is worse by 5.75 dB in comparison. In the plot, errors due to a missing allocation are spread randomly over the entire duration of the transmission after deinterleaving.

Figure 3 It is seen that UL-MAP errors in the aggregated allocation have little practical significance on overall code error rates; the aggregated frame spanned four separate PHY frames. The curve without subframe aggregation assumes that 4 subchannels are needed to transmit the coded data within one PHY frame and accounts for the 4-fold decrease in power spectral density. All simulations were performed in AWGN and used an information block size of 480 bits with Rate-1/2 CTC code and QPSK.

# Proposed Text

(1) Change to UL-MAP Extended-2 IE format: Please change Table 379 as follows

8.4.5.4.4.2 UL-MAP Extended-2 IE format

| 05 | Aggregated UL-Map IE Format |
| --- | --- |

(2) Please add Section 8.4.5.4.32

8.4.5.4.32 Aggregated UL-MAP IE

This IE is used to define the all parts of a set of allocations in consecutive frames that will be aggregated together at the receiver. The base station will ensure that all allocations over the total number of frames in the transmission use identical values for the UIUC, and the number of repetitions.  The composition of coded blocks as defined by the slot concatenation rules for various coding schemes will be based on the total number of slots in the aggregated allocation.

In addition to the number of slots allocated in a frame, the allocation IE always indicates the number of slots left in the Slots Remaining field. Additionally, the allocation IE uses a Begin Flag to indicate whether the current frame of transmission will be the first or a succeeding transmission. On receiving an allocation IE where the Begin_Flag is set, the MS constructs a MAC PDU, applies the appropriate coding and repetition, and divides the coded and repeated block into multiple segments of suitable length to be transmitted over the allocated number of slots in each frame using the appropriate modulation. The MS shall use the Slots_Remaining field to determine the correct segment to send during that frame. The MS shall ignore an Aggregated UL-MAP IE indicating a succeeding frame if it has not already received a previous Aggregated UL-MAP IE with the Begin Flag set.

Table 448a Aggregated UL-MAP IE Format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| Aggregated_UL-MAP_IE () { | --- | --- |
| Extended-2 UIUC | 4 | 0x05 |
| Length | 8 | Length in bytes |
| Extended_Aggregated_Format | 1 | Indicates that the encapsulated Extended UIUC 2 dependent IE is addressed to more than one CID |
| UIUC | 4 |  |
| If Extended_aggregated_Format == 1 { | --- | Only valid when UIUC == 11 |
| N_CID | 4 | Number of CIDs addressed by encapsulated Extended-2 UIUC dependent IE |
| for cid = 1 to N_CID { | --- | --- |
| Slots_Remaining | 10 | Number of Slots remaining for the total allocation |
| Begin_Flag | 1 | The first frame of the aggregated allocation will set this bit, while succeeding transmissions of the same allocation will zero it. |
| } | --- | --- |
| } Else { | --- | Extended_Aggregated_Format == 0; only one mobile station is being addressed |
| Slots_Remaining | 10 | Set to the number of slots remaining for end of the aggregated allocation |
| Begin_Flag | 1 | The first frame of the aggregated allocation will set this bit, while succeeding transmissions of the same allocation will zero it. |
| } | --- | --- |
| If (UIUC == 11) { | --- | --- |
| Extended-2 UIUC dependent IE | Variable | See 8.4.5.4.4.2. Some Extended-2 UIUC code assignments may not be used. |
| } |  |  |
| else if (UIUC > 0) & (UIUC < 11) { | --- | --- |
| Duration | 10 | In OFDMA slots (see 8.4.3.1) |
| Repetition coding indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| if (AAS or AMC UL Zone) { | --- | AAS/AMC Allocations include absolute slot offset. |
| Slot offset | 12 | Offset from start of the AAS or AMC zone for this allocation, specified in slots. |
| } |  |  |
| } |  |  |
| } |  |  |

Extended_Aggregated_Format

A bit indicating whether the Aggregated UL-MAP IE addresses more than one mobile station; this is used only with UIUC = 11.

N_CID

The number of CIDs addressed by the Extended-2 UIUC Dependent IE contained in the Aggregated UL-MAP IE.

Slots_Remaining

The number of slots left for the end of the aggregated allocation

Begin Flag

The first frame of the transmission will have this field set to 1, while succeeding frames will have the field set to 0.

The other fields in Table 448a are defined exactly in the same way as the corresponding fields of the UL-MAP IE shown in Table 372 and defined in Section 8.4.5.4.

(3) Please add the following text at the end of section 8.4.4.5

8.4.4.5 UL Transmission Allocations

When uplink frame aggregation is used (section 8.4.5.4.32), allocations in consecutive allocated frames to an MS are treated as a single aggregated burst. The rules for fitting coded blocks within the aggregated allocation are the same as those for a non-aggregated allocation. The number of slots in the aggregated allocation is used as input to the appropriate rules. The transmission of bits starts from the first frame of the allocation and continues through subsequent frames of the allocation with the last bit being transmitted in the last frame of the allocation.

(4) Add Section 8.4.9.8:

8.4.9.8 Segmentation for UL Frame Aggregation

The Frame Aggregation UL-MAP IE, provides the MS with the number of slots allocated for aggregated transmission, where these slots are over multiple frames. After coding and repetition, bits from the resultant data block are sent in order and in accordance with the allocation in each frame and the slots remaining in the allocation; the MS shall discard bits if necessary to comply with this requirement.

(5)  Please add to OFDMA MAP Capability as follows

11.8.3.7.12 OFDMA MAP Capability

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 172 | 1 | Bit 0: HARQ MAP Capability Bit 1: Extended HARQ IE capability Bit 2: Sub MAP capability for first zone Bit 3: Sub MAP capability for other zones Bit 4: DL region definition support Bit 5: Aggregated UL-MAP IE support Bits 6-7: Reserved | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

# Conclusions

This contribution proposes uplink subframe aggregation to increase effective uplink data rates at the cell edge and improve coverage. The proposal enables the usage of the most robust modulation and coding schemes in power limited cases for extended coverage.

# References

- IEEE Std 802.16e-2005 and IEEE Std 802.16 Rev2/D5, “IEEE Standard for local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in License Bands,” June 2008.

- WiMAX Forum™ Mobile System Profile, Release 1.0 Approved Specification (Revision 1.6.1: 2008-04-01)