| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Solution for handling the loss of response messages from BS during network entry |  |
| Date Submitted | 2008-05-12 |  |
| Source(s) | Li Wang Hongyun Qu ZTE Corporation | Voice: E-mail: wangli@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16Rev2/D4, Letter Ballot 26c. Technical Comments |  |
| Abstract | This contribution proposed a solution for handling the loss of response messages from BS during network entry. |  |
| Purpose | Accept the proposed specification changes into IEEE 802.16Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Solution for handling the loss of response messages from BS during network entry

Li Wang, Hongyun Qu

ZTE Corporation

# Problem description

In the current standard IEEE 802.16 Rev2 D4, for the OFDMA PHY, after completion of contention-based initial ranging during network entry, BS shall send a CDMA allocation IE to MS, and then MS shall send a RNG-REQ message to BS.  When BS receives the RNG-REQ message, it shall respond with a RNG-RSP (with matching MAC address) message.  If RNG-RSP (with matching MAC address) message is lost, T3 of MS will expire, and MS shall perform contention-based initial ranging again.

Collisions may occur during contention-based initial ranging.  So it will take a long time to recover from the loss of RNG-RSP (with matching MAC address), and it wastes air resources to repeat contention-based initial ranging and RNG-REQ retransmission.

Actually, it’s not necessary to do contention-based initial ranging and RNG-REQ retransmission when RNG-RSP (with matching MAC address) message is lost. We should ONLY do RNG-RSP (with matching MAC address) message retransmission by BS.

The similar problem exists in the procedure of negotiating basic capabilities during network entry in the current standard. If SBC-RSP message is lost, T18 of MS will expire and MS shall send a SBC-REQ message to BS again. In fact, it’s not necessary to do SBC-REQ retransmission when SBC-RSP message is lost. We should ONLY do SBC-RSP retransmission by BS.

Similiarly in the register procedure, it’s not necessary to do REG-REQ retransmission by MS when REG-RSP message is lost. We should ONLY do REG-RSP retransmission by BS.

In handover case, if RNG-RSP message is lost when MS is performing network reentry to target BS, it will incur a longer interruption and data loss. So it’s required to do RNG-RSP retransmission by BS.

# Proposed solutions

We propose that it shall perform the following operations in sequence during network entry:

- MS sends a REQm message to BS;

- BS responds with a RSPm message to MS, and sets a timer Tn for waiting for the next message REQm+1 from MS;

- If BS receives REQm+1 from MS before Tn expires, BS shall stop the Tn, and respond with RSPm+1 message and set a timer Tn+1;

- If Tn expires and the RSPm message retries are not exhausted, BS shall resend RSPm message to MS and set Tn again;

- If Tn expires and the RSPm message retries are exhausted, BS shall not resend RSPm message to MS and shall send RNG-RSP(abort) message to MS.

As we know, when MS sends a request message to BS, it shall wait for a response message from BS.  The timer value of MS for waiting for a response message has to be long because BS needs to communicate with core network to exchange security information or registration information, etc. and has to handle a lot of MS simultaneously. The process time of BS is long and not predictable.  On the other hand, MS just needs to read configuration table in itself and perform simple operations after it receives a response message from BS, and then it could perform the next step to send the next request message to BS. So the timer value of BS for waiting for the next request message from MS is much shorter within just several frames. The proposed timer Tn  is predictable and short in different phases of network entry including ranging, negotiating basic capabilities and registration. Therefore, with the use of Tn of BS and RSPm message retransmission, it could recover network entry process quickly from losing the RSPm message of BS.

For performing ranging phase:

When BS receives a RNG-REQ message from MS, it shall respond with a RNG-RSP(with matching MAC address) message and set timer T60 for waiting for a SBC-REQ message from MS. BS should stop T60 when it receives a SBC-REQ from MS. If T60 expires and RNG-RSP retries are not exhausted, BS shall send a RNG-RSP(with matching MAC address) message to MS and set timer T60 again. If T60 expires and RNG-RSP retries are exhausted, BS shall not resend a RNG-RSP(with matching MAC address) message, and shall send a RNG-RSP(abort) message to MS.

For negotiating basic capabilities phase:

When BS receives a SBC-REQ message from MS, it shall respond with a SBC-RSP message and set timer T61 for waiting for a REG-REQ(authentication not supported) or an authenticating message from MS. BS shall stop T61 when it receives a REG-REQ or an authenticating message from MS. If T61 expires and SBC-RSP retries are not exhausted, BS shall send a SBC-RSP message to MS and set T61 again. If T61 expires and SBC-RSP retries are exhausted, BS shall not resend a SBC-RSP message, and shall send a RNG-RSP(abort) message to MS.

For performing register phase:

When BS receives a REG-REQ message from MS, it shall respond with a REG-RSP message and set timer T62 for waiting for a TFTP-CPLT or establish connection message from MS. BS shall stop T62 when it receives a TFTP-CPLT or establish connection message from MS. If T62 expires and REG-RSP retries are not exhausted, BS shall send a REG-RSP message to MS and set T62 again. If T62 expires and REG-RSP retries are exhausted, BS shall not resend a REG-RSP message, and shall send a RNG-RSP(abort) message to MS.

For MS reentry to target BS at handover:

When target BS receives a RNG-REQ message from MS, it shall respond with a RNG-RSP(with matching MAC address) message and set timer T60 for waiting for a next message from MS. BS shall stop T60 when it receives a next message from MS. If T60 expires and RNG-RSP retries are not exhausted, BS shall send a RNG-RSP(with matching MAC address) message to MS and set timer T60 again. If T60 expires and RNG-RSP retries are exhausted, BS shall not resend a RNG-RSP(with matching MAC address) message.

Proposed content changed in standard

Replace the figure 85 in section 6.3.9 of P802.16 Rev2 D4 with the following figure:

Replace the figure 80 in section 6.3.9 of P802.16 Rev2 D4 with the following figure:

Replace the figure 84 in section 6.3.9 of P802.16 Rev2 D4 with the following figure:

Add the following definition at the end of table 544 in section 10 of P802.16 Rev2 D4:

| System | Name | Time reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| BS | T60 | SBC-REQ reception timeout following the transmission of a RNG-RSP |  | 10ms | 20ms |
| BS | T61 | REG-REQ reception timeout following the transmission of a SBC-RSP |  | 10ms | 20ms |
| BS | T62 | Establish-connections REQ reception timeout following the transmission of a REG-RSP |  | 10ms | 20ms |
| BS | RNG-RSP Retries | Number of retries on RNG-RSP | 3 | 3 | 16 |
| BS | SBC-RSP Retries | Number of retries on SBC-RSP | 3 | 3 | 16 |
| BS | REG-RSP Retries | Number of retries on REG-RSP | 3 | 3 | 16 |

# References

IEEE Std 802.16Rev2-D4 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” April, 2008